TITLE: QB50 INMS Science Unit Interface Control Document

Document Number: QB50-INMS-MSSL-ID-12001 Issue 12a

Date 18 November 2015

| MSSL |  | VKI |  |
| --- | --- | --- | --- |
| Dhiren Kataria | X | Jean Muylaert |  |
| Alan Smith |  | Thorsten Scholz |  |
| Robert Wicks | X | Davide Masutti |  |
| Mark Hailey |  | Jan Thoemel |  |
| Rahil Chaudery | X | Amandine Denis |  |
| Hubert Hu |  |  |  |
| Berend Winter |  | ISIS |  |
| Alan Spencer | X | Jeroen Rotteveel |  |
| Matthew Whillock |  | Cesar Bernal |  |
| Abdullah Khalil |  | Michiel van Bolhuis |  |
| Gethyn Lewis |  | Arthur Overlack |  |
| Andrew Malpus |  |  |  |
| Theophile Brochant de Villiers | X |  |  |

| Authors: | R A Chaudery | Date: | 27/08/2015 |
| --- | --- | --- | --- |
| Manager/Project Office | R. Wicks | Date: | 27/08/2015 |
| PA: | A. Spencer | Date: | 27/08/2015 |

CHANGE RECORD

| Issue | Date | Sections changed | Comments |
| --- | --- | --- | --- |
| Draft A | 21/01/2012 | All new | First draft |
| Issue 1 | 28/02/2012 | Major revision of most sections | Focus on Science Unit rather than INMS |
| Issue 2 DRAFT | 01/02/2013 | Major revision of most sections | Added CubeSat Software requirements |
| Issue 2 Draft D | 26/03/2013 | 2 | Deleted IDs 1-4. Deleted ND 4. |
|  |  | 1 | Removed some text from introduction and changed SU to INMS_SU |
|  |  | All | Added INMS to references to SU. |
| Issue 2 Draft F | 08/04/2013 | 7, 10, Appendix | Updated keep-out-area drawing. Replaced Commanding, Script handling section as per Draft E version. Updated mechanical ICD to Iss 2. Added structure drawing |
| Issue 3 | 15/04/2013 | All | Minor editorial changes |
| Issue 4 | 11/06/2013 | Header | Corrected version error in header |
| Issue 4 Draft a | 17/07/2013 | All | Added INMS-I-nnnn requirements numbers. RID corrections |
| Issue 4 Draft B | 22/07/2013 | 6 | Renamed AD590 to refer to transducer rather than thermistor. |
|  |  | 4.2 | Added red tag item list. |
|  |  | 7.5 & 7.6 | Added labelling and purge sections for addition of this information. |
| Issue 4 Draft C | 22/07/2013 | 7.6 | Added text in purge section |
|  |  | Contents | Numbering updated |
|  |  | Entire document | Figures/tables renumbered |
|  |  | 4.4 | Section title changed |
|  |  | 4.6, 4.7, 4.8 | Added sections on data and power interfaces |
| Issue 4 Draft D | 23/07/2013 | 4.1, 4.4 | Added TM/TC I/F. Updated HV pinout details in table |
| Issue 4 Draft E | 24/07/2013 | 4.1,4.3, 9 | Updated transducer wiring details. Updated TBD/TBC wording. |
| Issue 4 Draft F | 29/07/2013 | 9 | Filled table 9-2 |
| Issue 5 | 31/07/2013 | Header, title, 4.1, 6,11,12,13 | Version number updated for issue, added shield for STM to connector, updated mechanical appendices |
| Issue 5a | 04/10/2013 | 4.1 | Names changes for pins on connector & corresponding table |
|  |  | 4.2 | Clarified RBI items |
|  |  | 4.3.1 | Clarified OBC V/I monitoring requirements & procedure |
|  |  | 4.5 | BAUD rate changed to 9600 |
|  |  | 4.6 | Clarified definition of duty cycle & updated power figures |
|  |  | 6 | Add STM monitor frequency value |
|  |  | 6.2 | Added schematic of STM circuit (also added larger version in Appendix) |
|  |  | 6.3 | Added requirement for INMS_TH as part of WOD. Also, reference to suitable circuit for measurement. |
|  |  | 7.1 | Updated INMS keep-out area drawing |
|  |  | All 10 | Updated all section to cater for new scheme of handling SCRIPTS |
|  |  | 10.2 | Added requirement of size of MASS_MEMORY storage |
|  |  | 11 | Updated INMS & CubeSat Orientation drawing |
|  |  | 12 | Added CCR part numbers |
|  |  | 13 | Added wording that MSSL do not supply the adapter plate |
|  |  | 14,15 | Added STM circuits |
| Issue 5b |  | Front Cover | Updated distribution list |
|  |  | 1 | Moved reference to CCR & Sun-Sensor to new respective sections. Updated wording in introduction. |
|  |  | 3 | Updated Abbreviations table |
|  |  | 4 | Added new “INMS Science Unit Log Book” section. Subsequent section numbers updated. |
|  |  | 5 | Corrected broken reference link, and link to connector pin-out. |
|  |  | 5.3.1 | Changed description of INMS RESET procedure. Corrected reference link. |
|  |  | 7 | Added STM packet description |
|  |  | 7.1 | Added reference to STM locations |
|  |  | 7.3 | Updated text. Added ref. number |
|  |  | 8 | Added section for Attitude Control |
|  |  | 9 | Modified wording for orientation |
|  |  | 9.5 | Removed TBD/TBC’s. Updated text |
|  |  | 12 | Added section for CCR |
|  |  | 13 | Added section for Sun-Sensor |
|  |  | 14.1.1 | Updated text |
|  |  | 14.2 | Corrected typing error. Converted packet header format into a table. Added ref. to Attitude Control section. |
|  |  | 14.4.1.1 | Converted OBC_SU_ERR format into a table |
|  |  | 14.4.2 | Re-formatted paragraph. Updated Error-Handling Procedure. |
|  |  | 14.5 | Updated figure |
|  |  | 14.6 onwards | Re-arranged sections: |
|  |  | 14.6 | Is now SCRIPT: Script Header Definition. Updated table. Re-worded NOTE Re-worded REQ: INMS-I-185, 186 |
|  |  | 14.7.1 | Changed wording |
|  |  | 14.8 | Changed wording for REQ:INMS-I-228 |
|  |  | 14.9 | Updated figure |
|  |  | 14.11 | Changed example - TEXT version |
|  |  | 14.12 | Changed example - BINARY code |
|  |  | 14.13 | Added size of FIXED packet Corrected MSB to LSB |
|  |  | 16 | Updated MICD drawing |
|  |  | 18 | Added drawing for STM Locations |
|  |  | 19 | Changed wording in heading, body, & figure |
| Issue 5e | 29/10/13 | 5.7 | Added power interface hardware figure |
| Issue 5f | 31/10/13 | 1 | Added thermal interface to description |
|  |  | 3 | Abbreviations put into alphabetical order, added CONOPS |
|  |  | 1 | Added Engineering requirements INMS-E-nnn |
|  |  | 12 | Grammar corrected wrt. “CCR’s” |
|  |  | 14.1.1 | Changed wording of uploaded scripts Grammar corrected wrt. “TIME-FIELD’s” |
| Issue 5g | 31/10/13 | 5.1 | Revised figure 5-1 pin 9 name to STM_SHIELD |
|  |  | 5.3 | Added power on timing requirements |
|  |  | 14.4.2 | Text re-ordered |
|  |  | 18 | Drawing updated with numbered STMs |
|  |  | 9.5 | Corrected drawing reference |
|  |  | 12 | CCR datasheet reference removed |
|  |  | 7.1 | “Channels STM_TH[0,1,2,3,4] are located…” changed to “…[0,1,2,3,4] can be located…” |
| Issue 6 | 31/10/2013 |  | Released. |
| Issue 7 | 22/11/2013 | 1 | Text for Introduction |
|  |  | 2.1 | Update ND5 Issue |
|  |  | 2.1 | Added ND7 |
|  |  | 3 | Updated Abbreviations table |
|  |  | 4 | Added REQ: INMS-I-016 |
|  |  | 5 | Corrected paragraph formatting |
|  |  | 5.1 | Reworded & added 25-way MDM connector part number |
|  |  | 5.2 | Added new section for INMS Savers |
|  |  | 5.3 | Deleted NOTE for WOD |
|  |  | New 5.2 & 5.3 | Moved “Serial Interface Hardware Architecture” & “INMS Serial Interface Protocol” sections to 5.3 & 5.4. |
|  |  | 5.5 | Added new requirement for CS side to handle UART using an interrupt & FIFO buffer |
|  |  | 5.6 | Modified heading & clarified description |
|  |  | 5.6.1 | Updated text for INMS aperture covers |
|  |  | 5.7 | Added clarification on use of redundant power interface signals. |
|  |  | 5.7.1 | Added new OBC_SU_HK response packet requirement. |
|  |  | 5.7.1 | Add new requirement INMS-I-101 that CS team shall supply conversion equations for OBC_SU_HK data |
|  |  | 5.7.1 | Split REQ INMS-I 100 and 110 |
|  |  | 5.8 | Clarified Table/text description |
|  |  | 5.9 | Added INMS state & orbit power profiles |
|  |  | 7 | Clarified STM text |
|  |  | 7 | Added requirements INMS-I-300, INMS-I-310 & added figure to explain harness wiring |
|  |  | 7 | Added AD590*F part number |
|  |  | 7.1 | Clarified STM text & added ref. for ND7 |
|  |  | 7.1 | Added REQ: INMS-I-330, 340 |
|  |  | 7.2 | Clarified STM text & updated figure 7-2 text |
|  |  | 7.3 | INMS System temp. text clarified & added harness wiring drawing. |
|  |  | 7.3 | Added INMS_TH AD590KF part number |
|  |  | 7.3 | Added new requirement INMS-I-350, 360 & circuit for INMS_TH monitoring |
|  |  | 9.1 | Changed text to –Z face |
|  |  | 9.1 | Added NOTE to CS teams to take note of fig. showing CS side connector. |
|  |  | 10 | Added drawing showing INMS to CubeSat orientation. |
|  |  | 10 | Added new requirement INMS-I-370 for INMS to CubeSat orientation |
|  |  | 10 | Added new requirement INMS-I-380 for 3-axis stabilised CubeSats |
|  |  | 10.1 | Changed “shall” to “will” |
|  |  | 10.3 | Updated text for mass estimate |
|  |  | 11 | Added ref. to ND7 for handling |
|  |  | 12 | Added new requirement INMS-I-390 for how to handle non-operational condition |
|  |  | 13 | Updated CCR text |
|  |  | 15.1.1 | Re-write of REQ: INMS-I-120 to handle SEVEN on-board scripts. |
|  |  | 15.1.1 | Added that ONLY CONOPS & INMS team can write FLIGHT SCRIPTs |
|  |  | 15.1.1 | Added fig for On-board INMS Script Buffer usage |
|  |  | 15.1.1 | Re-write of REQ:INMS-I-121 |
|  |  | 15.1.1 | Added REQ: INMS-I-122 for uplink command OBC_SCRIPT_UPLOAD |
|  |  | 15.1.1 | Added REQ: INMS-I-126 for checksum check of script |
|  |  | 15.1.1 | updated section |
|  |  | 15.2.1 | Updated INMS Header table information |
|  |  | 15.2.1 | Changed paragraph wording. |
|  |  | 15.2.1 | Clarified INMS-I-150: MASS_MEMORY requirements |
|  |  | 15.3 | Updated figure |
|  |  | 15.4 | Clarified that no protocol for ACK/NACK is between CubeSat and INMS |
|  |  | 15.4 | Added NOTE that ONLY CONOPS & INMS team can write FLIGHT SCRIPT |
|  |  | 15.4 | Removed SU_HK & SU_STM sample timing. |
|  |  | 15.4.1.1 | Added new OBC_SU_HK Packet definition |
|  |  | 15.4.2 | Table 14-4 removed “NO_ERR” code, and given CS teams option to add NULL bytes on ground. |
|  |  | 15.4.2.1 | Updated table to cater for new SEVEN script handling. |
|  |  | 15.4.3 | Changed wording |
|  |  | 15.5 | Renamed fig: RUNNING SCRIPT: Operation Flow Diagram |
|  |  | 15.6 | Clarified that SCRIPT_LENGTH is the count of the whole script file |
|  |  | 15.6 | Corrected bit count values in table |
|  |  | 15.6 | NOTE: Changed “can” to “shall” |
|  |  | 15.6 | Reworded REQ: INMS-I-185 |
|  |  | 15.6 | Reworded REQ:INMS-I-186 |
|  |  | 15.6 | Added REQ:INMS-I-187 |
|  |  | 15.7 | Reworded & corrected typing error |
|  |  | 15.8 | Clarified description of SEQ_CNT |
|  |  | 15.9 | Updated figure |
|  |  | 15.10 | Updated INMS Command/Response Table 15-8 & added new RESPONSE type: OBC_SU_HK |
|  |  | 15.10 | INMS Command/Response Table 15-8: renumbered CMD_ID/RSP_ID for: OBC_SU_ON OBC_SU_OFF OBC_SU_HK OBC_EOT |
|  |  | 15.11 | Added section to describe example script |
|  |  | 15.12 | Re-formatted script & colour coded comments to show these were added by myself AFTER INMS_ScriptTool made the script TEXT file |
|  |  | 15.13 | Corrected typing error for CHECK SUM byte |
|  |  | 15.14 | Updated text for SEQ_CNT description |
|  |  | 15.4.1.1 | Added OBC_SU_HK packet definition |
|  |  | 15.4.2.1 | Corrected last byte count value to byte 173 |
|  |  | 15.6 | Corrected bit field ranges in table 15-5 |
|  |  | 15.15 | Reworded section |
|  |  | 15.16 | Added INMS ON/OFF in-orbit timing periods figure, table and text |
|  |  | 16 | Removed orientation drawing from appendix & moved to main body of document |
|  |  | 16 | Removed mass figure from MICD |
|  |  | 16 | Updated orientation axis on MICD drawing |
|  |  | 18 | Updated STM locations drawing |
|  |  | 20 | Deleted section 20: INMS_TH circuit example – moved to ICD as requirement instead. |
| Issue 8 | 07/05/2014 | 2 | Amended ND5 reference |
|  |  | 2.2 | Removed “Deleted” & replaced with “none” |
|  |  | 5.7.1 | Added specification for monitoring V & I |
|  |  | 5.7.2 | Added INMS power ON/OFF procedure |
|  |  | 6.1 | Added INMS Shielding specification |
|  |  | 5.6 | Removed REQ:INMS-I-095 Aperture cover section |
|  |  | 5.7.2 | Updated power ON/OFF procedure with correct sequencing |
|  |  | 5.8 | Power budget updated with bench testing results |
|  |  | 12 | Updated max non-ops to +70°C |
|  |  | 5.1 | Updated figure caption to refer to diagram as harness side |
|  |  | 15.1.1 | Edited formatting of sequence |
|  |  | 15 | Removed SU_HK packet definition section (formally 15.4.1.1), renumbering of sections. |
|  |  | 15.10 | Updated CubeSat Script Command table (15-8) |
|  |  | All | References to SU_HK packet structure moved to ND8 – User manual |
|  |  | 5.1 | Added Figure showing location of pin 1 on connector & updated connector drawing |
|  |  | 5.1 | Unified wording: all descriped as SCREEN |
|  |  | 5.2 | Added INMS harness Screen Connections |
|  |  | 5.4 to 5.8 | Moved sections around |
|  |  | 5.5 | Added INMS Power Requirements |
|  |  | 5.7 | Added In-rush current section |
|  |  | (5.8.1) | Removed REQ: INMS-I-100: Supply Rail Monitoring by CubeSat |
|  |  | REQ:INMS-I-101 | Removed requirement |
|  |  | REQ:INMS-I-110 | Reworded |
|  |  | 5.7.1 | Updated power on/off procedure |
|  |  | 5.8 | Updated power budget |
|  |  | 6 | Updated |
|  |  | 7 | updated |
|  |  | REQ:INMS-I-350 | updated |
|  |  | 9.1 | Updated NOTE |
|  |  | 10.2 | Updated surface finish description |
|  |  | 10.3 | Released 10% or mass margin, now includes 20% margin. |
|  |  | 11 | Added handling procedure doc ref. |
|  |  | 12 | Changed temp. range |
|  |  | REQ:INMS-I-390 | Updated/clarified requirement |
|  |  | 13 | Spelling corrected |
|  |  | 15 | Updated entire section |
|  |  | 5.1 | Corrected part-number & position of pin #1 on photo |
|  |  | 5.5 | Added power figures |
|  |  | 5.9 | Updated power profiles |
|  |  | 7 & 7.1 | Moved/deleted text |
|  |  | 15.15 | Added REQ:INMS-I-250 for data rates, and note about INMS operating within power budget. |
|  |  | 12 | Thermal properties updated for Ebonol C |
|  |  | 5.9.2 | Reworded to remove TBC |
| 9 | 04/07/2014 | 5.7, table 15-4 | Typo/editorial corrections |
|  |  | 5.5 | Clarified figures |
|  |  | 5.9 | Tables updated with PFM results |
|  |  | 15.2.1 | Changed ECI units to INT16 from UINT16 |
|  |  | 15.4.1.1 | Corrected sequential XSUM_* numbers |
|  |  | 5.8 | Added REQ-INMS-I-129 |
|  |  | 15.10 | Updated script command formats |
|  |  | 15.4.1.1 | INMS-I-182 added |
| 10 | 15/09/2014 | 2 | Updated document list |
|  |  | 5.4,5.5, 5.6 | Re-inserted UART sections |
|  |  | 5.7 | Re-inserted RED TAG section – subsequent sections/figures/tables numbers adjusted |
|  |  | 5.11.1 | “REQ:INMS-I-115 INMS Power ON/OFF…” – renumbered to REQ:INMS-I-114 *I-115 was a duplicate of previous REQ:INMS-I-115 |
|  |  | 10.3 | Updated mass estimate to 240 grams |
|  |  | 15.1.1 | Clarified explanation. |
|  |  | 15.3 | Updated figure |
|  |  | 15.4.1.1 | Changed table formatting. |
|  |  | 15.4.1.1 | Added new REQ:INMS-I-183 |
|  |  | 15.8 | Changed formatting/reworded section |
|  |  | 15.10 | Deleted SCRIPT command OBC_SU_HK |
|  |  | 15.11 | Added new section for OBC_xx Script commands |
|  |  | 15.13 | Updated Script example |
|  |  | 15.14 | Updated BINARY script |
|  |  | 15.15 | Corrected wording in text & table. |
|  |  | 15.16 | Updated packet data rate table & numbers in text |
| 11 | 10/04/2015 | Distribution list | Updated |
|  |  | 5.1 | Table 5-1 -5V typo |
|  |  | 5.1,7.3 | Added note to separate 5V supply to INMS and INMS_TH_GO drive circuit. |
|  |  | 7 | Removed pin/socket from Figure 7-1 |
|  |  | 7.1 | Replaced pin/socket reminder with epoxy |
|  |  | 2.1 | ND7 name updated |
|  |  | 11 | Handling reference updated to ND8 User Manual |
|  |  | 7.3 | Requirement INMS-I-350 sentence fixed – data to be reported in S/C telemetry |
|  |  | 10.3 | Mass figure updated to remove margin |
|  |  | 15.6 | Table 15-5 updated to correct byte/bit error on B10 and B11 |
|  |  | 15.13,15.14 | Updated scripts to corrected QB50 epoch and corrected checksum |
|  |  | 15.1.1, 15.6 | Added note stating “At all times, the most recent script should be the active script.“ |
|  |  | 16 | MICD drawing updated |
|  |  | 2.1 | System requirements document updated |
|  |  | 15.2.1 | Resolution and range of header data increased |
|  |  | 15.2.1 | Reverted resolution and range of header data |
|  |  | 5.1 | Clarified that text is copied from a REQ later in document. |
|  |  | 7.3 | Added new REQ: INMS-I-345 – separate power supply for temp. sensor and INMS |
|  |  | 15.2.1 | Reverted resolution and range of header data. Changed TABLE header to show “Resolution/bit” |
|  |  | 15.6 | Table 15-5 updated to correct byte B10 and B11 bit labels |
|  |  | 15.6 | REQ:INMS-I-186 – clarified wording to which script to run |
|  |  | 5.7 | Added HV_DISARM CubeSat hatch diagram |
|  |  | 8 | Added REQ: INMS-I-116: Position accuracy |
| 12 | 24/08/2015 | Front Page | Updated distribution list |
|  |  | Change record | Re-formatted layout |
|  |  | 5.5 | Clarified 16-bit transmission BYTE order |
|  |  | 15.13 | Corrected formatting |
|  |  | REQ: INMS-I-122 | Corrected terminology |
|  |  | REQ INMS-I-123 | Added new requirement: telecommand to allow all SCRIPT’s to be enabled/disabled |
|  |  | 15.4.1.1 | Corrected typing error |
|  |  | REQ: INMS-I-184 | Added new requirement for Team’s to send OBC_ERR definition to MSSL |
|  |  | Table 15-5 | Deleted B12 definition |
|  |  | 17 | Corrected typing error |
|  |  | 20 | Added INMS Response Packet Structures |
|  |  | 5.13.1 | Corrected ref. link |
|  |  | 5.13.2 | Corrected ref. link |
|  |  | REQ: INMS-I-122 | Reworded, such that, Teams’ can implement Telecommand as req’d. |
|  |  | REQ: INMS-I-126 | Clarified that it is the Team’s responsibility to handle script uploading errors/checksum errors |
|  |  | REQ: INMS-I-142 to REQ: INMS-I-144 | Added new REQ’s: how to package INMS response data on ground |
|  |  | REQ: INMS-I-200 | Updated fig. caption |
|  |  | 15.13/15.4 | Updated example to match latest SCRIPT TOOL implementation |
|  |  | REQ: INMS-I-142 | Requirement dropped |
|  |  | 16 | Updated mechanical ICD to ver 9 |
|  |  |  | Contributors R Wicks added, |
| 12a | 18/11/15 | 5.4 | Added warning off inactive pull-ups on UART when power is OFF |
|  |  | 5.4 | Added recommendation of how to handle UART interface on OBC side |
|  |  | 5.11 | Added note that there is no in-rush current limiting element or control |
|  |  | 5.11 | Note for PSU’s with rise-time control |
|  |  | 5.11.1 | Added note that +5V & +3V3 can switch together in an emergency case |
|  |  | 5.7 | Added note that HV_DISARM plug can be removed for testing at STAGE #2 |
|  |  | 12 | Modified text for REQ:INMS-I-390 |
|  |  | 15 | Added clarification of how to sync to 1st byte of UART data from INMS |
|  |  | 15.1.1 (1) | Updated wording |
|  |  | 15.1.1 (4) | Reworded to clarify to wait AFTER executing command |
|  |  | 15.1.1 (7) | New meaning for EOT: STOP script, and look for next script to run |
|  |  | REQ: INMS-I-122 | Added note that Teams responsible for tracking which slots used for scripts |
|  |  | REQ: INMS-I-123 | Added recommendation for numbers to use for “value” param |
|  |  | REQ: INMS-I-127 | Simplified FLETCHER-16 alogrithm |
|  |  | 15.2.3 | Moved INMS-I-150 here |
|  |  | 15.2.3 | Added note for estimate of max script size |
|  |  | 15.2.3 | Clarified need for two independent memory devices |
|  |  | 15.2.3 | Added REQ: INMS-I-155 |
|  |  | 15.2.3 | Added REQ: INMS-I-160 |
|  |  | 15.4 | Updated TIMEOUT explanation |
|  |  | 15.4 | Changed “CubeSat” to “OBC” |
|  |  | REQ: INMS-I-175 | Added new requirement for timeout counter resetting |
|  |  | 15.4 | Added “Recommendation” of how to detect possible corrupted INMS packet |
|  |  | 15.4.1 | Changed “CubeSat” to “OBC” in heading |
|  |  | 15.4.1.1 | Correct UINT16 comment |
|  |  | 15.5 | Updated diagram |
|  |  | 15.8 | Corrected wording |
|  |  | 15.8 | Corrected Table 15-7 |
|  |  | REQ: INMS-I-228 | Deleted requirement INSM-I-228 |
|  |  | REQ: INMS-I-229 | Deleted requirement INSM-I-229 |
|  |  | 15.9 | Updated flow chart |
|  |  | 15.10 | Changed column heading |
|  |  | 15.10.1 | Changed “CubeSat” to “OBC” |
|  |  | 15.11.2 | Added Table for OBC_SU_ON param’s |
|  |  | 15.11.3 | Clarified OBC_SU_OFF command w.r.t. UART powering OFF |
|  |  | 15.13 | Updated wording |

CONTRIBUTORS

| Name | Company |
| --- | --- |
| D Kataria | MSSL |
| M Whillock | MSSL |
| P Coker | MSSL |
| M Hailey | MSSL |
| H Hu | MSSL |
| A Khalil | MSSL |
| G Lewis | MSSL |
| B Taylor | MSSL |
| R Wicks | MSSL |

CONTENTS

Table of Figures	20

1	Introduction	21

2	Normative and Informative documents	22

2.1	Normative Documents	22

2.2	Informative Documents	22

3	Abbreviations	22

4	INMS Science Unit Log Book	24

5	Electrical Interfaces	24

5.1	INMS Connector Definition	24

5.2	INMS Harness Screen Connections	27

5.3	INMS Savers	27

5.4	Serial Interface Hardware Architecture	28

5.5	INMS Serial Interface Protocol	28

5.6	CubeSat Side Communications Handling of INMS Serial Interface	29

5.7	RBF RED-TAG HV_DISARM Plug Accommodation	29

5.8	SWITCHED Power Pins	31

5.9	INMS Power Requirements	31

5.10	Power Interface Hardware Architecture	32

5.11	In Rush Current	33

5.11.1	INMS Power ON/OFF Procedure	34

5.12	Power Budget	35

5.13	INMS Power Profiles	36

5.13.1	INMS State Power Profile	36

5.13.2	INMS Orbit Power Profile	36

6	Grounding	37

7	Surface Thermal Monitor (STM) temperature sensors	38

7.1	Surface Thermal Monitor Transducer Locations	39

7.2	Surface Thermal Monitor (STM) Interface Circuit	40

7.3	INMS System Temperature Sensor	41

8	Attitude Control & Position	43

9	Mechanical interface	43

9.1	Accommodation and Field of View	43

10	INMS & CubeSat Orientation	44

10.1	Interface control drawing	46

10.2	Surface finish	46

10.3	Mass	46

10.4	Instrument Identification	46

10.5	Purge interface	47

11	Handling	47

12	Thermal	47

13	Corner Cube Reflector (CCR)	48

14	Sun Sensor	48

15	CubeSat Software Requirements for INMS	49

15.1	Overview of INMS science unit Control Strategy	49

15.1.1	Script Handling	50

15.2	CubeSat Data Handling	55

15.2.1	INMS Science Header Format	55

15.2.2	INMS Telemetry File Structure & Naming Convention on Ground	57

15.2.3	INMS Script Strorage and Size	57

15.3	INMS State Transition Diagram	59

15.4	Command & Response Handling	60

15.4.1	CubeSat Error Handling Procedure	61

15.4.1.1	OBC_SU_ERR Packet Definition	62

15.4.2	INMS Error Handling Procedure	68

15.5	Script Handling (v2.0)	69

15.6	SCRIPT: Script Header Definition	70

15.7	SCRIPT: Times-Table	73

15.7.1	SCRIPT: Times-Table: Script_INDEX	73

15.8	SCRIPT: Script Sequences	74

15.9	SCRIPT: Flow Chart of Operation	75

15.10	CubeSat Script Commands	76

15.10.1	CubeSat and INMS SCRIPT Commands	76

15.11	OBC_xx Script Commands	77

15.11.1	OBC_SU_ON	77

15.11.2	OBC_SU_ON command parameters:	77

15.11.3	OBC_SU_OFF	77

15.11.4	OBC_SU_ERR	77

15.11.5	OBC_SU_HK	77

15.11.6	OBC_SU_EOT	77

15.12	An Example INMS Script	78

15.13	Example INMS Command Sequence Script (TEXT VERSION)	79

15.14	Example INMS Command Sequence Script (BINARY VERSION)	81

15.15	INMS Response Packet Structure to CubeSat	82

15.16	Data Rates and Volumes	83

15.17	Example INMS Operation Scenario	84

16	Appendix 1 – INMS Mechanical Structure	86

17	Appendix 2 – INMS to Pumpkin Structure Adapter Plate	87

18	Appendix 3 – STM Transducer Monitor Locations In INMS	88

19	Appendix 4 – STM Transducer Monitor Circuit Used In INMS	89

20	Appendix 5 – INMS Response Packets’ Structure	90

20.1	SU_STIM (0x04) response packet format:	90

20.2	SU_HC (0x06) response packet format:	93

20.3	SU_CAL(0x07) response packet format:	96

20.4	SU_SCI(0x08) response packet format:	99

20.5	SU_HK(0x09) response packet format:	103

20.5.1	HK Header Definition	106

20.6	SU_STM (0x0A) response packet format:	107

20.7	SU_DUMP(0x0B) response packet format:	110

20.8	SU_ERR(0xBB) response packet format:	113

20.8.1	SU_ERR Error Codes Definition	116

20.9	OBC_SU(0xFA)_ERR response packet format:	117

# Table of Figures

Figure 5-1 - Glenair GMR7580-25S-2BPN	24

Figure 5-2 - Straight PCB mount MDM25F Connector Pin-Out (front-end view)	25

Figure 5-3 - Image showing location of pin 1 on INMS connector	26

Figure 5-4 - INMS Harness Screen & Shield Connections	27

Figure 5-5 - FPGA Serial Interface Hardware Architecture	28

Figure 5-6 - HV_DISARM RBF Connection Diagram	30

Figure 5-7 - Power interface hardware	32

Figure 5-8 - In rush current plot for the 5V supply	33

Figure 5-9 - In rush current plot for the 3.3V supply	33

Figure 5-10 - INMS State Power Profile	36

Figure 5-11 - INMS Power Over 96 Minute Orbit	36

Figure 7-1 - CubeSat Side of STM Harness	38

Figure 7-2 - INMS Side of STM Interface Circuit	40

Figure 7-3 - CubeSat Side of INMS_TH Harness	41

Figure 7-4 - Circuit to use by CubeSat Side to measure INMS_TH temperature	42

Figure 9-1 - INMS Science Unit Keepout Areas within CubeSat	44

Figure 10-1 - QB50 INMS Science Unit Orientation to CubeSat & to Earth	45

Figure 15-1 - On-Board INMS Script Buffer Usage	50

Figure 15-2 - INMS State Transition Diagram	59

Figure 15-3 - CubeSat Error Handling State Diagram	61

Figure 15-4 - INMS Error Handling State Diagram	68

Figure 15-5 - RUNNING SCRIPT: Operation Flow Diagram	69

Figure 15-6 - Script Operation Flow Chart	75

Figure 15-8 - Example INMS ON/OFF Orbit Periods	84

Figure 16-1 - QB50 INMS Science Unit Mechanical Interface Drawing ver 9	86

Figure 17-1 - INMS to Pumpkin Structure Adapter Plate	87

Figure 18-1 - STM Transducer Locations	88

Figure 19-1 - STM Interface Circuit on INMS Board	89

# Introduction

This document controls the required interfaces (mechanical, electrical, thermal and software) between the INMS Science Unit and a QB50 CubeSat.

NOTE: This ICD not only produces requirements for CubeSat teams to follow, but, also has to respond to higher INMS specification requirements, and so, to show that the ICD meets these higher requirement numbers too, these are also shown in this document.

These higher level requirements are shown with the following formatting:

Science requirements are numbered as:

INMS-S-nnn

Engineering requirements are numbered:

INMS-E-nnn

New requirement numbers that are generated by this ICD are shown as:

REQ: INMS-I-nnn

# Normative and Informative documents

## Normative Documents

ND1: Deleted.

ND2: CubeSat Design Specification Rev. 12

ND3: QB50-INMS-MSSL-RQ-13001 INMS Science Requirements

ND4: QB50-INMS-MSSL-RP-13001 INMS Design Report

ND5: QB50_Systems_Requirements Issue 7

ND6: QB50-INMS-MSSL-SP-13001 INMS Instrument Specification

ND7:	 QB50-INMS-MSSL-PR-13001 QB50 Surface Thermal Monitor Sensor Mounting Procedure

ND8: QB50-INMS-MSSL-UM-13002 INMS User Manual

## Informative Documents

None

# Abbreviations

| ADC | Analogue to Digital Converter |
| --- | --- |
| CEM | Channel Electron Multiplier |
| CCR | Corner Cube Reflector |
| CONOPS | Concept of Operations |
| DAC | Digital to Analogue Converter |
| EGSE | Electronic Ground Support Equipment |
| EMC | Electromagnetic Compatibility |
| EMI | Electromagnetic Interference |
| EOT | End Of Table |
| ESD | Electro-Static Discharge |
| FMMU | Flash Mass Memory Unit |
| FPGA | Field Programmable Gate Array |
| HSDR | High Speed Data Recorder |
| HV | High Voltage |
| I/F | Interface |
| INMS | Ion/Neutral Mass Spectrometer |
| kbps | Kilo-bits per second |
| LEO | Low Earth Orbit |
| LSB | Least Significant Byte |
| LUT | Look Up Table |
| LV | Low Voltage |
| LVDS | Low Voltage Differential Signalling |
| Mb/day | Mega-bit per day |
| MSB | Most Significant Byte |
| MSSL | Mullard Space Science Laboratory |
| OBC | CubeSat On-Board Computer |
| PCB | Printed Circuit Board |
| PWM | Pulse Width Modulator |
| RBF | Remove Before Flight |
| RBI | Remove Before Integration |
| S/C | Spacecraft |
| SEE | Single Event Effects |
| SMD | Surface Mount Device |
| SPI | Serial Peripheral Interface |
| STM | Surface Thermal Monitor experiment |
| SU | Science Unit |
| TBC | To be confirmed |
| TBD | To be decided |
| TBI | To be included |
| TLE | Two Line Elements |
| WOD | Whole Orbit Data |

# INMS Science Unit Log Book

REQ: INMS-I-010

A logbook will be kept with the INMS Science Unit.

REQ: INMS-I-015

A log of ALL mates/de-mates of the RBF_HV_DISARM plug shall be recorded.

REQ: INMS-I-016

A log of ALL mates/de-mates of the connector to the INMS MDM25 connector shall be recorded.

# Electrical Interfaces

INMS-E-221

There will be a single electrical connector between the INMS science unit and a CubeSat. The INMS science unit will provide a 25-way STRAIGHT MDM-female connector for the CUBESAT side connector to attach to. Position of the connector is as shown in the appendix: Figure 16-1.  The CubeSat connector shall provide the signals as shown in Figure 5-2.

## INMS Connector Definition

The 25-way MDM connector used on the INMS is a Glenair 25-way, MDM, Socket:

- part number: GMR7580-25S-2BPN

Figure 5-1 - Glenair GMR7580-25S-2BPN

Figure 5-2 - Straight PCB mount MDM25F Connector Pin-Out (front-end view)

| Pin | Signal Name | Comment |
| --- | --- | --- |
| 1 | +5V | SWITCHED Power for +5V logic |
| 2 | +3V3 | SWITCHED Power for +3V3 logic |
| 3 | STM_TH_GO_0 | TWIST with PIN #16. Surface Thermal Monitor TRANSDUCER signal for CH0 |
| 4 | STM_TH_GO_1 | TWIST with PIN #17. Surface Thermal Monitor TRANSDUCER signal for CH1 |
| 5 | STM_TH_GO_2 | TWIST with PIN #18. Surface Thermal Monitor TRANSDUCER signal for CH2 |
| 6 | STM_TH_GO_3 | TWIST with PIN #19. Surface Thermal Monitor TRANSDUCER signal for CH3 |
| 7 | STM_TH_GO_4 | TWIST with PIN #20. Surface Thermal Monitor TRANSDUCER signal for CH4 |
| 8 | INMS_TH_GO | TWIST with PIN #21. INMS science unit SYSTEM TRANSDUCER signal GO |
| 9 | STM_SCREEN | SCREEN connection for all 5 off STM harness shields |
| 10 | INMS_TX | Serial line to SEND data from INMS to CubeSat |
| 11 | INMS_RX | Serial line to RECEIVE data from CubeSat to INMS |
| 12 | HV_Disarm | RBF RED TAG: TWIST with PIN #22. Signal for plug situated in CubeSat Access Hatch to DISABLE HV. |
| 13 | GND | System GROUND |
| 14 | +5V | SWITCHED Power for +5V logic |
| 15 | +3V3 | SWITCHED Power for +3V3 logic |
| 16 | STM_TH_RET_0 | TWIST with PIN #3. Surface Thermal Monitor TRANSDUCER RETURN for CH0 |
| 17 | STM_TH_RET_1 | TWIST with PIN #4. Surface Thermal Monitor TRANSDUCER RETURN for CH1 |
| 18 | STM_TH_RET_2 | TWIST with PIN #5. Surface Thermal Monitor TRANSDUCER RETURN for CH2 |
| 19 | STM_TH_RET_3 | TWIST with PIN #6. Surface Thermal Monitor TRANSDUCER RETURN for CH3 |
| 20 | STM_TH_RET_4 | TWIST with PIN #7. Surface Thermal Monitor TRANSDUCER RETURN for CH4 |
| 21 | INMS_TH_RET | TWIST with PIN #8. INMS science unit SYSTEM TRANSDUCER signal RETURN |
| 22 | HV_DISARM_GND | RBF RED TAG: TWIST with PIN #12. GND for plug situated in CubeSat Access Hatch to DISABLE HV |
| 23 | INMS_TX_SCN | SCREEN connection for TX |
| 24 | INMS_RX_SCN | SCREEN connection for RX |
| 25 | GND | System GROUND |

Table 5-1- - INMS Instrument Connector Pin Assignment

Below is the image of the INMS MDM connector which shows the position of pin #1.

Figure 5-3 - Image showing location of pin 1 on INMS connector

Copied from section 7.3:

REQ: INMS-I-345

The 5V supply to drive the INMS_TH_GO (pin #8) shall be separate from the power switch driving the +5V supply to the instrument (pins #1,#14) so that the temperature of the INMS can be measured without powering the unit on. This allows for a check that the INMS is within the operational temperature range before switch on.

## INMS Harness Screen Connections

Figure 5-4 shows the required harness screen connections.

Figure 5-4 - INMS Harness Screen & Shield Connections

## INMS Savers

To preserve the flight MDM25 connector on the INMS, teams will be limited to a total of 10 mate/demate cycles to the flight connector.

Teams are advised to use a harnessed connector saver for bench testing of the INMS.

At integration of the INMS into the CubeSat structure, there is not expected to be space for a connector saver.

## Serial Interface Hardware Architecture

Figure 5-5 shows the electrical circuit for the Serial UART interface used on the INMS DIGITAL board.

Figure 5-5 - FPGA Serial Interface Hardware Architecture

NOTE: pull-up’s are only active when INMS is switched ON!!

So when OBC switches INMS OFF, the UART lines WILL drop to 0V!!

This may lead to spurious behaviour in your INMS UART handler on the OBC, i.e. you may observe that the UART RX line goes to 0V, therefore the OBC UART will see a logic HIGH to LOW transition - i.e. START BIT of UART transmission.

RECOMMENDATION:

Assuming you are using a microcontroller which has a reconfigurable PORT/SERIAL DEVICE module: before you switch OFF power to INMS, make sure you disable the UART interface (and interrupts if used), then when the UART RX line goes low, it should not trigger.

## INMS Serial Interface Protocol

INMS-E-105

A 3V3 I/O standard UART serial interface shall be used to control the INMS from the CubeSat.

The serial I/F settings to use shall be:

BAUD Rate: 		9600

Data Width:		8-bit

Parity bit:			No Parity

START bit:		ONE

STOP bit:			ONE

When sending 16-bit data, the BYTES are sent in LITTLE ENDIAN BYTE order, i.e. the LOW-ORDER-BYTE is sent first, then the HIGH-ORDER-BYTE.

Serial interface packet formats and ERROR handling are described in section 10.

## CubeSat Side Communications Handling of INMS Serial Interface

REQ: INMS-I-020

The CubeSat UART interface shall be designed in such a manner that no packets transmitted by INMS to the CubeSat are lost.

Remember that as the INMS produces data asynchronously, the CubeSat OBC needs to be able to respond and capture all data produced at any time.

The recommendation is to implement an interrupt driven UART interface to the OBC, and to have a UART FIFO to buffer INMS data to guarantee reception at the CubeSat side.

## RBF RED-TAG HV_DISARM Plug Accommodation

REQ: INMS-I-091

Any “test” harness mating to the INMS MDM25 connector shall have, as a minimum, the HV_DISARM (pin #12), HV_DISARM_GND(pin #22) signals AND the RBF plug.  So, a “test” harness could be one with only POWER, UART, and HV_DISARM signals connected (with the RBF plug in place).

REQ: INMS-I-092

The RBF_HV_DISARM plug shall be inserted at ALL times when the INMS is NOT being tested.

REQ: INMS-I-090

The CubeSat teams shall make a RBF RED-TAG plug available for the HV_DISARM function through the CubeSat access hatch.

Figure 5-6 - HV_DISARM RBF Connection Diagram

The "HV_DISARM" & "HV_DISARM_GND" signals shall be routed from the MDM25 connector directly to the HV_DISARM RBF plug in the CubeSat access hatch as a TWISTED PAIR.

The HV_DISARM RBF plug signal is shorted to HV_DISARM_GND when the RBF plug is present (disabling the HV) and then pulled-up internally (inside INMS) when removed (enabling the HV).

| Item | Red Tag Item (RBF) | Function |
| --- | --- | --- |
| 1 | HV_DISARM plug | When removed the HV generators are armed – when connected the HV generators are disabled. |

Table 5-2- Red tag RBF item list

NOTE:

*** This APPLIES TO STAGE #2 ONLY ****

Teams are advised to leave the HV_DISARM plug attached UNLESS specifically carrying out HV testing.

To be clear: when running scripts at STAGE #2 that turn ON HV, teams are allowed to take out the HV_DISARM plug.

## AT STAGE #3: ONLY MSSL SCRIPTS SHALL BE RUN ##

## SWITCHED Power Pins

INMS-E-110

The +5, +3V3 and GND connections are duplicated to provide redundancy in the harness. These have to be controlled by the CubeSat in order to control switching the INMS power ON or OFF by SCRIPT command.

The redundant +5V, +3V3 & GND signals may be either routed from a redundant set of CubeSat power switches, or be a single set of CubeSat power switches and have the outputs routing to both prime & redundant +5V, +3V3 & GND pins.

## INMS Power Requirements

INMS-E-115

The specifications for the +5V and +3V3 voltage rails are:

|  | Voltage | Current (nominal) | Current ( In-rush -peak) |
| --- | --- | --- | --- |
| +5V | +/- 10% | 140mA (168mA peak during SU_SCI) | 4.02 A |
| +3V3 | +/- 10% | 15mA | 830 mA |

Table 5-3- – INMS Voltage & Current Specifications

## Power Interface Hardware Architecture

Figure 5-7 shows the INMS MDM25-way connector power and control interface, plus attached power filtering. The +5V and +3.3V rails are on pins 1, 14 and 2, 15 of SK1 respectively.

Figure 5-7 - Power interface hardware

## In Rush Current

The in rush current is measured for both 5V and 3.3V supplies. There is no current limiting element or control for in-rush current. Figure 5-8 shows the in rush current for the 5V supply with a peak value of about 4.02A and decays to steady state in about 800us.

NOTE: For PSU’s with rise-time control, set to give shortest possible rise time

Figure 5-8 - In rush current plot for the 5V supply

Figure 5-9 shows the in rush current for the 3.3V supply with a peak value of 830mA and it decays in about 40us.

Figure 5-9 - In rush current plot for the 3.3V supply

REQ: INMS-I-110

The INMS CANNOT switch itself ON or OFF. In the event of the OBC detecting an over-current condition or voltage droop/brown-out the INMS shall be RESET by following the following steps:

- turn OFF the INMS

- generate OBC_SU_ERR packet (see 15.4.1.1 for OBC_SU_ERR definition)

- wait 60 sec

- turn ON the INMS

- re-run SCRIPT from the next TIMES-TABLE time-field entry after the current UTC time.

### INMS Power ON/OFF Procedure

REQ: INMS-I-114

The CubeSat electrical switch characteristics shall be that the +5V and +3V3 rails shall be switched ON and achieve nominal voltage levels within 120 ms (max).

The +5V rail should be switched on first, followed by the +3V3 rail.

Time between the two rails being switched on should be less than 1s.

When switching the unit OFF, the +3V3 rail should be switched OFF first, with the +5V switched OFF within 1s of the +3V3 line.

Both rails should settle to ground within 120ms.

NOTE: In the event of a failure, when INMS power has to be turned OFF in an emergency, BOTH +5V & +3V3 rail may be switched off TOGETHER without causing damage to INMS.

NOTE: The FPGA is held in RESET for 226 ms from when 3V3 is stable, and the RESET Supervisor IC is operational.

## Power Budget

INMS-E-100

The power budget averages & maximum values (in INMS states) over an orbit are shown in in Table 5-4. These numbers are preliminary based on precursor testing and are TBC when hardware testing can be carried out.

| INMS State | Power at +5V (mW) |  |  | Power at +3.3V (mW) |  |
| --- | --- | --- | --- | --- | --- |
|  | Orbit average (Duty cycle averaged) duty cycle= 66% | Nominal during Unit operations | Maximum during Unit operations | Orbit average (Duty cycle averaged) duty cycle = 66% | Maximum during Unit operations |
| RESET | 333 | 510 | 510 | 33 | 50 |
| SCIENCE, HEALTH CHECK, CALIBRATION | 475 | 685 | 840 | 26 | 40 |

Table 5-4- - Power Budget

## INMS Power Profiles

### INMS State Power Profile

Figure 5-10 shows the minimum and maximum power for each state. The current on the +5V line will naturally rise by ~15mA approximately 2 minutes after unit switch on. Variations in power consumption during operations are driven by HV level and voltage sweeping.

Figure 5-10 - INMS State Power Profile

### INMS Orbit Power Profile

Figure 5-11 - INMS Power Over 96 Minute Orbit

Figure 5-11 shows the INMS power over an example 96 min orbit. This graph also includes the current variations during the various science modes.

# Grounding

INMS-E-215

The INMS science unit electronics shall be electrically grounded to the INMS structure. As some CubeSat teams have suggested that their CubeSat structure may not be metallic and that a large area of the Solar-panel is also not metallic, the INMS to CubeSat ground connection shall be made through the INMS connector GND pin. The electrical resistance shall be <50 mΩ.

If the structure that the INMS is attached to provides a low impedance ground connection the GND link between the INMS structure and GND will not be made.

The INMS is ESD sensitive and should be handled accordingly with suitable ESD preventative measures in accordance with IEC 61340-5-1.

# Surface Thermal Monitor (STM) temperature sensors

INMS-S-160, INMS-E-250, 255, 260, 265

The Surface Thermal Monitor experiment (STM) consists of six temperature transducers mounted as:

- Five channels are mounted on the CubeSat side (on inner side of solar panels).

- Sixth channel is mounted inside the INMS.

As soon as INMS is switched ON, the STM experiment starts to run, and INMS reads the temperature values from these transducers and produces SU_STM data packets at the default frequency of every 5 mins 50 sec.

NOTE: STM is an INMS experiment.  CubeSat teams are NOT required to perform any control, or action to run the STM experiment – INMS shall control the running of STM.

CubeSat teams only need to provide harnessing and the correct mounting positions of the transducers.

REQ: INMS-I-300

The AD590*F transducer shall be used for the STM circuit. This is read inside the INMS by a 12bit ADC.

REQ: INMS-I-310

CubeSat teams shall provide harnessing for FIVE STM transducers, with each STM transducer harness consisting of a pair of TWISTED, shielded pair of wires. The common SHIELD shall only be connected at the connector end. Figure 7-1 shows the harness wiring on the CubeSat side.

Figure 7-1 - CubeSat Side of STM Harness

## Surface Thermal Monitor Transducer Locations

REQ: INMS-I-330

CubeSat side STM channels (STM_TH[0,1,2,3,& 4]) shall be mounted on the back of the solar panel substrate (or mechanical panel available), at the locations shown in section 18.

REQ: INMS-I-340

The procedure in ND7 shall be followed to mount the STM transducers to the CubeSat structure panels.

The STM channel, STM_TH[5], is located inside of the INMS science unit.

REMEMBER to secure the harness to the panel close to the solder joint to avoid stressing the joint.

## Surface Thermal Monitor (STM) Interface Circuit

The circuit in Figure 7-2shows the INMS side schematic of the circuit used to monitor the STM sensors.

The ADC128S102 part is used for monitoring.  This is an 8-channel, 12-bit ADC with a standard SPI interface.

- This figure is repeated in the appendix as a larger view.

Figure 7-2 - INMS Side of STM Interface Circuit

## INMS System Temperature Sensor

INMS-S-160, 250, 260, 270, 280, INMS-E-240

The INMS_TH GO/RTN pair are AD590KF transducer signals, used by the CubeSat to measure the INMS temperature. This consists of a pair of TWISTED, SHIELDED pair of wires. The INMS_TH SHIELD shall only be connected at CubeSat end of the harness to the CubeSat GROUND point. Figure 7-3 shows the required harness wiring on the CubeSat side.

Figure 7-3 - CubeSat Side of INMS_TH Harness

REQ: INMS-I-345

The 5V supply to drive the INMS_TH_GO (pin #8) shall be separate from the power switch driving the +5V supply to the instrument (pins #1,#14) so that the temperature of the INMS can be measured without powering the unit on. This allows for a check that the INMS is within the operational temperature range before switch on.

REQ: INMS-I-350

The CubeSat shall monitor INMS_TH temperature at a frequency of once every minute. The INMS_TH values shall be reported as part of the spacecraft telemetry.

`

REQ: INMS-I-360

The following circuit shall be used by the CubeSat teams to measure the INMS_TH temperature.

Figure 7-4 - Circuit to use by CubeSat Side to measure INMS_TH temperature

# Attitude Control & Position

REQ: INMS-I-115

The CubeSat shall provide attitude control with a pointing accuracy of +/- 10 deg, and pointing knowledge of +/- 2 deg.

REQ: INMS-I-116

The CubeSat shall determine its position to within 10 km accuracy.

# Mechanical interface

## Accommodation and Field of View

INMS-S-090, QB50-SYS-1.1.1

The INMS will be accommodated at the –Z face of the CubeSat on a 100mm x 100mm face (see ND5). The vector normal to this face shall be in the spacecraft ram velocity direction. This face shall not be available for Solar cells, or any other sensor, or subsystem and nothing must project forward of this face.

Figure 9-1 - INMS Science Unit Keepout Areas within CubeSat

NOTE: the harness & connector shown in Figure 9-1 are from the CUBESAT side.

# INMS & CubeSat Orientation

REQ: INMS-I-370

The orientation of INMS to CUBESAT shall be as shown in Figure 10-1.

REQ: INMS-I-380

For 3-axis stabilised CubeSats the orientation the complete CUBESAT relative to Earth (and so, relative to INMS) shall be as shown in Figure 10-1.

The orientation is specified as shown so that science data from all CubeSats has the same co-ordinate reference system for analysis.

Figure 10-1 - QB50 INMS Science Unit Orientation to CubeSat & to Earth

## Interface control drawing

INMS-E-230, 235, QB50-SYS-1.7.4

The full size mechanical interface drawing is provided in Appendix 1. The INMS is designed to interface to commercially available CubeSat structures either through an adapter or through appropriate relocation of mounting holes on the structures.

NOTE: The INMS team will NOT provide any required adapter plate to fit the INMS to your CubeSat structure.

## Surface finish

INMS-E-240

The overall finish for the Aluminium structure of the INMS is Alocrom 1200. The top hat of the INMS is coated in Ebonol-C for flight. Precautions must be taken to not have any direct contact with the Ebonol-C coating due to the sensitive nature of the surface. The INMS will only have this coating when in flight configuration.

## Mass

INMS-E-225

The mass figure for INMS is 200 grams.

## Instrument Identification

INMS-E-226

The INMS instrument shall be labelled with the following information as a minimum:-

MANUFACTURED BY: MSSL

ITEM: INMS

SER. NO. XX

Where “XX” is the serial number: an incrementing decimal number given to each INMS produced. See the drawing in Appendix 2 for details of instrument identification markings.

## Purge interface

No purge interface is required. The INMS will be purged at spacecraft level on deployer. If purging is required due to project issues, the unit can be purged by bagging of the unit. A purging procedure will be supplied. Vent paths are available from the internal of the unit to the external facets.

# Handling

INMS-E-300

The INMS shall be handled in a Cleanroom environment, Class 8.

The INMS is ESD sensitive and protection protocols shall be followed in accordance with IEC 61340-5-1.

Complete details of procedures for handling can be found in ND8.

# Thermal

INMS-E-285, 290, 295

The following thermal environment shall be maintained by the CubeSat side for the INMS

| Item | Science Unit |
| --- | --- |
| Operational Temperature Range | -20°C to +40°C |
| Non-Operational Temperature Range | -30°C to +70°C |
| Minimum Standby temperature | -20°C |

Table 12-1- - Thermal operating requirements

REQ: INMS-I-390

In the event that the OBC_SU_ON command is time-triggered, the OBC shall ensure that INMS stays switched OFF if the INMS temperature is outside the Operational Temperature Range unless the OBC_SU_ON SAFTEY_ON parameter = OFF (33h).

| Item | Science Unit |
| --- | --- |
| Thermal capacity | 220 J/K |
| Radiative properties | Base Plate: Alpha = 0.44; Epsilon = 0.15 Top Hat: Alpha = 0.97; Epsilon = 0.73 |
| Contact area | 2000 mm² |
| I/F conductance | 250 W/m2K (Estimate) |
| Thermal interface filler | Bare metal contact envisaged |

Table 12-2- - Thermal properties

# Corner Cube Reflector (CCR)

The CCRs are not part of the INMS set.  If a CubeSat team selects to implement the CCRs, it shall be their responsibility to select the location and interface of the Corner Cubes.

CCRs can be supplied to teams who wish to implement them.  These are supplied by Edmund Optics.  The proposed CCR lens material is N-BK7.

The options for the CCR are:

- Part number:  #45-203 (12.7 mm Mounted Corner Cube Reflector)

URL: http://www.edmundoptics.com/optics/prisms/retroreflection-prisms/mounted-n-bk7-corner-cube-retroreflectors/45203

- Part number:  #45-296 (12.7 mm Unmounted Corner Cube Prism)

URL: http://www.edmundoptics.com/optics/prisms/retroreflection-prisms/n-bk7-corner-cube-retroreflectors/43296

# Sun Sensor

There is no provision for mounting a Sun Sensor on the external body of the INMS.

# CubeSat Software Requirements for INMS

The following sections describe INMS command and response packet structures and the software requirements that the CubeSat shall follow to implement servicing the INMS instrument.

NOTE: After switching ON the INMS, there is a 10 second delay BEFORE the first packet (SU_HK) is automatically generated.  It is the team’s responsibility to design the OBC UART interface such that the OBC_UART_RX buffer is flushed and empty within the 10 second period before INMS send the first response data packet. This mechanism should give a clean start state to allow the OBC to synchronise to data packets without losing the first byte due to switch-on noise on the UART interface.

## Overview of INMS science unit Control Strategy

The script file is in BINARY and represents the commands to be executed by the CubeSat and INMS.

The scripts provide time-tagged sequences of commands to be executed either by the OBC or INMS.

The general format of the SCRIPT file is:

<SCRIPT_HDR> <TIMES-TABLE> <SCRIPT_SEQUENCES> <XSUM>

### Script Handling

REQ: INMS-I-120

Servicing of the INMS is accomplished by running a “Script-Handler” on the CubeSat OBC, which runs one of the on-board scripts.

Figure 15-1 - On-Board INMS Script Buffer Usage

REQ: INMS-I-121

The CubeSat shall provide on-board storage for 7 “IDLE-SLOT” script buffers.

NOTE: CubeSat teams do not have to run multiple scripts simultaneously, rather: only the script header UTC time needs to be read to determine when to mark an IDLE-SLOT script buffer as the RUNNING-SCRIPT. Then this script is executed continuously, until one of the other IDLE SLOT scripts is eligible to run. At all times, the most recent script should be the active script.

The tasks of the Script-handler are:

- Check which of the seven “IDLE SLOT” pre-loaded scripts is eligible to run by checking the script header UTC time value. Wait/keep checking until one is found

- Identify that script as the “RUNNING SCRIPT”.

- Working with the “RUNNING SCRIPT”, check the current on-board time against the Times-Table value, and start executing the script-sequences of COMMANDs when that time is reached.

- For each script-sequence COMMAND: execute the command (i.e. send it to INMS, or OBC executes it), then wait for deltaTime seconds.  This allows the command to finish executing in the INMS.

  - REMEMBER there is no handshaking between INMS and CubeSat.

- After executing every command check if any other “IDLE SLOT” script is ready to run (by checking header UTC time value), if so, abort current script, and repeat above procedure from STEP #2.

- Else, move to the next sequence command, or times-table field.

When the script EOT marker is reached STOP this script and goto (1)

NOTE: Scripts to run on the FM_INMS shall ONLY be written by the CONOPS and MSSL personnel.  The first seven In-flight scripts shall be provided to CubeSat teams to pre-load into their CubeSat prior to launch.

REQ: INMS-I-122

The CubeSat Teams shall provide a telecommand of the form:

- OBC_SCRIPT_UPLOAD(slot, script)

Where:

  - Slot = IDLE SLOT script buffer number (0..6)

  - Script = the SCRIPT FILE to upload

This command shall overwrite the contents of the IDLE-SLOT buffer pointed to by “SLOT” with the new “SCRIPT FILE”.

NOTE: The telecommand “OBC_SCRIPT_UPLOAD” is a generalised example of the telecommand. Each Team may implement the function as it suits their telecommanding system.

The Teams are responsible for tracking which slot they have used to hold a particular script.

REQ: INMS-I-123

The CubeSat Teams shall provide a telecommand of the form:

- OBC_SCRIPT_STATE(value)

Where:

  - value = ‘0’ = ALL scripts are DISABLED

  - value = ‘1’ = ALL scripts are ENABLED

This CubeSat Team’s telecommand shall disable ALL running scripts when value = ‘0’. Only a subsequent telecommand with value = ‘1’ shall re-enable ALL scritps to run again.

NOTE: The telecommand “OBC_SCRIPT_STATE” is a generalised example of the telecommand. Each Team may implement the function as it suits their telecommanding system.

RECOMMENDATION:

A Team has asked if “value” can be used to turn OFF a particular slot: this is fine too.

As ‘0’ is DISABLED, ‘1’ is ENABLED, Teams could use “10d” to “17d” as an index to the SLOT number.

REQ: INMS-I-126

The CubeSat shall run a checksum on the uploaded script to ensure that it has been successfully uploaded without error.

Error handling on exceptions ON the OBC side is the Team’s own responsibility.

REQ: INMS-I-127

The algorithm to use for the Script checksum shall be FLETCHER-16.  In pseudo code, this is:

// SCRIPT is the input BYTE ARRAY

declare UINT16 C0_int, C1_int = 0x0000; //declare & initialise vars

declare UINT16 XSUM_W, = 0xFFFF;

for each index = 0 to sizeof(SCRIPT) do {

C0_int = C0_int + SCRIPT(index);

If (C0_int >= 0xFF) then C0_int = C0_int – 0xFF // this is same as x MOD y

C1_int = C1_int + C0_int;

If (C1_int >= 0xFF) then C1_int = C1_int – 0xFF

}

XSUM_W = (C1_int << 8) or (C0_int); //shift C1_int to top BYTE

Table 15-1- - Fletcher-16 Checksum Pseudo Code

A correct SCRIPT gives XSUM_W = 0x0000, when run through the Fletcher-16 routine.

REQ: INMS-I-128

The OBC shall read each command packet, decode it and perform the action required: either execute the command directly (an OBC instruction), or send the command packet to the INMS_SU for execution (SU instruction).

The script format is described later in this chapter.

In general, the CubeSat shall read the STARTTIME (in UTC format), in the script header and wait until that date & time to start running the script.  The script is written with absolute 24-hour period (h:m:s) time codes of when to execute a sequence of commands.

The sequences consist of a COMMAND, and a DELTA-TIME field.  The COMMAND is read and executed FIRST, then, a delay of DELTA-TIME seconds is elapsed.  The DELTA-TIME is the time that the command requires to execute. REMEMBER CubeSat does NOT know when an INMS command has finished executing, hence this delay, which CONOPS shall calculate when writing scripts for CubeSat teams.  The command can either be executed directly (an OBC_xx command), or the command packet can be sent to the INMS for execution (SU_xx instruction).

## CubeSat Data Handling

REQ: INMS-I-130

On receiving a data packet from the INMS instrument, the CubeSat shall attach a header to the START of the packet consisting of:

- Current space-craft time

- Current space-craft attitude

- Current space-craft position

### INMS Science Header Format

REQ: INMS-I-140

The format of the INMS SCIENCE HEADER packet shall be:

| Field | Parameter | Size | Resolution/bit |
| --- | --- | --- | --- |
| Time field: | UTC (secs since QB50 epoch) | UINT32 | 1 second |
| Attitude: | roll (360 deg) | INT16 | 2 deg |
|  | pitch (360 deg) | INT16 | 2 deg |
|  | yaw (360 deg) | INT16 | 2 deg |
|  | rolldot (deg/sec) | INT16 | milli-deg/sec |
|  | pitchdot (deg/sec) | INT16 | milli-deg/sec |
|  | yawdot (deg/sec) | INT16 | milli-deg/sec |
| Position: | X_ECI (km) | INT16 | 5km |
|  | Y_ECI (km) | INT16 | 5km |
|  | Z_ECI (km) | INT16 | 5km |
|  | TOTAL SCI_HDR | 22 Bytes |  |

Table 15-2- - INMS Science Packet Header Format

Note Table 15-2 shows resolution, not accuracy.

See section 8 for the required attitude specifications.

NOTE: The data in CubeSat MASS_MEMORY will need to be re-packetised into an appropriate CubeSat downlink protocol structure (e.g. CCSDS packet structure).

### INMS Telemetry File Structure & Naming Convention on Ground

REQ: INMS-I-143

All INMS telemetry received by the end-of-day operations shall be concatenated into a single BINARY data file in DATE/TIME order (earliest first).

REQ: INMS-I-144

The format of the FILE NAME shall be:

YYYYMMDD_hhmmss_INMS_TTTT.dat

Where:

YYYY	= year as 4 digits (i.e. 2015)

MM 		= month as 2 digits (i.e. 07)

DD 		= day of month as 2 digits (i.e. 27)

hh 		= hours as 2 digits (i.e. 13)

mm 		= minutes as 2 digits (i.e. 56)

ss 		= seconds as 2 digits (i.e. 37)

TTTT	= Teams Official ID as 4 alpha-numeric characters (i.e. GB03)

e.g:

20150727_135637_INMS_GB03.dat			// for July 27th pass

20150728_201510_INMS_GB03.dat			// for July 28th pass

### INMS Script Strorage and Size

REQ: INMS-I-150

The OBC shall have two independent memory storage units (MASS_MEMORY) to store all the INMS data packets on-board, prior to downloading. Each memory shall be at least 128 Mbytes in size.

The OBC shall insure that data from the PRIME memory store is copied to the SECONDARY memory store as a backup.

NOTE: Script size has been estimated to be a maximum size of 2 kBytes

REQ: INMS-I-155

OBC shall allow each memory storage device to be independently accessed for download of data

REQ: INMS-I-160

OBC shall allow each memory storage device to be independently accessed to allow of data packets to be deleted

## INMS State Transition Diagram

The diagram shows all the valid state transitions that commands from the SCRIPT may make upon execution.  Note, for instance, that science collection cannot be run until the high voltage (HV) has been turned ON correctly.

Figure 15-2 - INMS State Transition Diagram

## Command & Response Handling

There is NO checksum, or ACK/NACK protocol implemented between the INMS and the OBC on the UART interface. This is because there is very little scope for debugging or to correct errors in real time on board.

REQ: INMS-I-170

CubeSat teams shall check that, when INMS is ON, that response packets are being received within the TIMEOUT period of 400 sec.

REQ: INMS-I-175

The OBC shall reset the timeout counter after each BYTE received.

INMS shall automatically generate SU_HK and SU_STM data packets (at least) within a TIMEOUT period of 400 sec (6.6 mins).

NOTE: Every SCRIPT command DOES NOT necessarily generate a response packet!

RECOMMENDATION:

A method to detect if a packet from the INMS is possibly corrupted is to detect the FIRST byte, and compare that against the RSP_ID codes.  If it doesn’t match one of the RSP_ID’s then this cannot be a valid packet.

Remember – a valid packet is one with 174 bytes, AND where the FIRST byte matches a RSP_ID.

NOTE: CubeSat teams are not the CONOPS team.  The ONLY personnel that shall write in-flight or FM_INMS scripts shall be CONOPS or INMS personnel

### OBC Error Handling Procedure

REQ: INMS-I-180

In the event of an error being detected by the CubeSat, the procedure given in section 15.4.2 shall be followed – (repeated here for ease of reading):

- turn OFF the INMS

- generate OBC_SU_ERR packet (see 15.4.1.1 for OBC_SU_ERR definition)

- wait 60 sec

- turn ON the INMS

- re-run SCRIPT from the next TIMES-TABLE time-field value after the current UTC time (see CubeSat Software Requirements for INMS).

Figure 15-3 shows the state machine for CubeSat error handling.

Figure 15-3 - CubeSat Error Handling State Diagram

#### OBC_SU_ERR Packet Definition

REQ: INMS-I-181

The CubeSat teams shall produce The OBC_SU_ERR packet of the format:

|  |  | OBC_SU_ERR format |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte Addr | Hdr \ Data Field | name | bytes | Reg Name | Comment |  |  |  |  |  |  |  |
| b0 | 0 | RSP_ID | 1 | OBC_SU_ERR = 0xFA |  |  |  |  |  |  |  |  |
| b1 | 1 | SEQ_CNT | 1 |  |  |  |  |  |  |  |  |  |
| 0 | 2 |  | 1 | rsp_err_code |  |  |  |  |  |  |  |  |
| 1 | 3 | Script #running | 2 | XSUM_R | UNIT16 - Little Endian format (low byte first) |  |  |  |  |  |  |  |
| 3 | 5 | HDR #Running | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |  |
| 4 | 6 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |  |
| 5 | 7 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |  |
| 6 | 8 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |  |
| 7 | 9 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |  |
| 8 | 10 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |  |
| 9 | 11 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |  |
| 10 | 12 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |  |
| 11 | 13 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |  |
| 12 | 14 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |  |
| 13 | 15 | Script #0 | 2 | XSUM_0 |  |  |  |  |  |  |  |  |  |
| 15 | 17 | HDR #0 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |  |
| 16 | 18 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |  |
| 17 | 19 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |  |
| 18 | 20 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |  |
| 19 | 21 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |  |
| 20 | 22 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |  |
| 21 | 23 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |  |
| 22 | 24 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |  |
| 23 | 25 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |  |
| 24 | 26 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |  |
| 25 | 27 | Script #1 | 2 | XSUM_1 |  |  |  |  |  |  |  |  |  |
| 27 | 29 | HDR #1 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |  |
| 28 | 30 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |  |
| 29 | 31 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |  |
| 30 | 32 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |  |
| 31 | 33 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |  |
| 32 | 34 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |  |
| 33 | 35 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |  |
| 34 | 36 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |  |
| 35 | 37 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |  |
| 36 | 38 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |  |
| 37 | 39 | Script #2 | 2 | XSUM_2 |  |  |  |  |  |  |  |  |  |
| 39 | 41 | HDR #2 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |  |
| 40 | 42 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |  |
| 41 | 43 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |  |
| 42 | 44 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |  |
| 43 | 45 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |  |
| 44 | 46 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |  |
| 45 | 47 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |  |
| 46 | 48 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |  |
| 47 | 49 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |  |
| 48 | 50 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |  |
| 49 | 51 | Script #3 | 2 | XSUM_3 |  |  |  |  |  |  |  |  |  |
| 51 | 53 | HDR #3 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |  |
| 52 | 54 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |  |
| 53 | 55 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |  |
| 54 | 56 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |  |
| 55 | 57 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |  |
| 56 | 58 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |  |
| 57 | 59 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |  |
| 58 | 60 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |  |
| 59 | 61 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |  |
| 60 | 62 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |  |
| 61 | 63 | Script #4 | 2 | XSUM_4 |  |  |  |  |  |  |  |  |  |
| 63 | 65 | HDR #4 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |  |
| 64 | 66 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |  |
| 65 | 67 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |  |
| 66 | 68 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |  |
| 67 | 69 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |  |
| 68 | 70 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |  |
| 69 | 71 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |  |
| 70 | 72 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |  |
| 71 | 73 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |  |
| 72 | 74 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |  |
| 73 | 75 | Script #5 | 2 | XSUM_5 |  |  |  |  |  |  |  |  |  |
| 75 | 77 | HDR #5 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |  |
| 76 | 78 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |  |
| 77 | 79 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |  |
| 78 | 80 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |  |
| 79 | 81 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |  |
| 80 | 82 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |  |
| 81 | 83 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |  |
| 82 | 84 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |  |
| 83 | 85 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |  |
| 84 | 86 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |  |
| 85 | 87 | Script #6 | 2 | XSUM_6 |  |  |  |  |  |  |  |  |  |
| 87 | 89 | HDR #6 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |  |
| 88 | 90 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |  |
| 89 | 91 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |  |
| 90 | 92 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |  |
| 91 | 93 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |  |
| 92 | 94 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |  |
| 93 | 95 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |  |
| 94 | 96 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |  |
| 95 | 97 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |  |
| 96 | 98 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |  |
| 97 | 99 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 98 | 100 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 99 | 101 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 100 | 102 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 101 | 103 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 102 | 104 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 103 | 105 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 104 | 106 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 105 | 107 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 106 | 108 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 107 | 109 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 108 | 110 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 109 | 111 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 110 | 112 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 111 | 113 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 112 | 114 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 113 | 115 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 114 | 116 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 115 | 117 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 116 | 118 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 117 | 119 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 118 | 120 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 119 | 121 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 120 | 122 | 59 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 121 | 123 | 60 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 122 | 124 | 61 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 123 | 125 | 62 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 124 | 126 | 63 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 125 | 127 | 64 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 126 | 128 | 65 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 127 | 129 | 66 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 128 | 130 | 67 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 129 | 131 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 130 | 132 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 131 | 133 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 132 | 134 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 133 | 135 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 134 | 136 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 135 | 137 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 136 | 138 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 137 | 139 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 138 | 140 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 139 | 141 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 140 | 142 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 141 | 143 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 142 | 144 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 143 | 145 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 144 | 146 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 145 | 147 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 146 | 148 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 147 | 149 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 148 | 150 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 149 | 151 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 150 | 152 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 151 | 153 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 152 | 154 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 153 | 155 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 154 | 156 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 155 | 157 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 156 | 158 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 157 | 159 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 158 | 160 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 159 | 161 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 160 | 162 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 161 | 163 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 162 | 164 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 163 | 165 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 164 | 166 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 165 | 167 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 166 | 168 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 167 | 169 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 168 | 170 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 169 | 171 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 170 | 172 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |  |
| 171 | 173 | 68 | 1 | NULL | 63 |  |  |  |  |  |  |  |  |

Table 15-3- - OBC_SU_ERR Packet Format (repeated from Appendix 5)

REQ: INMS-I-182

The OBC_SU_ERR “rsp_err_code” byte is defined as:

| STATUS_REG |  | Comment (16-bit register) |
| --- | --- | --- |
| Rsp_err_code | 00h | No Error |
|  | F0h | INMS packet time-out error |
|  | F1h | INMS packet length error detected |
|  | F2h | INMS emergency turn OFF executed |
|  | F3h | INMS over-current condition detected |
|  | F4h | INMS script command error detected |
|  | F5h-FFh | To be defined by each CubeSat team |

Table 15-4- – OBC_ERR Error Code Definition

REQ: INMS-I-183

If a CubeSat team wishes to add additional error codes, these may be added into the “RSP_ERR_CODE” field using hex codes F5h to FFh.

These definitions shall be communicated back to MSSL

The OBC_SU_ERR packets produced by your CubeSat Script handler are part of the INMS Science data allocation, and are to be stored with INMS data in MASS MEMORY.

REQ: INMS-I-184

The CubeSat Teams shall send their OBC_ERR definition to MSSL as part of the FRR.

### INMS Error Handling Procedure

In the case of the INMS detecting an error, the INMS shall:

- ABORT the on-going operation.

- Turn OFF the HV unit.

- Set an ERROR_FLAG in the HK_STATUS_REG,.

- Send a SU_ERR packet.

- Switch OFF SU_HK & SU_STM packets.

- Wait for-ever (i.e. until OBC cycles INMS power)

Figure 15-4 shows the state machine for INMS error handling.

Figure 15-4 - INMS Error Handling State Diagram

## Script Handling (v2.0)

Figure 15-5 shows the script flow of operation. The arrowed numbers show the time sequence of operations.

Figure 15-5 - RUNNING SCRIPT: Operation Flow Diagram

There are THREE main sections in the script:

- Script Header

- Times-Table

- Script Sequences

These are described in the following sections.

## SCRIPT: Script Header Definition

The script header uses 12 bytes.

All parameters are in Little Endian format.

The script header consists of the following information:

| Bytes | Bit position | Name | Comment |  |
| --- | --- | --- | --- | --- |
| B1,B0 | b15..b0 | Script_LENGTH | Script length in BYTES of COMPLETE script file |  |
| B5..B2 | b31..b0 | T_STARTTIME | UTC time at which to start running the script |  |
| B9..B6 | b31..b0 | FILE_S/N | File Serial Number |  |
| B10 | b4..b0 | SW_ver | Version of SCRIPT TOOL used to write this script |  |
|  | b6,b5 | SU_ID |  | Science Unit : |
|  |  |  |  | 00: reserved |
|  |  |  |  | 01: INMS |
|  |  |  |  | 10: LP |
|  |  |  |  | 11: FIPEX |
|  | b7 | “0” | Not used = ‘0’ |  |
| B11 | b4..b0 | SCRIPT_TYPE | Identifies intended usage of script : |  |
|  |  |  | 00000 | general |
|  |  |  | 00001 | low power |
|  |  |  | 00010 | stage #1 test (INMS_SIM) |
|  |  |  | 00011 | stage #2 test (FM_DIGITAL pcb + dummy) |
|  |  |  | 00100 | stage #3 test (FM INMS on-ground test) |
|  |  |  | 00101 .. 00111 | SPARE |
|  |  |  | 01000 | User Defined#1 (for use by CubeSat team) |
|  |  |  | 01001 | User Defined#2 (for use by CubeSat team) |
|  |  |  | 01010 | User Defined#3 (for use by CubeSat team) |
|  |  |  | 01011 | User Defined#4 (for use by CubeSat team) |
|  |  |  | 01100 | User Defined#5 (for use by CubeSat team) |
|  |  |  | 01101 | User Defined#6 (for use by CubeSat team) |
|  |  |  | 01110 | User Defined#7 (for use by CubeSat team) |
|  |  |  | '01111 | User Defined#8 (for use by CubeSat team) |
|  |  |  | 10000 | FM science script #1 (SU team defined) |
|  |  |  | 10001 | FM science script #2 (SU team defined) |
|  |  |  | 10010 | FM science script #3 (SU team defined) |
|  |  |  | 10011 | FM science script #4 (SU team defined) |
|  |  |  | 10100 | FM science script #5 (SU team defined) |
|  |  |  | 10101 | FM science script #6 (SU team defined) |
|  |  |  | 10110 | FM science script #7 (SU team defined) |
|  |  |  | 10111 | FM science script #8 (SU team defined) |
|  |  |  | 11000 .. 11111 | SPARE |
|  | b6,b5 | SU_MD |  | SU model : |
|  |  |  |  | 00: BB : Bread Board |
|  |  |  |  | 01: EM: Engineering Model |
|  |  |  |  | 10: QM: Qualification Model |
|  |  |  |  | 11: FM: Flight Model |
|  | b7 | “0” |  | Not used = ‘0’ |

Table 15-5- - Script Header Definition

NOTE:

- In addition to the checksum field, a CubeSat script handler shall use the first parameter (<Script_LENGTH>) as an additional check that the script is correct.

- The SCRIPT_TYPE field (codes 0x8 to 0xF) may be used by CubeSat teams to define the script that they write for testing.  It is up to the CubeSat team to document the usage (meaning) of codes 0x8 to 0xF.

REQ: INMS-I-185

The CubeSat script handler shall check the script headers for the “t_STARTTIME” UTC time to determine when & which script to start running.

This script shall be identified as the RUNNING SCRIPT.

REQ: INMS-I-186

Once the “t_STARTTIME” UTC time of a script in any IDLE SLOT buffer becomes ready to run, it shall replace the script in the RUNNING SCRIPT buffer and start to run.

NOTE: At all times, the script with the most recent date/timestamp shall be the active script.

REQ: INMS-I-187

“t_STARTTIME” shall be relative to the UTC epoch given in ND5.

## SCRIPT: Times-Table

REQ: INMS-I-220

The CubeSat shall read the first three bytes form the TIMES-TABLE field and decode these as the 24-hour period TIME at which to start running the SCRIPT_SEQUENCE pointed to by SCRIPT_INDEX.

The TIMES-TABLE fields are of the format:

| Byte No. | Parameter | Comment |
| --- | --- | --- |
| B0 | TIME - Seconds | Range “00” to “59” |
| B1 | TIME – Minutes | Range “00” to “59” |
| B2 | TIME - Hours | Hours range “00” to “23” |
| B3 | Script_INDEX | INDEX values: |
|  |  | 0x41 = S1 script sequence |
|  |  | 0x42 = S2 script sequence |
|  |  | 0x43 = S3 script sequence |
|  |  | 0x44 = S4 script sequence |
|  |  | 0x45 = S5 script sequence |
|  |  | 0x55 = EOT: End-of-Table |

Table 15-6- - SCRIPT: Times-Table Format

### SCRIPT: Times-Table: Script_INDEX

As can be seen, the values of the Script_INDEX are above the valid range of the TIME field, so INDICES are easy to find.  Index EOT (0x55) is the code that indicates the end of the script.

REQ: INMS-I-225

On reaching the EOT (0x55) code in the Times-Table, the SCRIPT shall be re-run from the start of the TIMES-TABLE.

## SCRIPT: Script Sequences

As can be seen from Figure 15-5, there are 5 SCRIPT-SEQUENCE “slots”: S1..S5, which could be coded as 5 ARRAY’s, then each sequence (array) would contain the following sub-fields:

- 2-byte delay time field: deltaTIME:  the length of time (in seconds) to wait AFTER executing the CMD. This is how long OBC should wait to allow the INMS to execute the command.

- nn-bytes of  the CMD to decode and execute.  This could be a command for the CubeSat to execute (OBC_xx), or an INMS command (SU_xx).

The Script-Sequence fields are of the format:

| Byte No. | Parameter | Comment |
| --- | --- | --- |
| B0 | deltaTIME (sec) | Range “00” to “59” – seconds to wait for command to finish |
| B1 | deltaTIME (mins) | Range “00” to “59” – minutes to wait for command to finish |
| B2 | CMD_ID | Command ID |
| B3 | LEN | Length in BYTES of parameter field : B4 to LEN, i.e. 1 to 255 |
| B4… | SEQ_CNT | Command Counter: 0 to 255 (then roll-over). NOTE: SEQ_CNT parameter in the COMMAND is to count contiguous INMS commands – see ND8 section for details about SEQ_CNT usage for response packets. NOTE: SEQ_CNT is in fact param_0 |
|  | Param_1 | First command parameter. |
|  | Param_2 | Second command parameter |
| ..LEN | Param_LEN | Last command parameter |

Table 15-7- - SCRIPT: Script-Sequences Format

## SCRIPT: Flow Chart of Operation

Figure 15-6 illustrates how the script operation should run by the CubeSat.

Figure 15-6 - Script Operation Flow Chart

## CubeSat Script Commands

REQ: INMS-I-190

Table 15-8 lists the SCRIPT commands to control the INMS operation from the CubeSat.

| Name | CMD_ID | RSP_ID | LEN | Parameters |
| --- | --- | --- | --- | --- |
| OBC_SU_ON | 0xF1 | - | 2 | seq_cnt , SAFETY_ON |
| OBC_SU_OFF | 0xF2 | - | 1 | seq_cnt |
| SU_RESET | 0x02 | - | 1 | seq_cnt |
| SU _STIM | 0x04 | 0x04 | 2 | seq_cnt, t_STIM_run, |
| SU _LDP | 0x05 | - | LEN | seq_cnt, MODE , addr, B0, B1, …B_LEN-3 |
| SU _HC | 0x06 | 0x06 | 4 | seq_cnt, [STIM+V_start]U16, sw_HC_anaU8 |
| SU _CAL | 0x07 | 0x07 | 4 | seq_cnt, [STIM+V_start]U16, sw_CAL_anaU8 |
| SU _SCI | 0x08 | 0x08 | 6 | seq_cnt, [offset +STIM+V_start]U16, t_dwellU16, rpt |
| SU HK | - | 0x09 | - | Fixed 174 bytes packet – see SU_HK for details |
| SU _STM | - | 0x0A | - | Fixed 174 bytes packet – see SU_STM for details |
| SU_DUMP | 0x0B | 0x0B | 1 | seq_cnt |
| SU _HVARM | 0x53 | - | 1 | seq_cnt |
| SU _HVON | 0xC9 | - | 1 | seq_cnt |
| SU_ERR | - | 0xBB | - | Fixed 174 bytes – see SU_ERR for details |
| OBC_SU_ERR | - | 0xFA | - | Fixed 174 bytes – see OBC_SU_ERR for details |
| OBC_SU_HK | - |  | - | command deprecated |
| OBC_EOT | 0xFE | - | 1 | seq_cnt |

Table 15-8 - INMS Script Commands/Response Packets

### OBC and INMS SCRIPT Commands

REQ: INMS-I-200

The OBC shall decode and execute any SCRIPT command prefixed by “OBC_”, and shall NOT send it to the INMS.

As can be seen OBC specific commands begin with HEX 0xF in the CMD_ID field. All other (valid) codes represent INMS commands.

Note: The command names shown here are MNEMONICS to help human users to understand & write scripts in an easy manner: these ASCII strings shall be encoded into binary codes for actual uplink to CubeSats. See next section for details.

## OBC_xx Script Commands

### OBC_SU_ON

To execute this script command, the OBC shall turn ON the power to INMS.

##### OBC_SU_ON command parameters:

| Parameter | Unit | Default Value | Comment |
| --- | --- | --- | --- |
| LEN | U8 | 02 | the number of BYTES to follow |
| SEQ_CNT | U8 | SEQ_CNT++ | wrap-around sequential packet count: range 0 to FFh |
| SAFTEY_ON | U8 | ON | ON = AAh, OFF = 33h. When ON, OBC shall only turn ON INMS when within SAFE operational temperatures. When OFF, OBC shall ALWAYS turn ON INMS, EVEN if outside SAFE operational temperatures. |

Table 15-9 – SU_ON Command Parameter Format

### OBC_SU_OFF

To execute this script command, OBC shall first DISABLE the UART interface to the INMS, to stop spurious interrupt triggers being seen by UART handler when INMS power is switched OFF, then OBC shall turn OFF the power to INMS.

### OBC_SU_ERR

On determining an error, the OBC shall produce an OBC_SU_ERR packet.  See section 15.4.1

### OBC_SU_HK

Command has been deprecated.

### OBC_SU_EOT

The CMD_ID for OBC_EOT indicates that this is the end of the SEQUENCE.  Move to the next TIME-FIELD.

## An Example INMS Script

Below is shown an example of an INMS script for reference only.

Remember that ONLY the CONOPS or INMS personnel shall ever write a script to be run on a FM_INMS as based on the Integration Plan.

## Example INMS Command Sequence Script (TEXT VERSION)

Below is an example of the “prettified” human readable TEXT output produced by the INMS_ScriptTool.  I have additionally added extra comments, shown in BLUE, to aid explanation.

Script_LEN:

258d																	// The Script file length

Script_HDR:

t_Start:2015/07/18 11:00:06, S/N:D1CE90B6, Model:Stage #2, type:0, ID:INMS, Ver:06h  // first wait for

//t_STARTTIME

//to be reached

TimeTable:

00 05 00           S1													// S1 index – My once-a-day check of INMS

00 10 00           S2													// S2 index – run sequence at POL

00 30 00           S3													// S3 index – run sequence at EQUATOR

00 50 00           S2													// S2 index – run sequence at POL

01 10 00           S3													// S3 index – run sequence at EQUATOR

01 30 00           S2													// S2 index – run sequence at POL

EOT												// EOT marker, so repeat from S1 index

ScriptSequences:

S1:

00 10 OBC_SU_ON	02 01 ON								// OBC command to turn ON INMS

00 10 SU_STIM      	02 02 64 msec							// send this command to INMS to execute

00 10 SU_DUMP   		01 03 										// recommended SU__DUMP before SU_OFF

00 10 OBC_SU_OFF  01 04										// OBC to turn OFF INMS

00 10 OBC_EOT      	01 05										// marker to move to NEXT Script-Sequence

S2:

00 10 OBC_SU_ON    02 06 ON								// OBC command to turn ON INMS

00 10 SU_LDP       	51 07 n_step_hv(100), t_step_hv(1), hv_max(2500), hv_min(2000),

N0(79h), I0(1Ah), N1(1Ch), I1(49h), N2(0Ah), I2(8Eh),

n_step_F(64), %X_F(99), k_VionF(1000), t_step_F(16), t_dwell(10),

v_ion(36), v_th(2111), v_th_step(4), v_led(959),

n_step_B(16), %X_B(99), t_step_B(32),

V_NB0(9435),V_NB1(16519),V_NB2(18875),V_IB0(9435),

V_IB1(16519),V_IB2(18875),

File_S/N(D1CE90B6), SCR_HDR(Model:Stage #2,

type:00, ID:INMS, ver:06h) 			//send this command to INMS to execute

00 10 SU_HVARM		01 08

02 00 SU_HVON   		01 09

10 00 SU_SCI   		06 10 offSet_OFF, STIM_OFF, 3784 mV, 10000 msec, rpt:5	   // NOTE: delay time

// of TEN mins to

// allow SU_SCI to

// to finish

00 10 SU_DUMP    	01 11 										// recommended SU_DUMP before SU_OFF

00 10 OBC_SU_OFF	01 12 										// OBC to turn OFF INMS

00 10 OBC_EOT  		01 13 										// End-of-sequence marker

S3:

00 10 OBC_SU_ON    02 14 ON

00 10 SU_LDP       	51 15 n_step_hv(100), t_step_hv(1), hv_max(2500), hv_min(2000),

N0(79h), I0(1Ah), N1(1Ch), I1(49h), N2(0Ah), I2(8Eh),

n_step_F(64), %X_F(99), k_VionF(1000), t_step_F(16), t_dwell(10),

v_ion(36), v_th(2111), v_th_step(4), v_led(959),

n_step_B(16), %X_B(99), t_step_B(32),

V_NB0(9435),V_NB1(16519),V_NB2(18875), V_IB0(9435),

V_IB1(16519),V_IB2(18875),

File_S/N(D1CE90B6), SCR_HDR(Model:Stage #2, type:00, ID:INMS, ver:06h)

00 10 SU_HVARM    	01 16

02 00 SU_HVON     	01 17

10 00 SU_SCI      		06 18 offSet_OFF, STIM_OFF, 3784 mV, 10000 msec, rpt:10  // NOTE: delay to

// allow SU_SCI

// to finish

00 10 SU_DUMP    	01 19

00 10 OBC_SU_OFF  01 20

00 10 OBC_EOT      	01 21

XSUM:																			// my checksum values

28

6B

## Example INMS Command Sequence Script (BINARY VERSION)

This is the corresponding BYTE file that would be uploaded to your CubeSat:

02 01 36 EE 3C 1D B6 90 CE D1 26 40 00 05 00 41

00 0A 00 42 00 1E 00 43 00 32 00 42 00 0a 01 43

00 1E 01 42 55 0A 00 F1 02 01 AA 0A 00 04 02 02

40 0A 00 0B 01 03 0A 00 F2 01 04 0A 00 FE 01 05

0A 00 F1 02 06 AA 0A 00 05 33 07 77 00 64 01 C4

09 D0 07 79 49 1A 0A 1C 8E 40 63 E8 03 10 0A 00

B4 00 4F 0A 05 00 5F 09 10 63 20 20 10 3B 1C 42

20 20 10 3B 1C 42 20 B6 90 CE D1 26 40 0A 00 53

01 08 00 02 C9 01 09 00 0A 08 06 0A 1C 02 10 27

05 0A 00 0B 01 0B 0A 00 F2 01 0C 0A 00 FE 01 0D

0A 00 F1 02 0E AA 0A 00 05 33 0F 77 00 64 01 C4

09 D0 07 79 49 1A 0A 1C 8E 40 63 E8 03 10 0A 00

B4 00 4F 0A 05 00 5F 09 10 63 20 20 10 3B 1C 42

20 20 10 3B 1C 42 20 B6 90 CE D1 26 40 0A 00 53

01 10 00 02 C9 01 11 00 0A 08 06 12 1C 02 10 27

0A 0A 00 0B 01 13 0A 00 F2 01 14 0A 00 FE 01 15

28 6B

## INMS Response Packet Structure to CubeSat

REQ: INMS-I-240

Response packets are of a FIXED 174 BYTE size. The packet structure given in Table 15-8 shall be used to transmit responses from the INMS to the CubeSat.

NOTE: the RSP_ID’s shown below are the ONLY response packets generated by commands. The RSP_ID corresponds to the SCRIPT CMD that generated the response packet.

| Byte # | Parameter | Comment |
| --- | --- | --- |
| B0 | RSP_ID | A response packet can be any one of the following ID’s: |
|  |  | 04 = SU_STIM packet |
|  |  | 06 = SU_HC packet |
|  |  | 07 = SU_CAL packet |
|  |  | 08 = SU_SCI packet |
|  |  | 09 = SU_HK packet |
|  |  | 0A = SU_STM packet |
|  |  | 0B = SU_DUMP packet |
|  |  | BB = SU_ERR packet |
| B1 | SEQ_CNT | Wrap-around counters: 0 to 255 per RSP_ID type i.e. for EACH RSP_ID type, there are SEPARATE seq_cnt counters: each rolling over at 255 decimal. So, SU_STM has its own SEQ_CNT (0..255), then SU_HC has its own SEQ_CNT counter, etc |
| B173.. B2 | DATA | Fixed size DATA field of 172 bytes. Packet format details given in ND8. |

Table 15-10- - INMS Response Packet Format

NOTE: In addition to the above INMS response packets, the CubeSat can generate the following response packet:

- OBC_SU_ERR

## Data Rates and Volumes

REQ: INMS-I-250

Each CubeSat carrying an INMS science unit shall communicate a volume of at least 2 Megabits of science data per day to the ground station that is operated by the university providing the CubeSat.

INMS shall be designed to operate within an orbital average power budget of 500 mW

Scripts will control the ON/OFF times of INMS such that the orbit average power consumption of INMS is within 500 mW

INMS-S-150

Data rates & volumes are all aspects of Concept-of-Operation (CONOPS).  As such, cadence, duty cycle & instrument resolution can vastly impact the size of data generated by the INMS instrument.

The aim is to write the INMS control script so that the allocated SCIENCE data rate of 2Mb/day is generated by the instrument.

## Example INMS Operation Scenario

The baseline 0.5 W power usage and 2 Mbit/day downlink allocation could be achieved by preparing the CONOPS plan shown below.

NOTE: This is ONLY an example to show the principle of CONOPS.

From the INMS power profile, the following INMS ON/OFF time periods are calculated, per orbit, to achieve an average power usage of 0.5 W:

Figure 15-8 - Example INMS ON/OFF Orbit Periods

Iterating the parameters for each INMS command and orbit totals, yields the following table:

| pkt rate (min) |  | 21 | OFF | 9 | OFF | 21 | OFF | 9 | OFF | Line total pkts/orbit | Line total pkts per/day |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SU_HK(min) | 6 | 3 | - | 1 | - | 3 | - | 1 | - | 8 | 120 |
| SU_STM(min) | 5 | 4 | - | 1 | - | 4 | - | 1 | - | 10 | 150 |
| SU_SCI(tSCI) | 0.8992 | 23 | - | 10 | - | 23 | - | 10 | - | 66 | 990 |
| SU_DUMP(@OFF) | 1 | 1 |  | 1 |  | 1 |  | 1 |  | 4 | 60 |
| SU_HC(per day) | 1 | 1 | - | - | - | - | - | - | - | 1 | 15 |
| SU_CAL(per day) | 1 | 1 | - | - | - | - | - | - | - | 1 | 15 |
| SU_STIM(per day) | 1 | 1 | - | - | - | - | - | - | - | 1 | 15 |
| TOTAL pkts/day | 1365 |
| bits/day | 1,900,080 |

Table 15-11- - INMS packet data rates per day

So the INMS script would have the following settings:

- Set SU_SCI to generate 174 byte SCIENCE data packets every 53.952 sec (tSCI).

- Set STM to generate ONE 174 byte STM data packet every 5 mins with each of the six STM channels being sampled every 13 sec.

- Set HK to generate ONE 174 byte HK data packet every 6 mins with all 10 HK channels being sampled every 39 sec.

The periods of INMS ON/OFF function as a way of controlling the average INMS power consumption over an orbit (and hence also over the 24 hour period too).  Thus INMS power is kept within the budget figure.

With the above example, the various SCRIPT parameters for the INMS commands would achieve approx. 1.9Mb/day of SCIENCE DATA.

Note: These figures DO NOT include the COMMS protocol overhead, but DO include the SU header that OBC shall attach to every SU data packet.

# Appendix 1 – INMS Mechanical Structure

INMS-E-230

Figure 16-1 - QB50 INMS Science Unit Mechanical Interface Drawing ver 9

# Appendix 2 – INMS to Pumpkin Structure Adapter Plate

For your information only:  MSSL does not supply the adapter plate.

Figure 17-1 - INMS to Pumpkin Structure Adapter Plate

# Appendix 3 – STM Transducer Monitor Locations In INMS

Figure 18-1 - STM Transducer Locations

# Appendix 4 – STM Transducer Monitor Circuit Used In INMS

Figure 19-1 - STM Interface Circuit on INMS Board

# Appendix 5 – INMS Response Packets’ Structure

## SU_STIM (0x04) response packet format:

One full 174 byte packet is sent back to the OBC. The STIM circuit is run at t_STIM_run intervals repeatedly until the COUNTS fill upto the limit of one packet.

|  |  | STIM format |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | name | bytes | Reg Name | Comment |
| 0 | B0 | RSP_ID | 1 | SU_STIM= 0x04 |  |
| 1 | B1 | SEQ_CNT | 1 |  |  |
| 2 | 0 | COUNTS | 2 | COUNTS_1 | UINT16 - Little Endian format (low byte first) |
| 4 | 2 |  | 2 | COUNTS_2 |  |
| 6 | 4 |  | 2 | COUNTS_3 |  |
| 8 | 6 |  | 2 | COUNTS_4 |  |
| 10 | 8 |  | 2 | COUNTS_5 |  |
| 12 | 10 |  | 2 | COUNTS_6 |  |
| 14 | 12 |  | 2 | COUNTS_7 |  |
| 16 | 14 |  | 2 | COUNTS_8 |  |
| 18 | 16 |  | 2 | COUNTS_9 |  |
| 20 | 18 |  | 2 | COUNTS_10 |  |
| 22 | 20 |  | 2 | COUNTS_11 |  |
| 24 | 22 |  | 2 | COUNTS_12 |  |
| 26 | 24 |  | 2 | COUNTS_13 |  |
| 28 | 26 |  | 2 | COUNTS_14 |  |
| 30 | 28 |  | 2 | COUNTS_15 |  |
| 32 | 30 |  | 2 | COUNTS_16 |  |
| 34 | 32 |  | 2 | COUNTS_17 |  |
| 36 | 34 |  | 2 | COUNTS_18 |  |
| 38 | 36 |  | 2 | COUNTS_19 |  |
| 40 | 38 |  | 2 | COUNTS_20 |  |
| 42 | 40 |  | 2 | COUNTS_21 |  |
| 44 | 42 |  | 2 | COUNTS_22 |  |
| 46 | 44 |  | 2 | COUNTS_23 |  |
| 48 | 46 |  | 2 | COUNTS_24 |  |
| 50 | 48 |  | 2 | COUNTS_25 |  |
| 52 | 50 |  | 2 | COUNTS_26 |  |
| 54 | 52 |  | 2 | COUNTS_27 |  |
| 56 | 54 |  | 2 | COUNTS_28 |  |
| 58 | 56 |  | 2 | COUNTS_29 |  |
| 60 | 58 |  | 2 | COUNTS_30 |  |
| 62 | 60 |  | 2 | COUNTS_31 |  |
| 64 | 62 |  | 2 | COUNTS_32 |  |
| 66 | 64 |  | 2 | COUNTS_33 |  |
| 68 | 66 |  | 2 | COUNTS_34 |  |
| 70 | 68 |  | 2 | COUNTS_35 |  |
| 72 | 70 |  | 2 | COUNTS_36 |  |
| 74 | 72 |  | 2 | COUNTS_37 |  |
| 76 | 74 |  | 2 | COUNTS_38 |  |
| 78 | 76 |  | 2 | COUNTS_39 |  |
| 80 | 78 |  | 2 | COUNTS_40 |  |
| 82 | 80 |  | 2 | COUNTS_41 |  |
| 84 | 82 |  | 2 | COUNTS_42 |  |
| 86 | 84 |  | 2 | COUNTS_43 |  |
| 88 | 86 |  | 2 | COUNTS_44 |  |
| 90 | 88 |  | 2 | COUNTS_45 |  |
| 92 | 90 |  | 2 | COUNTS_46 |  |
| 94 | 92 |  | 2 | COUNTS_47 |  |
| 96 | 94 |  | 2 | COUNTS_48 |  |
| 98 | 96 |  | 2 | COUNTS_49 |  |
| 100 | 98 |  | 2 | COUNTS_50 |  |
| 102 | 100 |  | 2 | COUNTS_51 |  |
| 104 | 102 |  | 2 | COUNTS_52 |  |
| 106 | 104 |  | 2 | COUNTS_53 |  |
| 108 | 106 |  | 2 | COUNTS_54 |  |
| 110 | 108 |  | 2 | COUNTS_55 |  |
| 112 | 110 |  | 2 | COUNTS_56 |  |
| 114 | 112 |  | 2 | COUNTS_57 |  |
| 116 | 114 |  | 2 | COUNTS_58 |  |
| 118 | 116 |  | 2 | COUNTS_59 |  |
| 120 | 118 |  | 2 | COUNTS_60 |  |
| 122 | 120 |  | 2 | COUNTS_61 |  |
| 124 | 122 |  | 2 | COUNTS_62 |  |
| 126 | 124 |  | 2 | COUNTS_63 |  |
| 128 | 126 |  | 2 | COUNTS_64 |  |
| 130 | 128 |  | 2 | COUNTS_65 |  |
| 132 | 130 |  | 2 | COUNTS_66 |  |
| 134 | 132 |  | 2 | COUNTS_67 |  |
| 136 | 134 |  | 2 | COUNTS_68 |  |
| 138 | 136 |  | 2 | COUNTS_69 |  |
| 140 | 138 |  | 2 | COUNTS_70 |  |
| 142 | 140 |  | 2 | COUNTS_71 |  |
| 144 | 142 |  | 2 | COUNTS_72 |  |
| 146 | 144 |  | 2 | COUNTS_73 |  |
| 148 | 146 |  | 2 | COUNTS_74 |  |
| 150 | 148 |  | 2 | COUNTS_75 |  |
| 152 | 150 |  | 2 | COUNTS_76 |  |
| 154 | 152 |  | 2 | COUNTS_77 |  |
| 156 | 154 |  | 2 | COUNTS_78 |  |
| 158 | 156 |  | 2 | COUNTS_79 |  |
| 160 | 158 |  | 2 | COUNTS_80 |  |
| 162 | 160 |  | 2 | COUNTS_81 |  |
| 164 | 162 |  | 2 | COUNTS_82 |  |
| 166 | 164 |  | 2 | COUNTS_83 |  |
| 168 | 166 |  | 2 | COUNTS_84 |  |
| 170 | 168 |  | 2 | COUNTS_83 |  |
| 172 | 169 |  | 2 | COUNTS_84 | 174 |

Table 20-1- - SU_STIM Response Packet Format

Total Science Packet Size = 174 Bytes

## SU_HC (0x06) response packet format:

|  |  | RSP_HC format |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Byte Addr | Hdr \ Data Field | name | Bytes | Reg Name | Comment |
| 0 | B0 | RSP_ID | 1 | SU_HC = 0x06 | response for SU_HC |
| 1 | B1 | SEQ_CNT | 1 |  |  |
| 2 | 0 |  | 1 | V_HV_H | Current value of HV_mon from ADC: MSB |
| 3 | 1 |  | 1 | V_HV_L | Current value of HV_mon from ADC: LSB |
| 4 | 2 |  | 2 | Counts_0 | UINT16 - BIG Endian format (high byte first) |
| 6 | 3 |  | 2 | Counts_1 |  |
| 8 | 4 |  | 2 | Counts_2 |  |
| 10 | 5 |  | 2 | Counts_3 |  |
| 12 | 6 |  | 2 | Counts_4 |  |
| 14 | 7 |  | 2 | Counts_5 |  |
| 16 | 8 |  | 2 | Counts_6 |  |
| 18 | 9 |  | 2 | Counts_7 |  |
| 20 | 10 |  | 2 | Counts_8 |  |
| 22 | 11 |  | 2 | Counts_9 |  |
| 24 | 12 |  | 2 | Counts_10 |  |
| 26 | 13 |  | 2 | Counts_11 |  |
| 28 | 14 |  | 2 | Counts_12 |  |
| 30 | 15 |  | 2 | Counts_13 |  |
| 32 | 16 |  | 2 | Counts_14 |  |
| 34 | 17 |  | 2 | Counts_15 |  |
| 36 | 18 |  | 2 | Counts_16 |  |
| 38 | 19 |  | 2 | Counts_17 |  |
| 40 | 20 |  | 2 | Counts_18 |  |
| 42 | 21 |  | 2 | Counts_19 |  |
| 44 | 22 |  | 2 | Counts_20 |  |
| 46 | 23 |  | 2 | Counts_21 |  |
| 48 | 24 |  | 2 | Counts_22 |  |
| 50 | 25 |  | 2 | Counts_23 |  |
| 52 | 26 |  | 2 | Counts_24 |  |
| 54 | 27 |  | 2 | Counts_25 |  |
| 56 | 28 |  | 2 | Counts_26 |  |
| 58 | 29 |  | 2 | Counts_27 |  |
| 60 | 30 |  | 2 | Counts_28 |  |
| 62 | 31 |  | 2 | Counts_29 |  |
| 64 | 32 |  | 2 | Counts_30 |  |
| 66 | 33 |  | 2 | Counts_31 |  |
| 68 | 34 |  | 2 | Counts_32 |  |
| 70 | 35 |  | 2 | Counts_33 |  |
| 72 | 36 |  | 2 | Counts_34 |  |
| 74 | 37 |  | 2 | Counts_35 |  |
| 76 | 38 |  | 2 | Counts_36 |  |
| 78 | 39 |  | 2 | Counts_37 |  |
| 80 | 40 |  | 2 | Counts_38 |  |
| 82 | 41 |  | 2 | Counts_39 |  |
| 84 | 42 |  | 2 | Counts_40 |  |
| 86 | 43 |  | 2 | Counts_41 |  |
| 88 | 44 |  | 2 | Counts_42 |  |
| 90 | 45 |  | 2 | Counts_43 |  |
| 92 | 46 |  | 2 | Counts_44 |  |
| 94 | 47 |  | 2 | Counts_45 |  |
| 96 | 48 |  | 2 | Counts_46 |  |
| 98 | 49 |  | 2 | Counts_47 |  |
| 100 | 50 |  | 2 | Counts_48 |  |
| 102 | 51 |  | 2 | Counts_49 |  |
| 104 | 52 |  | 2 | Counts_50 |  |
| 106 | 53 |  | 2 | Counts_51 |  |
| 108 | 54 |  | 2 | Counts_52 |  |
| 110 | 55 |  | 2 | Counts_53 |  |
| 112 | 56 |  | 2 | Counts_54 |  |
| 114 | 57 |  | 2 | Counts_55 |  |
| 116 | 58 |  | 2 | Counts_56 |  |
| 118 | 59 |  | 2 | Counts_57 |  |
| 120 | 60 |  | 2 | Counts_58 |  |
| 122 | 61 |  | 2 | Counts_59 |  |
| 124 | 62 |  | 2 | Counts_60 |  |
| 126 | 63 |  | 2 | Counts_61 |  |
| 128 | 64 |  | 2 | Counts_62 |  |
| 130 | 65 |  | 2 | Counts_63 | 128 |
| 132 | 66 |  | 1 | NULL |  |
| 133 | 67 |  | 1 | NULL |  |
| 134 | 68 |  | 1 | NULL |  |
| 135 | 69 |  | 1 | NULL |  |
| 136 | 70 |  | 1 | NULL |  |
| 137 | 71 |  | 1 | NULL |  |
| 138 | 72 |  | 1 | NULL |  |
| 139 | 73 |  | 1 | NULL |  |
| 140 | 74 |  | 1 | NULL |  |
| 141 | 75 |  | 1 | NULL |  |
| 142 | 76 |  | 1 | NULL |  |
| 143 | 77 |  | 1 | NULL |  |
| 144 | 78 |  | 1 | NULL |  |
| 145 | 79 |  | 1 | NULL |  |
| 146 | 80 |  | 1 | NULL |  |
| 147 | 81 |  | 1 | NULL |  |
| 148 | 82 |  | 1 | NULL |  |
| 149 | 83 |  | 1 | NULL |  |
| 150 | 84 |  | 1 | NULL |  |
| 151 | 85 |  | 1 | NULL |  |
| 152 | 86 |  | 1 | NULL |  |
| 153 | 87 |  | 1 | NULL |  |
| 154 | 88 |  | 1 | NULL |  |
| 155 | 89 |  | 1 | NULL |  |
| 156 | 90 |  | 1 | NULL |  |
| 157 | 91 |  | 1 | NULL |  |
| 158 | 92 |  | 1 | NULL |  |
| 159 | 93 |  | 1 | NULL |  |
| 160 | 94 |  | 1 | NULL |  |
| 161 | 95 |  | 1 | NULL |  |
| 162 | 96 |  | 1 | NULL |  |
| 163 | 97 |  | 1 | NULL |  |
| 164 | 98 |  | 1 | NULL |  |
| 165 | 99 |  | 1 | NULL |  |
| 166 | 100 |  | 1 | NULL |  |
| 167 | 101 |  | 1 | NULL |  |
| 168 | 102 |  | 1 | NULL |  |
| 169 | 103 |  | 1 | NULL |  |
| 170 | 104 |  | 1 | NULL |  |
| 171 | 105 |  | 1 | NULL |  |
| 172 | 106 |  | 1 | NULL |  |
| 173 | 107 |  | 1 | NULL | 42 |

Table 20-2- - SU_HC Response Packet Format

Total Health Check Packet Size = 174 Bytes

## SU_CAL(0x07) response packet format:

|  |  | RSP_CAL format |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Byte Addr | Hdr \ Data Field | name | Bytes | Reg Name | Comment |
| 0 | B0 | RSP_ID | 1 | SU_CAL = 0x07 | response for SU_CAL |
| 1 | B1 | SEQ_CNT | 1 |  |  |
| 2 | 0 |  | 1 | V_TH_H | Current value of calculated Vth: MSB |
| 3 | 1 |  | 1 | V_TH_L | Current value of calculated Vth: LSB |
| 4 | 2 |  | 2 | Counts_0 | UINT16 - BIG Endian format (high byte first) |
| 6 | 3 |  | 2 | Counts_1 |  |
| 8 | 4 |  | 2 | Counts_2 |  |
| 10 | 5 |  | 2 | Counts_3 |  |
| 12 | 6 |  | 2 | Counts_4 |  |
| 14 | 7 |  | 2 | Counts_5 |  |
| 16 | 8 |  | 2 | Counts_6 |  |
| 18 | 9 |  | 2 | Counts_7 |  |
| 20 | 10 |  | 2 | Counts_8 |  |
| 22 | 11 |  | 2 | Counts_9 |  |
| 24 | 12 |  | 2 | Counts_10 |  |
| 26 | 13 |  | 2 | Counts_11 |  |
| 28 | 14 |  | 2 | Counts_12 |  |
| 30 | 15 |  | 2 | Counts_13 |  |
| 32 | 16 |  | 2 | Counts_14 |  |
| 34 | 17 |  | 2 | Counts_15 |  |
| 36 | 18 |  | 2 | Counts_16 |  |
| 38 | 19 |  | 2 | Counts_17 |  |
| 40 | 20 |  | 2 | Counts_18 |  |
| 42 | 21 |  | 2 | Counts_19 |  |
| 44 | 22 |  | 2 | Counts_20 |  |
| 46 | 23 |  | 2 | Counts_21 |  |
| 48 | 24 |  | 2 | Counts_22 |  |
| 50 | 25 |  | 2 | Counts_23 |  |
| 52 | 26 |  | 2 | Counts_24 |  |
| 54 | 27 |  | 2 | Counts_25 |  |
| 56 | 28 |  | 2 | Counts_26 |  |
| 58 | 29 |  | 2 | Counts_27 |  |
| 60 | 30 |  | 2 | Counts_28 |  |
| 62 | 31 |  | 2 | Counts_29 |  |
| 64 | 32 |  | 2 | Counts_30 |  |
| 66 | 33 |  | 2 | Counts_31 |  |
| 68 | 34 |  | 2 | Counts_32 |  |
| 70 | 35 |  | 2 | Counts_33 |  |
| 72 | 36 |  | 2 | Counts_34 |  |
| 74 | 37 |  | 2 | Counts_35 |  |
| 76 | 38 |  | 2 | Counts_36 |  |
| 78 | 39 |  | 2 | Counts_37 |  |
| 80 | 40 |  | 2 | Counts_38 |  |
| 82 | 41 |  | 2 | Counts_39 |  |
| 84 | 42 |  | 2 | Counts_40 |  |
| 86 | 43 |  | 2 | Counts_41 |  |
| 88 | 44 |  | 2 | Counts_42 |  |
| 90 | 45 |  | 2 | Counts_43 |  |
| 92 | 46 |  | 2 | Counts_44 |  |
| 94 | 47 |  | 2 | Counts_45 |  |
| 96 | 48 |  | 2 | Counts_46 |  |
| 98 | 49 |  | 2 | Counts_47 |  |
| 100 | 50 |  | 2 | Counts_48 |  |
| 102 | 51 |  | 2 | Counts_49 |  |
| 104 | 52 |  | 2 | Counts_50 |  |
| 106 | 53 |  | 2 | Counts_51 |  |
| 108 | 54 |  | 2 | Counts_52 |  |
| 110 | 55 |  | 2 | Counts_53 |  |
| 112 | 56 |  | 2 | Counts_54 |  |
| 114 | 57 |  | 2 | Counts_55 |  |
| 116 | 58 |  | 2 | Counts_56 |  |
| 118 | 59 |  | 2 | Counts_57 |  |
| 120 | 60 |  | 2 | Counts_58 |  |
| 122 | 61 |  | 2 | Counts_59 |  |
| 124 | 62 |  | 2 | Counts_60 |  |
| 126 | 63 |  | 2 | Counts_61 |  |
| 128 | 64 |  | 2 | Counts_62 |  |
| 130 | 65 |  | 2 | Counts_63 | 128 |
| 132 | 66 |  | 1 | NULL |  |
| 133 | 67 |  | 1 | NULL |  |
| 134 | 68 |  | 1 | NULL |  |
| 135 | 69 |  | 1 | NULL |  |
| 136 | 70 |  | 1 | NULL |  |
| 137 | 71 |  | 1 | NULL |  |
| 138 | 72 |  | 1 | NULL |  |
| 139 | 73 |  | 1 | NULL |  |
| 140 | 74 |  | 1 | NULL |  |
| 141 | 75 |  | 1 | NULL |  |
| 142 | 76 |  | 1 | NULL |  |
| 143 | 77 |  | 1 | NULL |  |
| 144 | 78 |  | 1 | NULL |  |
| 145 | 79 |  | 1 | NULL |  |
| 146 | 80 |  | 1 | NULL |  |
| 147 | 81 |  | 1 | NULL |  |
| 148 | 82 |  | 1 | NULL |  |
| 149 | 83 |  | 1 | NULL |  |
| 150 | 84 |  | 1 | NULL |  |
| 151 | 85 |  | 1 | NULL |  |
| 152 | 86 |  | 1 | NULL |  |
| 153 | 87 |  | 1 | NULL |  |
| 154 | 88 |  | 1 | NULL |  |
| 155 | 89 |  | 1 | NULL |  |
| 156 | 90 |  | 1 | NULL |  |
| 157 | 91 |  | 1 | NULL |  |
| 158 | 92 |  | 1 | NULL |  |
| 159 | 93 |  | 1 | NULL |  |
| 160 | 94 |  | 1 | NULL |  |
| 161 | 95 |  | 1 | NULL |  |
| 162 | 96 |  | 1 | NULL |  |
| 163 | 97 |  | 1 | NULL |  |
| 164 | 98 |  | 1 | NULL |  |
| 165 | 99 |  | 1 | NULL |  |
| 166 | 100 |  | 1 | NULL |  |
| 167 | 101 |  | 1 | NULL |  |
| 168 | 102 |  | 1 | NULL |  |
| 169 | 103 |  | 1 | NULL |  |
| 170 | 104 |  | 1 | NULL |  |
| 171 | 105 |  | 1 | NULL |  |
| 172 | 106 |  | 1 | NULL |  |
| 173 | 107 |  | 1 | NULL | 42 |

Table 20-3- - SU_CAL Response Packet Format

Total Calibration Packet Size = 	174 Bytes

## SU_SCI(0x08) response packet format:

|  |  | RSP_SCI format |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Byte Addr | Hdr \ Data Field | name | bits | Reg Name | Comment |
| 0 | B0 | RSP_ID | 8 | SU_SCI = 0x08 |  |
| 1 | B1 | SEQ_CNT | 8 |  |  |
| 2 | 0 | N0: | 16 | I_ION | UINT16 - BIG Endian format (high byte first) |
| 4 | 2 | N0: | 5 | channel of Vpk1 |  |
| … | … |  | 5 | channel of Vpk2 |  |
|  |  | N0: | 12 | BURST_0_count_0 | UINT12 – BIG Endian format |
|  |  |  | 12 | BURST_0_count_1 |  |
|  |  |  | 12 | BURST_0_count_2 |  |
|  |  |  | 12 | BURST_0_count_3 |  |
|  |  |  | 12 | BURST_0_count_4 |  |
|  |  |  | 12 | BURST_0_count_5 |  |
|  |  |  | 12 | BURST_0_count_6 |  |
|  |  |  | 12 | BURST_0_count_7 |  |
|  |  |  | 12 | BURST_0_count_8 |  |
|  |  |  | 12 | BURST_0_count_9 |  |
|  |  |  | 12 | BURST_0_count_10 |  |
|  |  |  | 12 | BURST_0_count_11 |  |
|  |  |  | 12 | BURST_0_count_12 |  |
|  |  |  | 12 | BURST_0_count_13 |  |
|  |  |  | 12 | BURST_0_count_14 |  |
|  |  |  | 12 | BURST_0_count_15 |  |
|  |  |  | 12 | BURST_1_count_0 |  |
|  |  |  | 12 | BURST_1_count_1 |  |
|  |  |  | 12 | BURST_1_count_2 |  |
|  |  |  | 12 | BURST_1_count_3 |  |
|  |  |  | 12 | BURST_1_count_4 |  |
|  |  |  | 12 | BURST_1_count_5 |  |
|  |  |  | 12 | BURST_1_count_6 |  |
|  |  |  | 12 | BURST_1_count_7 |  |
|  |  |  | 12 | BURST_1_count_8 |  |
|  |  |  | 12 | BURST_1_count_9 |  |
|  |  |  | 12 | BURST_1_count_10 |  |
|  |  |  | 12 | BURST_1_count_11 |  |
|  |  |  | 12 | BURST_1_count_12 |  |
|  |  |  | 12 | BURST_1_count_13 |  |
|  |  |  | 12 | BURST_1_count_14 |  |
|  |  |  | 12 | BURST_1_count_15 |  |
|  |  |  | 12 | BURST_2_count_0 |  |
|  |  |  | 12 | BURST_2_count_1 |  |
|  |  |  | 12 | BURST_2_count_2 |  |
|  |  |  | 12 | BURST_2_count_3 |  |
|  |  |  | 12 | BURST_2_count_4 |  |
|  |  |  | 12 | BURST_2_count_5 |  |
|  |  |  | 12 | BURST_2_count_6 |  |
|  |  |  | 12 | BURST_2_count_7 |  |
|  |  |  | 12 | BURST_2_count_8 |  |
|  |  |  | 12 | BURST_2_count_9 |  |
|  |  |  | 12 | BURST_2_count_10 |  |
|  |  |  | 12 | BURST_2_count_11 |  |
|  |  |  | 12 | BURST_2_count_12 |  |
|  |  |  | 12 | BURST_2_count_13 |  |
|  |  |  | 12 | BURST_2_count_14 |  |
|  |  |  | 12 | BURST_2_count_15 |  |
|  |  | I0: | 5 | channel of Vpk1 |  |
|  |  |  | 5 | channel of Vpk2 |  |
|  |  | I0: | 12 | BURST_0_count_0 |  |
|  |  |  | 12 | BURST_0_count_1 |  |
|  |  |  | 12 | BURST_0_count_2 |  |
|  |  |  | 12 | BURST_0_count_3 |  |
|  |  |  | 12 | BURST_0_count_4 |  |
|  |  |  | 12 | BURST_0_count_5 |  |
|  |  |  | 12 | BURST_0_count_6 |  |
|  |  |  | 12 | BURST_0_count_7 |  |
|  |  |  | 12 | BURST_0_count_8 |  |
|  |  |  | 12 | BURST_0_count_9 |  |
|  |  |  | 12 | BURST_0_count_10 |  |
|  |  |  | 12 | BURST_0_count_11 |  |
|  |  |  | 12 | BURST_0_count_12 |  |
|  |  |  | 12 | BURST_0_count_13 |  |
|  |  |  | 12 | BURST_0_count_14 |  |
|  |  |  | 12 | BURST_0_count_15 |  |
|  |  |  | 12 | BURST_1_count_0 |  |
|  |  |  | 12 | BURST_1_count_1 |  |
|  |  |  | 12 | BURST_1_count_2 |  |
|  |  |  | 12 | BURST_1_count_3 |  |
|  |  |  | 12 | BURST_1_count_4 |  |
|  |  |  | 12 | BURST_1_count_5 |  |
|  |  |  | 12 | BURST_1_count_6 |  |
|  |  |  | 12 | BURST_1_count_7 |  |
|  |  |  | 12 | BURST_1_count_8 |  |
|  |  |  | 12 | BURST_1_count_9 |  |
|  |  |  | 12 | BURST_1_count_10 |  |
|  |  |  | 12 | BURST_1_count_11 |  |
|  |  |  | 12 | BURST_1_count_12 |  |
|  |  |  | 12 | BURST_1_count_13 |  |
|  |  |  | 12 | BURST_1_count_14 |  |
|  |  |  | 12 | BURST_1_count_15 |  |
|  |  |  | 12 | BURST_2_count_0 |  |
|  |  |  | 12 | BURST_2_count_1 |  |
|  |  |  | 12 | BURST_2_count_2 |  |
|  |  |  | 12 | BURST_2_count_3 |  |
|  |  |  | 12 | BURST_2_count_4 |  |
|  |  |  | 12 | BURST_2_count_5 |  |
|  |  |  | 12 | BURST_2_count_6 |  |
|  |  |  | 12 | BURST_2_count_7 |  |
|  |  |  | 12 | BURST_2_count_8 |  |
|  |  |  | 12 | BURST_2_count_9 |  |
|  |  |  | 12 | BURST_2_count_10 |  |
|  |  |  | 12 | BURST_2_count_11 |  |
|  |  |  | 12 | BURST_2_count_12 |  |
|  |  |  | 12 | BURST_2_count_13 |  |
|  |  |  | 12 | BURST_2_count_14 |  |
|  |  |  | 12 | BURST_2_count_15 |  |
|  |  | N1: | 5 | channel of Vpk1 |  |
|  |  |  | 5 | channel of Vpk2 |  |
|  |  | N1: | 12 | BURST_B0_count_0 |  |
|  |  |  | 12 | BURST_B1_count_1 |  |
|  |  |  | 12 | BURST_B2_count_2 |  |
|  |  | I1: | 5 | channel of Vpk1 |  |
|  |  |  | 5 | channel of Vpk2 |  |
|  |  | I1: | 12 | BURST_B0_count_0 |  |
|  |  |  | 12 | BURST_B1_count_1 |  |
|  |  |  | 12 | BURST_B2_count_2 |  |
|  |  | N2: | 5 | channel of Vpk1 |  |
|  |  |  | 5 | channel of Vp2 |  |
|  |  | N2: | 12 | BURST_B0_count_0 |  |
|  |  |  | 12 | BURST_B1_count_1 |  |
|  |  |  | 12 | BURST_B2_count_2 |  |
|  |  | I2: | 5 | channel of Vpk1 |  |
|  |  |  | 5 | channel of Vpk2 |  |
|  |  | I2: | 12 | BURST_B0_count_0 |  |
|  |  |  | 12 | BURST_B1_count_1 |  |
|  |  |  | 12 | BURST_B2_count_2 |  |
|  |  | Count_OvrFlw | 1 | Burst_n_OverFlow |  |
|  |  | VLED_OvrFlw | 1 | VLED_OVERLFOW |  |
|  |  | - | 1 | NULL |  |
|  | 1,392 | - | 1 | NULL |  |

Table 20-4- – SU_SCI Response Packet Format

Total Science Packet Size = 	1,392 bits,

= 174 Bytes

## SU_HK(0x09) response packet format:

|  |  | RSP_HK format |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte Addr | Hdr \ Data Field | name | bytes | Reg Name | Comment |  |  |  |  |  |  |
| b0 | 0 | RSP_ID | 1 | SU_HK = 0x09 |  |  |  |  |  |  |  |
| b1 | 1 | SEQ_CNT | 1 |  |  |  |  |  |  |  |  |
| b2 | 2 |  | 1 | cmd_id_err | UINT16 - Little Endian format (low byte first) |  |  |  |  |  |  |
| b3 | 3 |  | 1 | cmd_err_code |  |  |  |  |  |  |  |  |
| b4 | 4 |  | 1 | local_cmd_err |  | ADC0 | ADC1 |  | t |  |  |  |
| 0 | 5 | HK set #1 | 2 | 30V |  |  | 0 |  | 5 |  |  |  |
| 2 | 7 |  | 2 | 10V |  |  | 1 |  | 10 |  |  |  |
| 4 | 9 |  | 2 | -5V |  |  | 2 |  | 15 |  |  |  |
| 6 | 11 |  | 2 | +5V |  |  | 3 |  | 20 |  |  |  |
| 8 | 13 |  | 2 | 3V3 |  |  | 4 |  | 25 |  |  |  |
| 10 | 15 |  | 2 | V_th |  |  | 5 |  | 30 |  |  |  |
| 12 | 17 |  | 2 | V_ION |  |  | 6 |  | 35 |  |  |  |
| 14 | 19 |  | 2 | V_swp1 |  |  | 7 |  | 40 |  |  |  |
| 16 | 21 |  | 2 | HV_mon |  | 5 |  |  | 45 |  |  |  |
| 18 | 23 |  | 2 | I_LED | 20 | 7 |  |  | 50 |  |  |  |
| 20 | 25 | HK set #2 | 2 | 30V |  |  | 0 |  | 55 |  |  |  |
| 22 | 27 |  | 2 | 10V |  |  | 1 |  | 60 |  |  |  |
| 24 | 29 |  | 2 | -5V |  |  | 2 |  | 65 |  |  |  |
| 26 | 31 |  | 2 | +5V |  |  | 3 |  | 70 |  |  |  |
| 28 | 33 |  | 2 | 3V3 |  |  | 4 |  | 75 |  |  |  |
| 30 | 35 |  | 2 | V_th |  |  | 5 |  | 80 |  |  |  |
| 32 | 37 |  | 2 | V_ION |  |  | 6 |  | 85 |  |  |  |
| 34 | 39 |  | 2 | V_swp1 |  |  | 7 |  | 90 |  |  |  |
| 36 | 41 |  | 2 | HV_mon |  | 5 |  |  | 95 |  |  |  |
| 38 | 43 |  | 2 | I_LED | 20 | 7 |  |  | 100 |  |  |  |
| 40 | 45 | HK set #3 | 2 | 30V |  |  | 0 |  | 105 |  |  |  |
| 42 | 47 |  | 2 | 10V |  |  | 1 |  | 110 |  |  |  |
| 44 | 49 |  | 2 | -5V |  |  | 2 |  | 115 |  |  |  |
| 46 | 51 |  | 2 | +5V |  |  | 3 |  | 120 |  |  |  |
| 48 | 53 |  | 2 | 3V3 |  |  | 4 |  | 125 |  |  |  |
| 50 | 55 |  | 2 | V_th |  |  | 5 |  | 130 |  |  |  |
| 52 | 57 |  | 2 | V_ION |  |  | 6 |  | 135 |  |  |  |
| 54 | 59 |  | 2 | V_swp1 |  |  | 7 |  | 140 |  |  |  |
| 56 | 61 |  | 2 | HV_mon |  | 5 |  |  | 145 |  |  |  |
| 58 | 63 |  | 2 | I_LED | 20 | 7 |  |  | 150 |  |  |  |
| 60 | 65 | HK set #4 | 2 | 30V |  |  | 0 |  | 155 |  |  |  |
| 62 | 67 |  | 2 | 10V |  |  | 1 |  | 160 |  |  |  |
| 64 | 69 |  | 2 | -5V |  |  | 2 |  | 165 |  |  |  |
| 66 | 71 |  | 2 | +5V |  |  | 3 |  | 170 |  |  |  |
| 68 | 73 |  | 2 | 3V3 |  |  | 4 |  | 175 |  |  |  |
| 70 | 75 |  | 2 | V_th |  |  | 5 |  | 180 |  |  |  |
| 72 | 77 |  | 2 | V_ION |  |  | 6 |  | 185 |  |  |  |
| 74 | 79 |  | 2 | V_swp1 |  |  | 7 |  | 190 |  |  |  |
| 76 | 81 |  | 2 | HV_mon |  | 5 |  |  | 195 |  |  |  |
| 78 | 83 |  | 2 | I_LED | 20 | 7 |  |  | 200 |  |  |  |
| 80 | 85 | HK set #5 | 2 | 30V |  |  | 0 |  | 205 |  |  |  |
| 82 | 87 |  | 2 | 10V |  |  | 1 |  | 210 |  |  |  |
| 84 | 89 |  | 2 | -5V |  |  | 2 |  | 215 |  |  |  |
| 86 | 91 |  | 2 | +5V |  |  | 3 |  | 220 |  |  |  |
| 88 | 93 |  | 2 | 3V3 |  |  | 4 |  | 225 |  |  |  |
| 90 | 95 |  | 2 | V_th |  |  | 5 |  | 230 |  |  |  |
| 92 | 97 |  | 2 | V_ION |  |  | 6 |  | 235 |  |  |  |
| 94 | 99 |  | 2 | V_swp1 |  |  | 7 |  | 240 |  |  |  |
| 96 | 101 |  | 2 | HV_mon |  | 5 |  |  | 245 |  |  |  |
| 98 | 103 |  | 2 | I_LED |  | 7 |  |  | 250 |  |  |  |
| 100 | 105 | HK set #6 | 2 | 30V |  |  | 0 |  | 255 |  |  |  |
| 102 | 107 |  | 2 | 10V |  |  | 1 |  | 260 |  |  |  |
| 104 | 109 |  | 2 | -5V |  |  | 2 |  | 265 |  |  |  |
| 106 | 111 |  | 2 | +5V |  |  | 3 |  | 270 |  |  |  |
| 108 | 113 |  | 2 | 3V3 |  |  | 4 |  | 275 |  |  |  |
| 110 | 115 |  | 2 | V_th |  |  | 5 |  | 280 |  |  |  |
| 112 | 117 |  | 2 | V_ION |  |  | 6 |  | 285 |  |  |  |
| 114 | 119 |  | 2 | V_swp1 |  |  | 7 |  | 290 |  |  |  |
| 116 | 121 |  | 2 | HV_mon |  | 5 |  |  | 295 |  |  |  |
| 118 | 123 |  | 2 | I_LED | 20 | 7 |  |  | 300 |  |  |  |
| 120 | 125 | HK set #7 | 2 | 30V |  |  | 0 |  | 305 |  |  |  |
| 122 | 127 |  | 2 | 10V |  |  | 1 |  | 310 |  |  |  |
| 124 | 129 |  | 2 | -5V |  |  | 2 |  | 315 |  |  |  |
| 126 | 131 |  | 2 | +5V |  |  | 3 |  | 320 |  |  |  |
| 128 | 133 |  | 2 | 3V3 |  |  | 4 |  | 325 |  |  |  |
| 130 | 135 |  | 2 | V_th |  |  | 5 |  | 330 |  |  |  |
| 132 | 137 |  | 2 | V_ION |  |  | 6 |  | 335 |  |  |  |
| 134 | 139 |  | 2 | V_swp1 |  |  | 7 |  | 340 |  |  |  |
| 136 | 141 |  | 2 | HV_mon |  | 5 |  |  | 345 |  |  |  |
| 138 | 143 |  | 2 | I_LED |  | 7 |  |  | 350 |  |  |  |
| 140 | 145 | HK set #8 | 2 | 30V |  |  | 0 |  | 355 |  |  |  |
| 142 | 147 |  | 2 | 10V |  |  | 1 |  | 360 |  |  |  |
| 144 | 149 |  | 2 | -5V |  |  | 2 |  | 365 |  |  |  |
| 146 | 151 |  | 2 | +5V |  |  | 3 |  | 370 |  |  |  |
| 148 | 153 |  | 2 | 3V3 |  |  | 4 |  | 375 |  |  |  |
| 150 | 155 |  | 2 | V_th |  |  | 5 |  | 380 |  |  |  |
| 152 | 157 |  | 2 | V_ION |  |  | 6 |  | 385 |  |  |  |
| 154 | 159 |  | 2 | V_swp1 |  |  | 7 |  | 390 |  |  |  |
| 156 | 161 |  | 2 | HV_mon |  | 5 |  |  | 395 |  |  |  |
| 158 | 163 |  | 2 | I_LED | 20 | 7 |  |  | 400 |  |  |  |
| 160 | 165 | 60 | 1 | NULL |  |  |  |  |  |  |  |  |
| 161 | 166 | 61 | 1 | NULL |  |  |  |  |  |  |  |  |
| 162 | 167 | 62 | 1 | NULL |  |  |  |  |  |  |  |  |
| 163 | 166 | 63 | 1 | NULL |  |  |  |  |  |  |  |  |
| 164 | 167 | 64 | 1 | NULL |  |  |  |  |  |  |  |  |
| 165 | 168 | 65 | 1 | NULL |  |  |  |  |  |  |  |  |
| 166 | 168 | 66 | 1 | NULL |  |  |  |  |  |  |  |  |
| 167 | 169 | 67 | 1 | NULL |  |  |  |  |  |  |  |  |
| 168 | 170 | 68 | 1 | NULL | 0 |  |  |  |  |  |  |  |

Table 20-5- - SU_HK Response Packet Format

Total HK Packet Size = 	174 Bytes

### HK Header Definition

The HK Header bytes are defined as:

| STATUS_REG |  | Comment (16-bit register) |
| --- | --- | --- |
| Cmd_Id | 00h | No Command ID |
|  | 02h | SU_RESET ID |
|  | 04h | SU_STIM ID |
|  | 05h | SU_LDP ID |
|  | 06h | SU_HC ID |
|  | 07h | SU_CAL ID |
|  | 08h | SU_SCI ID |
|  | 09h | SU_HK ID |
|  | 0Ah | SU_STM ID |
|  | 0Bh | SU_DUMP ID |
|  | 53h | SU_HVARM ID |
|  | C9h | SU_HVON ID |
|  | BBh | SU_ERR ID |
| Cmd_error_code | 00h | No error |
|  | D0h | Board ID read error |
|  | D1h | command length error |
|  | D2h | UART RX error |
|  | D3h | UART fifo FF (FIFO-FULL) error |
|  | D4h | UART TO (timeout) error |
|  | D5h | HVARM command error |
|  | D6h | HVON command error |
| Local_error_code | 00h | HK_NO_ERR |
|  | A0h | HK_PKT_ERR (error in HK packet detected) |

Table 20-6- – HK Error Codes Definition

## SU_STM (0x0A) response packet format:

|  |  |  |  | STM format |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte Addr |  | Hdr \ Data Field |  | name | bytes | Reg Name |  | Comment |  |
| 0 |  | B0 |  | RSP_ID | 1 | SU_STM = 0x0A |  |  |  |
| 1 |  | B1 |  | SEQ_CNT | 1 |  |  |  |  |
| 2 |  | 0 |  | set #1 | 2 | STM_0 |  | UIN16 - Little Endian format (low byte first) |  |
| 4 |  | 2 |  |  | 2 | STM_1 |  |  |  |
| 6 |  | 4 |  |  | 2 | STM_2 |  |  |  |
| 8 |  | 6 |  |  | 2 | STM_3 |  |  |  |
| 10 |  | 8 |  |  | 2 | STM_4 |  |  |  |
| 12 |  | 10 |  |  | 2 | STM_5 |  | 25 |  |
| 14 |  | 12 |  | set #2 | 2 | STM_0 |  |  |  |
| 16 |  | 14 |  |  | 2 | STM_1 |  |  |  |
| 18 |  | 16 |  |  | 2 | STM_2 |  |  |  |
| 20 |  | 18 |  |  | 2 | STM_3 |  |  |  |
| 22 |  | 20 |  |  | 2 | STM_4 |  |  |  |
| 24 |  | 22 |  |  | 2 | STM_5 |  | 50 |  |
| 26 |  | 24 |  | set #3 | 2 | STM_0 |  |  |  |
| 28 |  | 26 |  |  | 2 | STM_1 |  |  |  |
| 30 |  | 28 |  |  | 2 | STM_2 |  |  |  |
| 32 |  | 30 |  |  | 2 | STM_3 |  |  |  |
| 34 |  | 32 |  |  | 2 | STM_4 |  |  |  |
| 36 |  | 34 |  |  | 2 | STM_5 |  | 75 |  |
| 38 |  | 36 |  | set #4 | 2 | STM_0 |  |  |  |
| 40 |  | 38 |  |  | 2 | STM_1 |  |  |  |
| 42 |  | 40 |  |  | 2 | STM_2 |  |  |  |
| 44 |  | 42 |  |  | 2 | STM_3 |  |  |  |
| 46 |  | 44 |  |  | 2 | STM_4 |  |  |  |
| 48 |  | 46 |  |  | 2 | STM_5 |  | 100 |  |
| 50 |  | 48 |  | set #5 | 2 | STM_0 |  |  |  |
| 52 |  | 50 |  |  | 2 | STM_1 |  |  |  |
| 54 |  | 52 |  |  | 2 | STM_2 |  |  |  |
| 56 |  | 54 |  |  | 2 | STM_3 |  |  |  |
| 58 |  | 56 |  |  | 2 | STM_4 |  |  |  |
| 60 |  | 58 |  |  | 2 | STM_5 |  | 125 |  |
| 62 |  | 60 |  | set #6 | 2 | STM_0 |  |  |  |
| 64 |  | 62 |  |  | 2 | STM_1 |  |  |  |
| 66 |  | 64 |  |  | 2 | STM_2 |  |  |  |
| 68 |  | 66 |  |  | 2 | STM_3 |  |  |  |
| 70 |  | 68 |  |  | 2 | STM_4 |  |  |  |
| 72 |  | 70 |  |  | 2 | STM_5 |  | 150 |  |
| 74 |  | 72 |  | set #7 | 2 | STM_0 |  |  |  |
| 76 |  | 74 |  |  | 2 | STM_1 |  |  |  |
| 78 |  | 76 |  |  | 2 | STM_2 |  |  |  |
| 80 |  | 78 |  |  | 2 | STM_3 |  |  |  |
| 82 |  | 80 |  |  | 2 | STM_4 |  |  |  |
| 84 |  | 82 |  |  | 2 | STM_5 |  | 175 |  |
| 86 |  | 84 |  | set #8 | 2 | STM_0 |  |  |  |
| 88 |  | 86 |  |  | 2 | STM_1 |  |  |  |
| 90 |  | 88 |  |  | 2 | STM_2 |  |  |  |
| 92 |  | 90 |  |  | 2 | STM_3 |  |  |  |
| 94 |  | 92 |  |  | 2 | STM_4 |  |  |  |
| 96 |  | 94 |  |  | 2 | STM_5 |  | 200 |  |
| 98 |  | 96 |  | set #9 | 2 | STM_0 |  |  |  |
| 100 |  | 98 |  |  | 2 | STM_1 |  |  |  |
| 102 |  | 100 |  |  | 2 | STM_2 |  |  |  |
| 104 |  | 102 |  |  | 2 | STM_3 |  |  |  |
| 106 |  | 104 |  |  | 2 | STM_4 |  |  |  |
| 108 |  | 106 |  |  | 2 | STM_5 |  | 225 |  |
| 110 |  | 108 |  | set #10 | 2 | STM_0 |  |  |  |
| 112 |  | 110 |  |  | 2 | STM_1 |  |  |  |
| 114 |  | 112 |  |  | 2 | STM_2 |  |  |  |
| 116 |  | 114 |  |  | 2 | STM_3 |  |  |  |
| 118 |  | 116 |  |  | 2 | STM_4 |  |  |  |
| 120 |  | 118 |  |  | 2 | STM_5 |  | 250 |  |
| 122 |  | 120 |  | set #11 | 2 | STM_0 |  |  |  |
| 124 |  | 122 |  |  | 2 | STM_1 |  |  |  |
| 126 |  | 124 |  |  | 2 | STM_2 |  |  |  |
| 128 |  | 126 |  |  | 2 | STM_3 |  |  |  |
| 130 |  | 128 |  |  | 2 | STM_4 |  |  |  |
| 132 |  | 130 |  |  | 2 | STM_5 |  | 275 |  |
| 134 |  | 132 |  | set #12 | 2 | STM_0 |  |  |  |
| 136 |  | 134 |  |  | 2 | STM_1 |  |  |  |
| 138 |  | 136 |  |  | 2 | STM_2 |  |  |  |
| 140 |  | 138 |  |  | 2 | STM_3 |  |  |  |
| 142 |  | 140 |  |  | 2 | STM_4 |  |  |  |
| 144 |  | 142 |  |  | 2 | STM_5 |  | 300 |  |
| 146 |  | 144 |  | set #13 | 2 | STM_0 |  |  |  |
| 148 |  | 146 |  |  | 2 | STM_1 |  |  |  |
| 150 |  | 148 |  |  | 2 | STM_2 |  |  |  |
| 152 |  | 150 |  |  | 2 | STM_3 |  |  |  |
| 154 |  | 152 |  |  | 2 | STM_4 |  |  |  |
| 156 |  | 154 |  |  | 2 | STM_5 |  | 325 |  |
| 158 |  | 156 |  | set #14 | 2 | STM_0 |  |  |  |
| 160 |  | 158 |  |  | 2 | STM_1 |  |  |  |
| 162 |  | 160 |  |  | 2 | STM_2 |  |  |  |
| 164 |  | 162 |  |  | 2 | STM_3 |  |  |  |
| 166 |  | 164 |  |  | 2 | STM_4 |  |  |  |
| 168 |  | 166 |  |  | 2 | STM_5 |  | 350 |  |
| 170 |  | 168 |  |  | 1 | NULL |  |  |  |
| 171 |  | 169 |  |  | 1 | NULL |  |  |  |
| 172 |  | 170 |  |  | 1 | NULL |  |  |  |
| 173 |  | 171 |  |  | 1 | NULL |  |  |  |

Table 20-7- - SU_STM Response Packet Format

Total STM Packet Size = 	174 Bytes

## SU_DUMP(0x0B) response packet format:

The SU_DUMP response packet includes the current HK parameters, and the complete LUT in RAM

|  |  |  | RSP_DUMP format |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Byte Addr | Hdr \ Data Field | name | bytes | Reg Name | Comment |  |  |  |  |  |  |  |
|  | b0 | 0 | RSP_ID | 1 | SU_DUMP = 0x0B |  |  |  |  |  |  |  |  |
|  | b1 | 1 | SEQ_CNT | 1 |  |  |  |  |  |  |  |  |  |
|  | b2 | 2 |  | 1 | cmd_id_err | UINT16 - Little Endian format (low byte first) |  |  |  |  |  |  |  |
|  | b3 | 3 |  | 1 | cmd_err_code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | b4 | 4 |  | 1 | local_cmd_err |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 0 | 5 | HK set #1 | 2 | 30V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | 7 |  | 2 | 10V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | 9 |  | 2 | -5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 | 11 |  | 2 | +5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 | 13 |  | 2 | 3V3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | 15 |  | 2 | V_th |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12 | 17 |  | 2 | V_ION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14 | 19 |  | 2 | V_swp1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 16 | 21 |  | 2 | HV_mon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 | 23 |  | 2 | I_LED | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 20 | 25 | HK set #2 | 2 | 30V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 22 | 27 |  | 2 | 10V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 24 | 29 |  | 2 | -5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 26 | 31 |  | 2 | +5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 | 33 |  | 2 | 3V3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 | 35 |  | 2 | V_th |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 32 | 37 |  | 2 | V_ION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 34 | 39 |  | 2 | V_swp1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 36 | 41 |  | 2 | HV_mon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 38 | 43 |  | 2 | I_LED | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 40 | 45 | HK set #3 | 2 | 30V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 42 | 47 |  | 2 | 10V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 | 49 |  | 2 | -5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 46 | 51 |  | 2 | +5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 48 | 53 |  | 2 | 3V3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50 | 55 |  | 2 | V_th |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 | 57 |  | 2 | V_ION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 54 | 59 |  | 2 | V_swp1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 | 61 |  | 2 | HV_mon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 58 | 63 |  | 2 | I_LED | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 | 65 | HK set #4 | 2 | 30V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 62 | 67 |  | 2 | 10V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 64 | 69 |  | 2 | -5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 71 |  | 2 | +5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 68 | 73 |  | 2 | 3V3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 70 | 75 |  | 2 | V_th |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 72 | 77 |  | 2 | V_ION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 74 | 79 |  | 2 | V_swp1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 76 | 81 |  | 2 | HV_mon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 78 | 83 |  | 2 | I_LED | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50 | 80 | 85 | HK set #5 | 2 | 30V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 82 | 87 |  | 2 | 10V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 84 | 89 |  | 2 | -5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 86 | 91 |  | 2 | +5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 88 | 93 |  | 2 | 3V3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 90 | 95 |  | 2 | V_th |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 92 | 97 |  | 2 | V_ION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 94 | 99 |  | 2 | V_swp1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 96 | 101 |  | 2 | HV_mon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 98 | 103 |  | 2 | I_LED/V_LED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 64H | 100 | 105 | LUT < 0 > | 1 | hv_step_hv |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 101 | 106 | 1 | 1 | t_step_hv |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 | 107 | 2 | 2 | v_hv_max_L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 103 | 108 | 3 |  | v_hv_max_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 104 | 109 | 4 | 2 | v_hv_min_L | b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |  |
|  | 105 | 110 | 5 |  | v_hv_min_H | End | Bswp | Ipkt | Mode | Iavg | sw3 | sw2 | sw1 |  |
|  | 106 | 111 | 6 | 1 | sw_N0_ana | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |  |
|  | 107 | 112 | 7 | 1 | sw_I0_ana | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  |
|  | 108 | 113 | 8 | 1 | sw_N1_ana | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |  |
|  | 109 | 114 | 9 | 1 | sw_I1_ana | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  |
|  | 110 | 115 | 10 | 1 | sw_N2_ana | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 |  |
|  | 111 | 116 | 11 | 1 | sw_I2_ana | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  |
|  | 112 | 117 | 12 | 1 | n_step_F |  |  |  |  |  |  |  |  |  |
|  | 113 | 118 | 13 | 1 | X%_F |
|  | 114 | 119 | 14 | 2 | K_V_IONF_swp_L |
|  | 115 | 120 | 15 |  | K_V_IONF_swp_H |
|  | 116 | 121 | 16 | 1 | t_step_F |
|  | 117 | 122 | 17 | 2 | V_LED_max_L |
|  | 118 | 123 | 18 |  | V_LED_max_H |
|  | 119 | 124 | 19 | 2 | V_IONL |
|  | 120 | 125 | 20 |  | V_IONH |
|  | 121 | 126 | 21 | 2 | V_THL |
|  | 122 | 127 | 22 |  | V_THH |
|  | 123 | 128 | 23 | 2 | V_TH_stepL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 124 | 129 | 24 |  | V_TH_stepH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 125 | 130 | 25 | 2 | VLED_L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 126 | 131 | 26 |  | VLED_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 127 | 132 | 27 | 1 | n_step_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 128 | 133 | 28 | 1 | X%_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 129 | 134 | 29 | 1 | t_step_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 130 | 135 | 30 | 2 | V_exp_ N0_B0L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 131 | 136 | 31 |  | V_exp_ N0_B0H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 132 | 137 | 32 | 2 | V_exp_ N0_B1L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 133 | 138 | 33 |  | V_exp_ N0_B1H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 134 | 139 | 34 | 2 | V_exp_ N0_B2L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 135 | 140 | 35 |  | V_exp_ N0_B2H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 136 | 141 | 36 | 2 | V_exp_ I0_B0L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 137 | 142 | 37 |  | V_exp_ I0_BH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 138 | 143 | 38 | 2 | V_exp_ I0_B1L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 139 | 144 | 39 |  | V_exp_ I0_B1H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 140 | 145 | 40 | 2 | V_exp_ I0_B2L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 141 | 146 | 41 |  | V_exp_ I0_B2H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 142 | 147 | 42 | 4 | File_S/N_B0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 143 | 148 | 43 |  | File_S/N_B1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 144 | 149 | 44 |  | File_S/N_B2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 145 | 150 | 45 |  | File_S/N_B3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 146 | 151 | 46 | 2 | Scr_Hdr_IDL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 93 | 147 | 152 | 47 |  | Scr_Hdr_IDH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 148 | 153 | 48 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 149 | 154 | 49 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 150 | 155 | 50 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 151 | 156 | 51 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | 152 | 157 | 52 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | 153 | 158 | 53 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | 154 | 159 | 54 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 155 | 160 | 55 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | 156 | 161 | 56 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | 157 | 162 | 57 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | 158 | 163 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | 159 | 164 | 59 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 160 | 165 | 60 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 161 | 166 | 61 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 162 | 167 | 62 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 163 | 168 | 63 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 164 | 169 | 64 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | 165 | 170 | 65 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | 166 | 171 | 66 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | 167 | 172 | 67 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21 | 168 | 173 | 68 | 1 | NULL | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 20-8- - SU_DUMP Response Packet Format

Total HK Packet Size = 	174 Bytes

## SU_ERR(0xBB) response packet format:

The SU_ERR response packet format is the same as SU_DUMP: repeated here:

|  |  | RSP_ERR format |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte Addr | Hdr \ Data Field | name | bytes | Reg Name | Comment |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b0 | 0 | RSP_ID | 1 | SU_ERR = 0xBB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b1 | 1 | SEQ_CNT | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b2 | 2 |  | 1 | cmd_id_err | UNIT16 - Little Endian format (low byte first) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b3 | 3 |  | 1 | cmd_err_code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b4 | 4 |  | 1 | local_cmd_err |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 5 | HK set #1 | 2 | 30V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | 7 |  | 2 | 10V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 9 |  | 2 | -5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | 11 |  | 2 | +5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 13 |  | 2 | 3V3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | 15 |  | 2 | V_th |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | 17 |  | 2 | V_ION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 19 |  | 2 | V_swp1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 21 |  | 2 | HV_mon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | 23 |  | 2 | I_LED | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | 25 | HK set #2 | 2 | 30V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 | 27 |  | 2 | 10V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 | 29 |  | 2 | -5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 | 31 |  | 2 | +5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | 33 |  | 2 | 3V3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | 35 |  | 2 | V_th |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32 | 37 |  | 2 | V_ION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34 | 39 |  | 2 | V_swp1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36 | 41 |  | 2 | HV_mon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 38 | 43 |  | 2 | I_LED | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 | 45 | HK set #3 | 2 | 30V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 42 | 47 |  | 2 | 10V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 44 | 49 |  | 2 | -5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 46 | 51 |  | 2 | +5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 48 | 53 |  | 2 | 3V3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50 | 55 |  | 2 | V_th |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 52 | 57 |  | 2 | V_ION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 54 | 59 |  | 2 | V_swp1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 56 | 61 |  | 2 | HV_mon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 58 | 63 |  | 2 | I_LED | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 60 | 65 | HK set #4 | 2 | 30V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 62 | 67 |  | 2 | 10V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 64 | 69 |  | 2 | -5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 66 | 71 |  | 2 | +5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 68 | 73 |  | 2 | 3V3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 70 | 75 |  | 2 | V_th |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 72 | 77 |  | 2 | V_ION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 74 | 79 |  | 2 | V_swp1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 76 | 81 |  | 2 | HV_mon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 78 | 83 |  | 2 | I_LED | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 80 | 85 | HK set #5 | 2 | 30V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 82 | 87 |  | 2 | 10V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 84 | 89 |  | 2 | -5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 86 | 91 |  | 2 | +5V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 88 | 93 |  | 2 | 3V3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 90 | 95 |  | 2 | V_th |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 92 | 97 |  | 2 | V_ION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 94 | 99 |  | 2 | V_swp1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 96 | 101 |  | 2 | HV_mon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 98 | 103 |  | 2 | I_LED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 100 | 105 | LUT < 0 > | 1 | hv_step_hv |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 101 | 106 | 1 | 1 | t_step_hv |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 102 | 107 | 2 | 2 | v_hv_max_L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 103 | 108 | 3 |  | v_hv_max_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 104 | 109 | 4 | 2 | v_hv_min_L | b7 | b6 |  | b5 |  | b4 | b3 |  | b2 |  | b1 |  | b0 |  |
| 105 | 110 | 5 |  | v_hv_min_H | end | BSwp |  | Ipkt |  | Mode | Iavg |  | sw3 |  | sw2 |  | sw1 |  |
| 106 | 111 | 6 | 1 | sw_N0_ana | 0 | 1 |  | 1 |  | 1 | 1 |  | 0 |  | 0 |  | 1 |  |
| 107 | 112 | 7 | 1 | sw_I0_ana | 0 | 1 |  | 0 |  | 0 | 0 |  | 0 |  | 0 |  | 1 |  |
| 108 | 113 | 8 | 1 | sw_N1_ana | 0 | 0 |  | 1 |  | 1 | 1 |  | 0 |  | 1 |  | 0 |  |
| 109 | 114 | 9 | 1 | sw_I1_ana | 0 | 0 |  | 0 |  | 0 | 0 |  | 0 |  | 1 |  | 0 |  |
| 110 | 115 | 10 | 1 | sw_N2_ana | 0 | 0 |  | 1 |  | 1 | 1 |  | 1 |  | 0 |  | 0 |  |
| 111 | 116 | 11 | 1 | sw_I2_ana | 1 | 0 |  | 0 |  | 0 | 0 |  | 1 |  | 1 |  | 0 |  |
| 112 | 117 | 12 | 1 | n_step_F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 113 | 118 | 13 | 1 | X%_F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 114 | 119 | 14 | 2 | K_V_IONF_swp_L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 115 | 120 | 15 |  | K_V_IONF_swp_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 116 | 121 | 16 | 1 | t_step_F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 117 | 122 | 17 | 2 | V_LED_max_L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 118 | 123 | 18 |  | V_LED_max_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 119 | 124 | 19 | 2 | V_IONL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 120 | 125 | 20 |  | V_IONH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 121 | 126 | 21 | 2 | V_THL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 122 | 127 | 22 |  | V_THH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 123 | 128 | 23 | 2 | V_TH_stepL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 124 | 129 | 24 |  | V_TH_stepH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 125 | 130 | 25 | 2 | VLED_L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 126 | 131 | 26 |  | VLED_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 127 | 132 | 27 | 1 | n_step_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 128 | 133 | 28 | 1 | X%_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 129 | 134 | 29 | 1 | t_step_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 130 | 135 | 30 | 2 | V_exp_ N0_B0L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 131 | 136 | 31 |  | V_exp_ N0_B0H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 132 | 137 | 32 | 2 | V_exp_ N0_B1L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 133 | 138 | 33 |  | V_exp_ N0_B1H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 134 | 139 | 34 | 2 | V_exp_ N0_B2L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 135 | 140 | 35 |  | V_exp_ N0_B2H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 136 | 141 | 36 | 2 | V_exp_ I0_B0L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 137 | 142 | 37 |  | V_exp_ I0_BH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 138 | 143 | 38 | 2 | V_exp_ I0_B1L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 139 | 144 | 39 |  | V_exp_ I0_B1H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 140 | 145 | 40 | 2 | V_exp_ I0_B2L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 141 | 146 | 41 |  | V_exp_ I0_B2H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 142 | 147 | 42 | 4 | File_S/N_B0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 143 | 148 | 43 |  | File_S/N_B1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 144 | 149 | 44 |  | File_S/N_B2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 145 | 150 | 45 |  | File_S/N_B3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 146 | 151 | 46 | 2 | Scr_Hdr_IDL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 147 | 152 | 47 |  | Scr_Hdr_IDH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 148 | 153 | 48 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 149 | 154 | 49 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 150 | 155 | 50 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 151 | 156 | 51 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 152 | 157 | 52 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 153 | 158 | 53 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 154 | 159 | 54 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 155 | 160 | 55 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 156 | 161 | 56 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 157 | 162 | 57 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 158 | 163 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 159 | 164 | 59 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 160 | 165 | 60 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 161 | 166 | 61 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 162 | 167 | 62 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 163 | 168 | 63 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 164 | 169 | 64 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 165 | 170 | 65 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 166 | 171 | 66 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 167 | 172 | 67 | 1 | NULL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 168 | 173 | 68 | 1 | NULL | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 20-9- – SU_ERR Response Packet Format

Total SU_ERR Packet Size = 	174 Bytes

### SU_ERR Error Codes Definition

The SU_ERR error codes are defined as:

| STATUS_REG |  | Comment (16-bit register) |
| --- | --- | --- |
| Cmd_Id | 00h | No Command ID |
|  | 02h | SU_RESET ID |
|  | 04h | SU_STIM ID |
|  | 05h | SU_LDP ID |
|  | 06h | SU_HC ID |
|  | 07h | SU_CAL ID |
|  | 08h | SU_SCI ID |
|  | 09h | SU_HK ID |
|  | 0Ah | SU_STM ID |
|  | 0Bh | SU_DUMP ID |
|  | 53h | SU_HVARM ID |
|  | C9h | SU_HVON ID |
|  | BBh | SU_ERR ID |
| Cmd_error_code | 00h | No error |
|  | D0h | Board ID read error |
|  | D1h | command length error |
|  | D2h | UART RX error |
|  | D3h | UART fifo FF (FIFO-FULL) error |
|  | D4h | UART TO (timeout) error |
|  | D5h | HVARM command error |
|  | D6h | HVON command error |
| Local_error_code | 00h | SU_NO_ERROR |
|  | EAh | Error in SU_ERR state machine |
|  | EBh | Error in from HV control |
|  | ECh | Error in SU_SCI command |
|  | EDh | Error in SU_HC command |
|  | EEh | Error in SU_CAL command |

Table 20-10- – SU_ERR Error Code Definition

## OBC_SU(0xFA)_ERR response packet format:

|  |  | OBC_SU_ERR format |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte Addr | Hdr \ Data Field | name | bytes | Reg Name | Comment |  |  |  |  |  |  |  |
| b0 | 0 | RSP_ID | 1 | OBC_SU_ERR = 0xFA |  |  |  |  |  |  |  |  |
| b1 | 1 | SEQ_CNT | 1 |  |  |  |  |  |  |  |  |  |
| 0 | 2 |  | 1 | rsp_err_code | UNIT16 - Little Endian format (low byte first) |  |  |  |  |  |  |  |
| 1 | 3 | Script #running | 2 | XSUM_R |  |  |  |  |  |  |  |  |
| 3 | 5 | HDR #Running | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |
| 4 | 6 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |
| 5 | 7 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |
| 6 | 8 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |
| 7 | 9 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |
| 8 | 10 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |
| 9 | 11 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |
| 10 | 12 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |
| 11 | 13 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |
| 12 | 14 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |
| 13 | 15 | Script #0 | 2 | XSUM_0 |  |  |  |  |  |  |  |  |
| 15 | 17 | HDR #0 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |
| 16 | 18 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |
| 17 | 19 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |
| 18 | 20 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |
| 19 | 21 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |
| 20 | 22 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |
| 21 | 23 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |
| 22 | 24 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |
| 23 | 25 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |
| 24 | 26 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |
| 25 | 27 | Script #1 | 2 | XSUM_1 |  |  |  |  |  |  |  |  |
| 27 | 29 | HDR #1 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |
| 28 | 30 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |
| 29 | 31 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |
| 30 | 32 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |
| 31 | 33 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |
| 32 | 34 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |
| 33 | 35 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |
| 34 | 36 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |
| 35 | 37 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |
| 36 | 38 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |
| 37 | 39 | Script #2 | 2 | XSUM_2 |  |  |  |  |  |  |  |  |
| 39 | 41 | HDR #2 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |
| 40 | 42 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |
| 41 | 43 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |
| 42 | 44 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |
| 43 | 45 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |
| 44 | 46 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |
| 45 | 47 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |
| 46 | 48 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |
| 47 | 49 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |
| 48 | 50 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |
| 49 | 51 | Script #3 | 2 | XSUM_3 |  |  |  |  |  |  |  |  |
| 51 | 53 | HDR #3 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |
| 52 | 54 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |
| 53 | 55 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |
| 54 | 56 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |
| 55 | 57 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |
| 56 | 58 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |
| 57 | 59 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |
| 58 | 60 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |
| 59 | 61 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |
| 60 | 62 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |
| 61 | 63 | Script #4 | 2 | XSUM_4 |  |  |  |  |  |  |  |  |
| 63 | 65 | HDR #4 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |
| 64 | 66 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |
| 65 | 67 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |
| 66 | 68 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |
| 67 | 69 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |
| 68 | 70 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |
| 69 | 71 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |
| 70 | 72 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |
| 71 | 73 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |
| 72 | 74 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |
| 73 | 75 | Script #5 | 2 | XSUM_5 |  |  |  |  |  |  |  |  |
| 75 | 77 | HDR #5 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |
| 76 | 78 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |
| 77 | 79 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |
| 78 | 80 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |
| 79 | 81 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |
| 80 | 82 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |
| 81 | 83 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |
| 82 | 84 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |
| 83 | 85 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |
| 84 | 86 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |
| 85 | 87 | Script #6 | 2 | XSUM_6 |  |  |  |  |  |  |  |  |
| 87 | 89 | HDR #6 | 1 | UTC_start_0 |  |  |  |  |  |  |  |  |
| 88 | 90 |  | 1 | UTC_start_1 |  |  |  |  |  |  |  |  |
| 89 | 91 |  | 1 | UTC_start_2 |  |  |  |  |  |  |  |  |
| 90 | 92 |  | 1 | UTC_start_3 |  |  |  |  |  |  |  |  |
| 91 | 93 |  | 1 | hdr_sn_0 |  |  |  |  |  |  |  |  |
| 92 | 94 |  | 1 | hdr_sn_1 |  |  |  |  |  |  |  |  |
| 93 | 95 |  | 1 | hdr_sn_2 |  |  |  |  |  |  |  |  |
| 94 | 96 |  | 1 | hdr_sn_3 |  |  |  |  |  |  |  |  |
| 95 | 97 |  | 1 | SW_ver+SU_ID |  |  |  |  |  |  |  |  |
| 96 | 98 |  | 1 | type+SU_MD | 12 |  |  |  |  |  |  |  |
| 97 | 99 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 98 | 100 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 99 | 101 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 100 | 102 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 101 | 103 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 102 | 104 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 103 | 105 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 104 | 106 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 105 | 107 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 106 | 108 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 107 | 109 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 108 | 110 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 109 | 111 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 110 | 112 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 111 | 113 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 112 | 114 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 113 | 115 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 114 | 116 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 115 | 117 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 116 | 118 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 117 | 119 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 118 | 120 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 119 | 121 | 58 | 1 | NULL |  |  |  |  |  |  |  |  |
| 120 | 122 | 59 | 1 | NULL |  |  |  |  |  |  |  |  |
| 121 | 123 | 60 | 1 | NULL |  |  |  |  |  |  |  |  |
| 122 | 124 | 61 | 1 | NULL |  |  |  |  |  |  |  |  |
| 123 | 125 | 62 | 1 | NULL |  |  |  |  |  |  |  |  |
| 124 | 126 | 63 | 1 | NULL |  |  |  |  |  |  |  |  |
| 125 | 127 | 64 | 1 | NULL |  |  |  |  |  |  |  |  |
| 126 | 128 | 65 | 1 | NULL |  |  |  |  |  |  |  |  |
| 127 | 129 | 66 | 1 | NULL |  |  |  |  |  |  |  |  |
| 128 | 130 | 67 | 1 | NULL |  |  |  |  |  |  |  |  |
| 129 | 131 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 130 | 132 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 131 | 133 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 132 | 134 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 133 | 135 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 134 | 136 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 135 | 137 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 136 | 138 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 137 | 139 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 138 | 140 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 139 | 141 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 140 | 142 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 141 | 143 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 142 | 144 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 143 | 145 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 144 | 146 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 145 | 147 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 146 | 148 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 147 | 149 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 148 | 150 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 149 | 151 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 150 | 152 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 151 | 153 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 152 | 154 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 153 | 155 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 154 | 156 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 155 | 157 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 156 | 158 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 157 | 159 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 158 | 160 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 159 | 161 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 160 | 162 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 161 | 163 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 162 | 164 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 163 | 165 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 164 | 166 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 165 | 167 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 166 | 168 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 167 | 169 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 168 | 170 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 169 | 171 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 170 | 172 | 68 | 1 | NULL |  |  |  |  |  |  |  |  |
| 171 | 173 | 68 | 1 | NULL | 63 |  |  |  |  |  |  |  |

Table 20-11- - OBC_SU_ERR response packet format

Total OBC_SU_ERR Packet Size = 174 Bytes