SWIFT UVOT Detector Safety Circuit Specification

MSSL/SWT-UVOT/SP/006.01

17 September 2001

B.K.Hancock

Table of Contents

1. Introduction	3

2. Top Level Requirements	3

4. Safety Circuit Description	3

4.1 The Detector Rate Monitor	5

4.2 The Saturated Pixel Detector	5

4.3 ICB Interface	6

Detector Safety Circuit Register allocation	6

WRITE register 1 ICB sub  address 0x0	6

WRITE  register 2 ICB sub address 0x1	6

READ register 1 ICB sub address 0x0	6

READ register 1 ICB sub address 0x0	7

READ register 2 ICB address 0x1	7

READ register 2 ICB address 0x1	7

4.4 Cathode Safe Signal	7

# 1. Introduction

The information in this specification is drawn from the documents shown below and discussions and e-mail correspondence with Tom Kennedy and Hajime Kowakami and Phil Thomas.

SWIFT UVOT Bright Source  safety Philosophy Issue 1

Blue detector processing electronics detailed design  XMM-OM/MSSL/SP/0056.03

# 2. Top Level Requirements

- Protect detector system against bright sources

- Immune to false triggers due to penetrating radiation in the detector system

- Fast reaction time to prevent damage to detector system

- Fit within existing UVOT TM electronics architecture

- Must not affect system performance

- Must be controlled by ICU

- Flexibility to cope with changing circumstances

- Monitor detector count rates

- Must be able to be calibrated inflight

- Must be able to be TURNED OFF

3. Operational Philosophy

- Use characteristics of optics to detect bright sources, the brighter the source the greater flair is observed

- Use raw CCD output to detect consecutive pixels above a preset threshold

- Count pixels above a preset threshold

- Alert on multiple consecutive frames with consecutive pixels to counter false triggers due to penetrating radiation

- Safe detector system by setting cathode voltage to 0V (this gives a detector gain reduction of ~10,000)

# 4. Safety Circuit Description

The purpose of the Detector safety circuit is to protect the UVOT detector system against damage from prolonged exposure to bright sources. This is achieved by two methods:

- monitoring the number of pixels above a preset threshold for a fixed integration period. This counter will be accessable by the ICU via the ICB.

- Detecting a preset number of consecutive frames with a preset number of consecutive pixels above a preset threshold.

Only valid pixels are counted by the safety circuit, the ~IRUN signal is used to detect the start of a frame and ~HRUN signal is used to determine when valid data is clocked out of the CCD by the horizontal clock. Data from the DUMP period is not counted by the safety circuit. This is achieved by ignoring the first ~HRUN period following a new frame.

The safety circuit is designed to deal with single full frame windows, if small or multiple windows are selected, the safety circuit will not function as required.

The UVOT safety circuit consists of three parts:

- The detector rate monitor

- Saturated pixel detector

- BPE interface

The most significant 5bits of video are used in the safety circuit, the 3 LS bit are treated as don’t care.

The detector rate monitor and saturated pixel detector count only valid pixels. CCD dump phase after frame transfer is ignored and subsequent video data is enabled by the ~HRUN signal.

# 4.1 The Detector Rate Monitor

The rate monitor counts the number of above threshold pixels in a predetermined time period. The rate monitor is asynchronously sampled by the ICU via the MACS bus interface. The value read is the value from the previous sample period. The counter is sized to 24bits to overcome the problem of rollover at high-count rates. The value read back from the MACS bus is reduced to 23 bits to simplify bit packing in the read register. The means that the data bits transmitted are [23:1] this has the effect of dividing the output value by 2. The ICU should compensate for this.

In order to avoid incorrect values by sampling during the counter update period the ICU should sample at least 3 times (within a few hundred micro seconds) and only accept a value which occurs twice during the sample period.

Specification

|  | Fixed integration time | 0.7 seconds | 63 frame periods |
| --- | --- | --- | --- |
| Input | Input pixel threshold | 5 bits |  |
| Output (read back) | Input threshold | 5 bits |  |
|  | counter | 24 bits | From previous integration period |
|  |  | 23bits | read register |

# 4.2 The Saturated Pixel Detector

The saturated pixel detector counts the number of consecutive pixels above a threshold, and if these events occur for more than a predetermined number of frames then the alert output activates.  Writing to the control register controls the detector. The control functions are; alert reset, system enable and alert enable, see register allocation for more detail.

Specification

| Input | Input threshold | 5bits | Most significant |
| --- | --- | --- | --- |
|  | Consecutive pixels | 7bits |  |
|  | Consecutive frames | 5bits |  |
|  | Alert reset | 1 bit |  |
|  | System enable | 1 bit |  |
|  | Alert enable | 1 bit |  |
|  | Frame sync | 1 | ~IRUN |
|  | Horizontal read out control | 1 | ~HRUN |
| Output (read back ) | Input threshold | 5bits |  |
|  | Consecutive pixel preset | 7bits |  |
|  | Consecutive frame preset | 5bits |  |
|  | System reset | 1 bit |  |
|  | Alert reset | 1 bit |  |
|  | Alert flag output | 1 bit |  |
|  | System enable | 1 bit |  |
|  | Alert enable | 1 bit |  |
| Output | Cathode safe signal | 1 channel | link to monitor board |

# Control Signal Summary

# System enable

This function enables both rate monitor and the saturated pixel detector. When asserted the command  is internally synchronised to the next ~IRUN signal, allowing the system to start predictably.

Alert flag

This read register signal indicates that the SPD has fired, the cathode safe signal is asserted if the alert enable bit is set.

# Alert reset

This function resets the cathode safe signal register to its powerup default state “off” (allowing control of the cathode voltage) and the alert flag would return a  zero .  The SPD circuit is also reset to its powerup default state. Control register functions are not affected by this signal, any previous state will be maintained until being rewritten or reset by power cycling.

# Alert enable

This function enables the cathode safe signal register. The alert flag is not affected by this control. This allows for on orbit calibration of the system.

Typical operational sequence (after power up)

Set up SPD and ratemeter pixel thresholds (register 2)

Setup consecutive pixel and frame counts (register 1)

Read back and verify

Set system enable and alert enable bits.

Read back and verify

Powerup detector

Read ratemeter at regular intervals

Read alertflag bit at regular intervals

If alert flag =1

# 4.3 ICB Interface

The ICB interface uses the decoded MACS parallel bus from the BPE camera board. Two read and two write strobes are available. The implementation is shown below.

Read registers alternate between two possible sources, the source is identified by the REGID bit.

Memory Map

Read Addresses

| ICB address | Sub address | Function |
| --- | --- | --- |
| 0x18 | 0x0 | Read register 1 |
|  | 0x1 | Read register 2 |

Write Addresses

| ICB address | Sub address | Function |
| --- | --- | --- |
| 0x18 | 0x0 | Write register 1 |
|  | 0x1 | Write register 2 |

# Detector Safety Circuit Register allocation

| # WRITE register 1 ICB sub address 0x0 |  |  |
| --- | --- | --- |
| B15 | N/A | Not assigned |
| B14-B8 | PCTHOLD[6:0] | Consecutive pixel count |
| B7 | OPRST | Output reset 1=reset |
| B6 | SYSEN | System enable 1=enable |
| B5 | ALERTEN | Cathode control 1=enable |
| B4-B0 | FCTHOLD[4:0] | Consecutive frame count |

| # WRITE register 2 ICB sub address 0x1 |  |  |
| --- | --- | --- |
| B12-B8 | STHOLD[4:0] | Pixel threshold |
| B4-B0 | RTHOLD[4:0] | Ratemeter pixel threshold |

| # READ register 1 ICB sub address 0x0 |  |  |
| --- | --- | --- |
| REGID=0 |  |  |
| B15 | REGID | Register identification bit |
| B14-B8 | RATE[15:9] | Rate meter output |
| B7-B0 | RATE[8:1] | Rate meter output |

| # READ register 1 ICB sub address 0x0 |  |  |
| --- | --- | --- |
| REGID=1 |  |  |
| B15 | REGID | Register identification bit |
| B14-13 | N/A | Not assigned |
| B12-B8 | RTHOLD[4:0] | Ratemeter pixel threshold |
| B7-B0 | RATE[23:16] | Rate meter output |

| # READ register 2 ICB address 0x1 |  |  |
| --- | --- | --- |
| REGID=0 |  |  |
| B15 | REGID | Register identification bit |
| B14-B8 | PCTHOLD[6:0] | Consecutive pixel count |
| B7-B5 | N/A | Not assigned |
| B4-B0 | STHOLD[4:0] | Pixel threshold |

| # READ register 2 ICB address 0x1 |  |  |
| --- | --- | --- |
| REGID=1 |  |  |
| B15 | REGID | Register identification bit |
| B14 | ALERTFLAG | Alert output register |
| B13 | OPRST | Safety circuit reset |
| B12 | ALERTEN | Enables cathode control |
| B11 | SYSEN | Enables safety circuit |
| B10-B5 | N/A | Not assigned |
| B4-B0 | FCTHOLD[4:0] | Consecutive frame count |

# 4.4 Cathode Safe Signal

The safety circuit can power down the cathode of the detector via a direct connection on the BPE motherboard to the monitor card. The output is reset when the OPRST bit is set to 1 and returned to 0 when operation is to continue.