UVOT Launch & Early Orbit Timeline

and Turnon Procedure

###### SWIFT-UVOT-040

Author: Pete Roming

Version 1.1

Date: 8 April 2003

Rev: 28 January 2004

| Approved by: | Signature | Date |
| --- | --- | --- |
| Pete Roming, PSU UVOT Lead |  |  |
| Phil Smith, MSSL ICU Software Engineer |  |  |
| Keith Mason, MSSL UVOT-TM Principal Investigator |  |  |
| John Nousek, PSU Narrow Field Instruments Lead |  |  |
| Margaret Chester MOC Lead |  |  |
| Frank Marshall Ground Systems Lead |  |  |

| REVISION SUMMARY |  |  |  |
| --- | --- | --- | --- |
| REV | RELEASE DATE | BRIEF DESCRIPTION/REASON FOR CHANGE | EFFECTED PAGES |
| 0.0 | 8 April 2003 | Draft copy for comments. | All |
| 0.1 | 14 April 2003 | Included comments from Scott Koch and Phil Smith. | All |
| 0.2 | 24 April 2003 | Included changes from Pat Broos. | 5, 9-11 |
| 1.0 | 30 April 2003 | Initial release incorporating changes from OAWG meeting. | All |
| 1.1 | 28 January 2004 | Minor changes based on the mission L&EO timeline and comments from Phil Smith | 8-10 |

# Initial Conditions

Check the following:

- 5 S/C UVOT temperature sensors

- 3 Door microswitches (Telescope door and latches closed)

- Survival heaters on

Do the following:

- Open ITOS archives for UVOT

| Time | Event | Observatory Status | Telecommands | UVOT Activities | Ground Activities | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| L+5 days (2 orbits) | Power on Redundant DEM | Malindi contact | Start uvot_dem_commissioning | Enable 1553, power on DEM, check ICU & DPU watch dog, perform time test, test SSI, test EEPROM, & place DEM in SAFE state | Monitor DEM startup, current, interface test messages, HK, & verify that DEM is in SAFE state | Verify correct telemetry before continuing. Since TM is not powered expect TM related HK to have Red Limits |
| L+5d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+6d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+7d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+8d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+9d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+10d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+11d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |

| Time | Event | Observatory Status | Telecommands | UVOT Activities | Ground Activities | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| L+12d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+13d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+14d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+15d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+16d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+17d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+18d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+19d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+20d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |
| L+21d | SAFE state | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state | Verify that all HK values are nominal |

| Time | Event | Observatory Status | Telecommands | UVOT Activities | Ground Activities | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| L+22d (1 orbit) | Open Telescope Door | ACS Inertial Malindi contact | Start TBD | Open Telescope Door | Monitor RT HK telescope door position | Not to be started until ACS testing is complete & not at the same time as XRT |
| L+22d | TM Outgassing | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state & TM is off | Verify that all HK values are nominal. TM can not be turned on until 3 days after door opening |
| L+23d | TM Outgassing | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state & TM is off | Verify that all HK values are nominal. TM can not be turned on until 3 days after door opening |
| L+24d | TM Outgassing | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify that DEM is in SAFE state & TM is off | Verify that all HK values are nominal. TM can not be turned on until 3 days after door opening |
| L+25d (3 orbits) | Power on Redundant TM | Malindi contact | Start uvot_tm_commissioning | Power on TM, test heaters, test filter wheel, setup detector, intensifier char, test DPU window, flat field | Monitor TM startup, current, HK, & examine images offline | Verify correct telemetry before continuing |

| Time | Event | Observatory Status | Telecommands | UVOT Activities | Ground Activities | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| L+25d (1 orbit) | Change to IDLE state | Malindi contact | Start istate_idle | Place UVOT in IDLE state w/ HV disabled RTSs | Monitor HK & verify UVOT is in IDLE state | Verify that all HK values are nominal |
| L+25d | Slew tests | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify UVOT is in IDLE state | Verify that all HK values are nominal |
| L+26d | Slew tests | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify UVOT is in IDLE state | Verify that all HK values are nominal |
| L+27d | Slew tests | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify UVOT is in IDLE state | Verify that all HK values are nominal |
| L+28d | Slew tests | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify UVOT is in IDLE state | Verify that all HK values are nominal |
| L+29d | Slew tests | TDRSS contact |  | Monitor pointing constraints & slews | Monitor HK & verify UVOT is in IDLE state | Verify that all HK values are nominal |
| L+30d (0.25 orbit) | Change to SAFE state | Malindi contact | Start icu_safe | Place UVOT in SAFE state | Monitor HK & verify UVOT is in SAFE state | Verify that all HK values are nominal |
| L+30d (0.25 orbit) | Upload new ICU RTSs | Malindi contact | Start iloads_r | Loads the HV enabled RTSs | Checksum the uploaded code | Verify code uploaded properly. From here on we care what the S/C & other instruments are doing |

| Time | Event | Observatory Status | Telecommands | UVOT Activities | Ground Activities | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| L+30d (0.25 orbit) | Change to IDLE state & Disable FOM requests | Malindi contact | Start icu_idle_disabled | Place UVOT in IDLE state w/ HV enabled; Ignore FOM requests | Monitor HK & verify UVOT is in IDLE state and FOM disable request received | Verify HK is nominal & that the UVOT is ignoring FOM requests. Potential Danger of Bright Source Damage Begins |
| L+31d (0.5 orbits) | Enable FOM PTs | Malindi contact | Start icu_enable_at_pt | Receives FOM PTs | Verify request received | Verify that the UVOT is receiving FOM PTs |
| L+31d (0.5 orbits) | Pulse Height exposure | Malindi contact |  | Take a Pulse height exposure during a scheduled PT | Analyze exposure | Check for a normal pulse height peaking at about channel 80 |
| L+31d (5 orbits) | HV adjustment | Malindi contact | ihvadjust(x) | Adjust voltage | Send new voltage to UVOT | Run intchar multiple times adjusting voltage each time w/ ihvadjust(x) |
| L+32d (0.5 orbits) | Centroid Exposure & Centroid Exposure confirm | No contact |  | Take a centroid exposure, calculate channel boundaries, & upload to detector RAM. This is scheduled during a PT | Analyze exposure | Check channel boundary & exposure & compare to calibration numbers |

| Time | Event | Observatory Status | Telecommands | UVOT Activities | Ground Activities | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| L+32d (1 orbits) | First light exposure | No contact |  | Take exposure of an uploaded PT | Check performance & analyze PSF | A PT is used for safety of UVOT |
| L+32d (20 orbits) | Focus test | Malindi contact | ifocus(x) | Take exposure of an uploaded PT every hour | Analyze images and adjust focus | Perform test 20 times alternating between the V filter and the magnifier |
| L+34d (3 orbits) | Safety circuit calibration | Malindi contact |  | Take multiple exposures of an uploaded PT | Adjust the safety circuit parameters | Multiple images of a star of known brightness are taken and the safety circuit adjusted until the circuit trips |
| L+35d | UVOT calibration begins | Malindi contact |  |  |  | See UVOT On-Orbit Calibration Plan |
| L+ACE (3 orbit) | Upload revised AT tables | Malindi contact | Start TBD | Loads the revised AT & PT tables | Verify tables uploaded | This activity occurs after calibration ends (ACE) |