XMM Optical Monitor

MULLARD SPACE SCIENCE LABORATORY

UNIVERSITY COLLEGE LONDON	Author: Rahil Chaudery

MSSL  XMMOM

DOCUMENT:

Distribution:

| XMM-OM Project Office | R Hunt |  |
| --- | --- | --- |
| ESA PX | H Eggel |  |
| CSL | P Rochus |  |
|  | S. Roose |  |
| Los Alamos | C Ho |  |
| Royal Greenwich Observatory | R Bingham |  |
| University College London | J Fordham |  |
| Mullard Space Science Laboratory | N Bray |  |
|  | R Card |  |
|  | M de la Nougerede |  |
|  | R Chaudery |  |
|  | A Dibbens |  |
|  | H Huckle |  |
|  | G Davidson |  |
|  | T Kennedy |  |
|  | M Carter |  |
|  | D Self |  |
|  | P Smith |  |
|  | P Thomas |  |
|  | M Whillock |  |
|  | K Mason |  |
|  | A Smith |  |

| Author: | Rahil Chaudery | Date: | 30 July, 2026 |
| --- | --- | --- | --- |
| OM Project Office |  | Date: |  |
| Distributed: |  | Date: |  |

| Issue | Date | Changes |
| --- | --- | --- |
| 1 |  |  |

| Machine File-System Directory: | n:\text\dd\qm_icu\ |
| --- | --- |
| Document File Name: | input.doc |
| Application: | MS Word 2 |
| Included files: |  |
| Print Date (this): |  |
| Printer: | Postscript |

# 1.	Schematics

Drawing Number:	A?/5229/038  Issue 2

Hierarchical list of sheets:

SSI.1		5-19-1996_18:25

SSI.2		5-19-1996_18:25

# 2.	PCB Manufacturing and Board Assembly and Parts List

Drawing Number:	A2/5229/038-5  Issue 2  4-Aug-96		drill plot

A2/5229/038-6  Issue 2  19-Aug-96	assembly drawing and parts list

X-1561-002				PCB

# 3.	Bill of Materials

Database printout for	ICU-EXP(SSI)	29-Jan-97

# 4.	Construction Operations

## 4.1	Links

U21,U22:	54hc245 IC's replaced by wire links: INPUT to OUTPUT

J1	IN

J2	IN

J3	IN

J4	IN

J5	IN

### 4.3	Modifications

### 4.3.1	Add Wires

X10 to X2		:2MHz clock for OMIF

X22 to X3		:1k9kHz clock for RESLATCH

X11 to X4 to X20	:1MHz clock for SSI

X12 to X5		:500kHz clock for OMIF

X14 to X19		:125kHz clock for SSI

X24 to X26		:Clock source

X6 to X7 to X8 to X9	:GND connections

see below for further link modifications.

### 4.3.2	Track Cuts

Track from P1/pin 43 to its via (label this via  'A')

Track from P1/pin 44 to its via (label this via  'B')

Track from P1/pin 45 to its via (label this via  'C')

Track from P1/pin 46 to resistor 'R4' ('P1' end)

Add wire link from P1/pin 43 to via-'B'

Add wire link from P1/pin 44 to via-'A'

Add wire link from P1/pin 45 to 'R4' ('P1' end)

Add wire link from P1/pin 46 to via-'C'

# 5.	Updated Schematic

no update

# 6.	Layout Instructions

## 6.1	Input to Mentor

Files are generated from a Viewlogic conversion utility to mentor format.

NO modifications allowed to COMPS_FILE or NETS_FILE without consent from RAC.

Files may be found in:

ftp://128.40.70.7/Public/Mentor/ICU/OMIF/QM_SSI

File names:	comps_file.cmp

netsfile.net

## 6.2	Notes

1. VICU is a power signal and requires large tracks.  Q1 and Q2 should be close to the MTB connector to keep these tracks as short as possible.

2. Component heights are limited:

ABOVE to 5.9 mm

BELOW to 2.7 mm

3. Q1 and Q2 conventionally mounted will be too high.  These transistors should be mounted upside-down in a hole in the board.

4. ALL capacitors need to be laid flat.

2. VDD and GND are power planes.

## 6.3	Feedback from Mentor

none.

# 7.	Layout Changes

none.

# 8.	Files

Viewlogic QM files dated up to 19-May-96 in

\\msslik\designs\xmm-om\qm_icu\omif_qm\men_ssi

# 9.	Design Description

Decoding:

U6-U9 perform the decoding for all interfaces; decoded enable signals are available at U7 and U9 outputs.

U23, U32, U25 and U24 give the various divided clock frequencies required by interfaces.

Q1 and Q2 perform power supply filtering/stabilising.

U26 is unused.

SSI Interface:

U21 and U22 are no longer required: wire links connect across I/O's 'A' and 'B'.

U1 is the STATUS flag register.

U27 is an interrupt flag bit; the processor deserts the flag by enabling U2 via U20 and decode line: SSIWR_ST

The SSI interface is sub-divided into two sections: SSI_OUT and SSI_IN.

SSI_OUT:

A 16 bit word is written to a latch (U12 and U13).  The write operation starts the state machine (U3, U4, U20, U30, U31 and U14) which loads the word to an internal latch (U10 and U11).  As data is clocked out through filter C4/R3, the data-envelope signal is generated by U19, U25, U4, U3, U24 and U26.  The envelope length is derived from the counter U19.

On moving the data to the internal latch a 'DATA_FULL' flag (U29) is deserted allowing further data to be loaded.  U30 provides a transmission-in-progress flag .

SSI_IN:

The envelope signal (U27) of the incoming data enables the clock/cycle to capture data.  Data is stored into a 16-bit serial register (U15 and U16).  When a complete word has been stored it is moved to a parallel latch (U17 and U18).

The falling edge of the envelope signal generates an interrupt signal to the processor (U27) and also to a control bit (U28).  If data is not read out before the next word has been captures an over-flow condition will exist, indicated by U28.

# 10.	Setup/Testing

After turning on power check that clock signals exist on U32, and at U25/pin 12 and U24/pin 2.

Check the state of pin 28 on the back-plane connector- as no software will be running, heartbeats being generated by the DPU will have caused an interrupt, therefore U27/pin 5 should be at logic '0' (0V).  Also, U1/pin 15 will be logic '0' as an over-flow will have occurred.

By having software writing/reading from a port the appropriate decode line should be observed going active on a storage 'scope (active state signal going to 0V).