XMM Optical Monitor

MULLARD SPACE SCIENCE LABORATORY

UNIVERSITY COLLEGE LONDON	Author: Rahil Chaudery

MSSL  XMMOM

DOCUMENT:

Distribution:

| XMM-OM Project Office | R Hunt |  |
| --- | --- | --- |
| ESA PX | H Eggel |  |
| CSL | P Rochus |  |
|  | S. Roose |  |
| Los Alamos | C Ho |  |
| Royal Greenwich Observatory | R Bingham |  |
| University College London | J Fordham |  |
| Mullard Space Science Laboratory | N Bray |  |
|  | R Card |  |
|  | M de la Nougerede |  |
|  | R Chaudery |  |
|  | A Dibbens |  |
|  | H Huckle |  |
|  | G Davidson |  |
|  | T Kennedy |  |
|  | M Carter |  |
|  | D Self |  |
|  | P Smith |  |
|  | P Thomas |  |
|  | M Whillock |  |
|  | K Mason |  |
|  | A Smith |  |

| Author: | Rahil Chaudery | Date: | 28 July, 2026 |
| --- | --- | --- | --- |
| OM Project Office |  | Date: |  |
| Distributed: |  | Date: |  |

| Issue | Date | Changes |
| --- | --- | --- |
| 1 |  |  |

| Machine File-System Directory: | n:\text\dd |
| --- | --- |
| Document File Name: | input.doc |
| Application: | MS Word 2 |
| Included files: |  |
| Print Date (this): |  |
| Printer: | Postscript |

# 1.	Schematics

Drawing Number:	5229/052  Issue 1

Hierarchical list of sheets:

MACS.1	8-17-1996_5:06

MACS.2	8-17-1996_5:06

# 2.	PCB Manufacturing and Board Assembly and Parts List

Drawing Number:	5229/052-5  Issue 2  20-Aug-96		drill plot

5229/052-6  Issue 2  21-Aug-96		assembly drawing and parts list

X-1563-002				PCB

# 3.	Bill of Materials

Database printout for	ICU-OMIF(ICB)	29-Aug-96

# 4.	Construction Operations

## 4.1	Links

J1	IN

J2	IN

J3	IN

J4	IN

J5	IN

J6	IN

J7	IN

## 4.2	Select on Test Items

R40, R41	:measure to give (15k + 1k8) and 12k respectively.

R42, R43	:measure to give 68k.

R60, R39	:measure to give (270k + 33k)for both.

## 4.3	Modifications

Remove resistors: R46, R49, R52, R55 in V_MON circuit.

Remove resistor R59 on ADC_MON circuit (required to implement schmitt gate action).

Cut track between U18/pin 4 and R62, add wire link from U18/pin 4 to U23/pin 10, and from X9 to U2/pin 41.  (This is required to stop tri-state condition of BUS[15:0] between MACS FU-02 and CMD/DATA registers).

### 4.3.1	Add Wires

X1 to 'STRUCTURE' of pcb card.

U2/pin 49 to VDD	; unused I/P 'HLREQ' on MACS IC tied high.

U3/pin 4 to GND		; floating I/P

U3/pin 6 to GND		; floating I/P

U3/pin 8 to GND		; floating I/P

U28/pin 9 to U3/pin 13	; clock-sync_ERR signal to STATUS register.

X3 to X16		; X3 to GND point

X4 to X17		; X4 to GND point

X5 to X21		; X5 to GND point

X6 to X20		; X6 to GND point

X7 to X18		; X7 to GND point

X8 to X19		; X8 to GND point

X10 to X15		; X10 to GND point

X12 to X2		; X12 to GND point

X13 to X11		; X13 to GND point

### 4.3.2	Track Cuts

see above

# 5.	Updated Schematic

none

# 6.	Layout Instructions

## 6.1	Input to Mentor

Files are generated from a Viewlogic conversion utility to mentor format.

NO modifications allowed to COMPS_FILE OR NETS_FILE without consent from RAC.

Files may be found in:

ftp://128.40.70.7/Public/Mentor/ICU/OMIF/QM_ICB

File names:	comps_file.cmp

nets_file.net

## 6.2	Notes

1.	'X1' connects to pins on the D-type connector.  This pin should be placed near the D-type connector, and should have no power planes underneath it.  This 'keep-out' area should ideally be at 10 mm radius .

2.	For flat-packs (RBI, U8), don't allow tracks to connect directly between adjacent pins. Short adjacent pins with a loop of track that extends at least 0.1" away from the pads. (Ideally longer if underneath the chip).

3.	VDEM is a power signal and requires large tracks. The transistors connected to it should be close to the connector. The length of the VDEM track should not exceed 3cm.

4.	VDD and GND are power planes.

5.	Component heights are limited:

ABOVE to 7 mm

BELOW to 4.7 mm

## 6.3	Feedback from Mentor

none

# 7.	Layout Changes

none

# 8.	Files

Viewlogic QM files dated up to 19-May-96 in

\\msslik\designs\xmm-om\qm_icu\omif_qm\men_icb

# 9.	Design Description

The pcb incorporates three interface designs: ICB, DPU Voltage Monitoring and Reset DPU circuit.

ICB:

U10, U11 form the 'COMMAND-WORD' latch;  U12, U13 form the 'DATA_WRITE' latch; U14, U15 form the 'DATA_READ' latch.  Data/Command words to be transmitted are latched with the appropriate write signal (~ICBWR_CMD or ~ICBWR_WD decoded on QM_EXP(SSI)).  The operation of writing a word to the COMMAND register starts the state machine(U5, U24, U26) to allow data from the COMMAND/DATA latches to be read into the internal registers of the MACS IC (U2).  The MACS controller then transmits the serial digital data (and performs all error/protocol checking).  Flip-flop U30 is used to delay out-going data w.r.t. the MACS-clock- to meet transmission criteria.  The hiericical block 'ANA_DRV' is the analogue drive circuitary which drives the differential serial lines - in and out.

Counter IC U19 and logic sections :U5, U23, U7, U8, U24, U22, U1 and U4, U8, U23, U5 decode serial bit positions to allow various transmission/error conditions to be latched into flip-flops U28, U29.  Status information is available via STATUS register U3 for both ICB and Voltage Monitoring circuits.

Circuit block 'CLK_GEN' generates the MACS-clock and SYNC signals.

Voltage Monitoring:

The basic operation blocks consist of: a multiplexer circuit to select the voltage to be monitored (collectors of  transistors Q9, Q10, Q11, Q12), a digital to analogue circuit providing the ramp voltage to a schmit trigger and the multiplxer output voltage giving the reference voltage to be measured.

By controlling the base voltage , only one of the transistors is switched on, so 'VIN' resents the selected collector voltage (though scaled down by the transistor voltage drop).  The op-amp U17 is configured to perform two operations: gain control to give required range and a level shift.   The other half of op-amp U17 is connected as a schmit trigger circuit: one input being the selected voltage to be monitored and the other input coming from the circuit block 'DAC' (a conventional R-2R ladder network followed by a unity gain amplifier stage).  U16 is used to latch the DAC ramp voltage in binary form and to select which voltage is to be monitored.

Reset DPU:

U7, U27 AND U26 provide a 1ms pulse when written to by the processor; this is sent to the DPU down the backplane - resetting the DPU.

# 10.	Setup/Testing

Initially the MACS-clock and SYNC signals need to be verified to specifications.  This is best done by monitoring the ICB_CK+ and ICB_CK- signals on the D-type connector.  Software writing to the COMMAND register and reading the STATUS register will verify if the ICB circuit is operating.

Writing to port address for ~RST_DPU and monitoring RESLATCH/pin 61 on backplane connector.

The junction of R40/R41: 'V_OFFSET' should first be verified as being 1V225.

By applying differing voltages (in the range 2V5 +/- 0V5) and then selecting one multiplexer input at a time and monitoring 'VIN' the operation of the multiplexer can be checked.  The DAC operation can be cverfied by writing an incrementing binary value and monitoring 'VO' on the DAC circuit.  The schmit output should go to logic '0' when the DAC output becomes greater than the voltage to be monitored.