XMM Optical Monitor

MULLARD SPACE SCIENCE LABORATORY

UNIVERSITY COLLEGE LONDON							Editor: H.E.Huckle

MSSL XMM-OM On-Board Software Requirements

Document: XMM-OM/MSSL/SP/0024.03

Distribution:

| XMM-OM Project Office | R Hunt | ü |
| --- | --- | --- |
| ESA PX | H Eggel | ü |
| CSL | P Rochus |  |
|  | S Roose |  |
| Los Alamos National Laboratory | C.Ho | ü |
| UCSB | T.Sasseen | ü |
| Royal Greenwich Observatory | R Bingham |  |
| University College London | J Fordham |  |
| Mullard Space Science Laboratory | R Card |  |
|  | M Carter | ü |
|  | R Chaudery |  |
|  | A Dibbens |  |
|  | H Huckle | Orig |
|  | H Kawakami |  |
|  | T Kennedy | ü |
|  | D Self |  |
|  | P Smith | ü |
|  | P Thomas |  |
|  | M Whillock | ü |
|  | K Mason |  |
|  | A Smith |  |

| Author: |  | Date: | 19-Sep-1997 |
| --- | --- | --- | --- |
| OM Project Office |  | Date: |  |
| Distributed: |  | Date: |  |

| Issue | Date | Comments |
| --- | --- | --- |
| Draft | May ‘94 | Draft Version for Comment |
| Issue 1 | 24 May ‘94 | Comments Included |
| Issue 2 | 5 April ‘95 | Update for EIDR |
| Issue 3 | Sep ‘97 | Update for EM delivery, reflecting XOMBI and red detector loss. |

Table Of Contents

1. Introduction

1.1 Purpose

1.2 Scope

1.3 Applicable Documents

1.4 Definitions, Acronyms and Abbreviations

2. General Description

2.1 Function and Purpose

2.2 ICU

2.2.1 ICU Environmental Considerations

2.3 DPU

2.3.1 DPU Environmental Considerations

2.4 Observation Overview

3. ICU Specific Requirements

3.1 Functional Requirements

3.1.1 Bootstrap

3.1.2 Monitor

3.1.3 Command Processing

3.1.4 Telemetry Processing

3.2 Performance Requirements

3.3 Interface Requirements

3.4 Operational Requirements

3.5 Verification and Validation Requirements

4. DPU Specific Requirements

4.1 Functional Requirements

4.1.1 Data/Command I/O

4.1.2 White DSP Operating System

4.1.3 Blue DSP Software

4.1.4 Red DSP Software

4.1.5 Tracking

4.1.6 Science Modes

4.1.7 Engineering Modes

4.1.8 Red Science Modes

4.2 Performance Requirements

4.3 Operational Requirements

4.4 Resource Requirements

4.5 Verification and Validation Requirements

5. Non- Specific Requirements

5.1 Documentation Requirements

5.2 Security Requirements

5.3 Portability Requirements

5.4 Quality Requirements

5.5 Reliability Requirements

5.6 General Maintainability Requirements

5.7 Safety Requirements

# Introduction

## Purpose

This SRD describes the operation of the OM. The general characteristics of the image data and science products required are highlighted. Some explanation of the in-orbit operational aspects of the OM is presented. Finally, specific requirements are listed.

This requirements document defines the functional and performance features required of the XMM optical monitor (OM) on-board software. This may be used as a reference for the discussion in the architectural design document, which includes detailed description of software modules which carry out these tasks. The important features of OM operation that require the on-board software in order to function are described. This leads to a description of the on-board software tasks, followed by a description of the interfaces and hardware that facilitate or constrain the software.

In defining the software requirements this document highlights the important features of the on-board software to allow quantifiable and verifiable measures to be implemented in its design, development and test. As such, the document is addressed to the software teams of the OM consortium and to the engineers of consortium groups developing sub-systems with which the software will interface, and finally to such external reviewers or potential scientific users who might wish to understand the nature of the tasks and examine critically the OM software development process.

## Scope

The software requirements described in this document comprise:

- The flight software requirements for the Instrument Control Unit (ICU).

- The flight software requirements for the Data Processor Unit (DPU).

The ICU software will provide overall instrument control. It is located with the DPU in the Digital Electronics Module (DEM).

The DPU software will capture photon or image data from the XMM-OM detectors, and perform spatial and temporal compression, together with compensation for artefacts such as spacecraft drift and background effects. The data will be stored in memory until transmitted to the telemetry stream via the Instrument Control Unit.

## Applicable Documents

| APP-1 | XMM-OM Packet Structure Definition | RS-PX-0032 |
| --- | --- | --- |
| APP-2 | DPU-ICU Interface Protocol Definitions | XMM-OM/MSSL/SP/0014 |
| APP-3 | Software Setup of the Blue Detector Electronics | XMM-OM/MSSL/SP/0077 |
| APP-4 | OBDH Users Guide | RS-PX-0015 |
| APP-5 | XMM-OM Bootstrap Specification | XMM-OM/MSSL/SP/0153 |
| APP-6 | Observing and Telemetry Modes for XMM-OM | XMM-OM - XMM-OM/MSSL/TC/0007 |
| APP-7 | XMM-OM C- Language Coding Conventions | XMM-OM/MSSL/SP/0008 |
| APP-8 | Hermes Ada Coding Standard | Derived from ESA Coding Standard WME/89-350/JMB |
| APP-9 | User Requirements Specification | XMM-OM/MSSL/SP/0030 |
| APP-10 | DPU Architectural Design Document | XMM-OM/PENN/SP/0003 |
| APP-11 | Telecommand & Telemetry Specification | XMM-OM/MSSL/SP/0061 |

## Definitions, Acronyms and Abbreviations

| ADD | Architectural Design Document |
| --- | --- |
| Blue DSP (1/2) | the DSPs on the DPU responsible for data capture and event registration. |
| CCD | Charge coupled device. A pixel-based sensor used as the centroiding element of the MIC detector. |
| DBI | Digital Bus Interface - the interface to the spacecraft |
| DDD | Detailed Design Document |
| DEM | Digital Electronics Module |
| DPU | Digital Processing Unit |
| DPUOS | DPU Operating System |
| DSP | Digital Signal Processor, specifically in the case of the DPU, a Motorola DSP56001 processor |
| GSE | Ground Support Equipment |
| ICU | Instrument Control Unit |
| MIC | Microchannel plate Intensified CCD, the photon counting detector system |
| OM | Optical Monitor |
| Red DSP | the DSP dedicated to perform high efficiency requirement tasks, e.g. SAA. |
| SAA | Shift and add, a critical algorithm in the DPU software which shifts successive frames according to the calculated spacecraft drift and adds the frame into an accumulated image. |
| SRD | Software Requirements Document |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TMPSU | Telescope Module Power Supply Unit |
| White DSP | the DSP responsible for the operational control and communication with the ICU |

General Description

Function and Purpose

The general scientific requirements and capability of the OM are described in document XMM-OM/MSSL/RV/0001, and the observing modes in APP-9. A complete description of the instrument is given in the Experiment Interface Document (Part B) RS-PX-0018. Broadband filter photometry, imaging and timing of targets that will be observed by the XMM main X-ray telescope cameras requires a wide field of view, arc second class imaging performance, a wide spectral coverage and timing capability with sub-second resolution.

To meet these requirements, a telescope module containing two redundant photon counting detectors. The beam is reflected by a dichroic through an associated filter wheel to the detector. Each of the 2 filter wheels carries 7 filters, a focal expander and a blank. The telescope unit also contains a power supply and electronics.

To control the instrument and process the data from it, there are two digital electronics modules (DEM), one prime and one cold redundant. They each contain an Instrument Control Unit (ICU) and Data Processing Unit (DPU), as well as power supply electronics. The DPU and ICU communicate via a Serial Synchronous Interface (SSI) . The DEMs and telescope module are connected via an interconnecting harness. This harness carries primary power and sync, detector data, keep alive line and the Interface Control Bus.(ICB). The ICB is used to control and monitor the detectors, mechanisms and power supply. It is based on the MACS-bus. Uplink and downlink data from and to the instrument are carried by a digital bus interface (DBI).

## ICU

The overall instrument function is provided by the instrument controller. Its main software functions are as follows:-

- Configuring the instrument.

- Monitoring for breakdown/failure conditions and safing if required.

- Controlling, and monitoring status of, the detectors, the telescope power supply and the DPU

- Interpreting and executing telecommands

- Incorporating new or modified code modules for itself or the DPU

- Collecting and preparing instrument housekeeping and engineering packets in preparation for telemetering.

- Accepting and reformatting into packets the science data from the DPU, in preparation for telemetering

- Interfacing with the OBDH for data and commands.

- Monitoring thermal sensors and control the thermal environment.

ICU Environmental Considerations

The code will be written in ADA except when timing or interface requirements dictate Assembler. The CPU will conform to the Mil-Spec 1750A standard.

## DPU

The detector must be a photon-counting system in order to match the required sensitivity. Estimates of the count rate from the field (2e5/sec) imply that for a 1024 x 1024 format image, the bit rate would be 4 Mbit/second. This grossly exceeds the available data rate for the observatory as a whole.

To compress this bit rate, it is desirable to store the images in an accumulating memory, for a time compatible with scientific objectives (typically 1000 seconds). However the spacecraft attitude will drift by more than one imaging pixel on these timescales, and produce image blurring on a scale greater than that necessary for high sensitivity imaging. It is the primary goal of the DPU to compensate for these aspects, but as described below, other functions must also be provided by the DPU. Without the DPU and its related software, the XMM-OM will not be able to fulfil the scientific objectives of obtaining wide field optical images of selected fields, together with time resolved photometry of selected targets

For the MIC detector, the high resolution images might be degraded by spacecraft attitude drift so that photon data must be re-registered with respect to reference sky co-ordinates.. The key elements of DPU operation are therefore to capture data, store them in correct memory locations, compress and then process them into appropriate format for downlink.

Secondary requirements are to provide higher time resolution data of both a reduced set of scientific data and of a engineering/housekeeping nature. For example, some X-ray targets will have interesting intensity variations with time-scale much shorter than an image collection interval. It would be important to extract that data, for limited portions of the image, on timescales from milliseconds to seconds.

All data types are presented to the Instrument Control Unit for reformatting into packets.

### DPU Environmental Considerations

The DPU is a sub-system which is a core element of the payload, in that the scientific requirements cannot be met without its operation. It is not in a general sense an instrument computer, however, because its tasks are very limited. The DPU merely processes science data in a manner requested by the observer to preserve the maximum scientific output.

It was realised at an early stage of the project that these functions had to be demonstrated early in the breadboard phase, and thus simulations of the tracking performance were initiated in order to

- Confirm the ability of the baseline instrument to perform real time tracking at the required accuracy

- Define the algorithms for the DPU in more detail to size the processor requirements

In addition an early decision was required on the choice of processor hardware, dictated in part by programmatic limitations and environmental considerations. The selected processor was the Motorola DSP56001, chosen for a number of reasons including a high performance measured in operations per second at modest electrical power consumption. Similarly design decisions for the memory allocation had to be forced early in the program, and a judicious choice based on known detector image characteristics was taken. Thus the hardware constraints for the DPU software are fixed  at an early stage. In addition the user requirements phase has led to a thorough prototyping and extensive testing of the DPU flight software.

The DPU hardware environment consists of four separate processor cards each housing one DSP. They are named the White DSP, the Blue DSP1/2, and Red DSP. Each of the processors performs and distinct specific functions with different software environmental support.

The White DSP is the primary processor responsible for controlling the operation of the DPU and communication with IDU. The White DSP software is  built around a custom operating system, the DPUOS. The DPUOS will be implemented in both C and assembly language. The application level flight software will be implemented mainly in ‘C’, except where performance requirements require faster execution. The operating system software comprises loaders, basic drivers, interrupt handlers, command processor, data I/O, and basic service routines. Applications software includes tracking, mode handling,  data compression etc.

The Blue DSPs are dedicated to perform real-time data capture and event processing. The Blue DSP flight software will be completely built in assembly language for efficiency. System functionality of control and status monitoring by the White DSP is tightly integrated with the application functionality of data capture and event processing to form the Blue DSP software.

The Red DSP provides additional generic processing power to perform dedicated tasks. Its operating system environment will be a subset of the DPUOS, with support to application software implemented in C.

The  DPU SW operates in a real-time environment and shall have necessary protection for safe operation and reliable data collection operations. Measures to ensure this include task time-outs, forward error correction, limited access to program storage, processor time margins or limitation of interrupt routine duration.

It is designed for 10 yr. operation in-orbit. All software elements that might be affected by changes in instrument performance, configuration or Observatory operation can be changed by parameter telecommands or DPUOS and application software updates.

Protection against SEUs is affected by hardware design, operational mode and software measures.

The relationship of ICU and DPU software to other systems is defined by the interfaces to those systems, where the detailed specification are covered in the appropriate system interface specification.

## Observation Overview

### An observation is initiated with an acquisition sequence to determine the exact position of the spacecraft attitude with respect to desired targets. A V filter image is taken with the MIC detector at low resolution (full field) 1024 x 1024 binned pixels. The DPU integrates events one-by-one in memory for a TBD period, and then compares the image with locations of reference star positions previously uplinked from TBD star catalogue data. The data (e.g. offsets from field of view centre) are transmitted to the ICU, so that it is able to set up restricted hardware defined windows on the MIC detector, and change filter position to that desired for astronomical observations to commence. User-defined science windows related to the MIC hardware windows are defined in the DPU memory. The ICU sends a start command to the DPU, and data from the MIC detector are accepted, checked for parity, and the appropriate memory locations incremented. After a user-definable duration of the order 10 seconds, the image is analysed for stars that can be used as guide  tars. The criteria can be altered, but generally comprise brightness, location, image size etc.. indows are set up around each of these locations in addition to the science windows.

The image data are captured in the same way for TBD seconds and re-directed to new memory locations, and incremented appropriately. The image accumulated in this area is now examined to find the guide stars again, and the mean shift in X, Y and roll calculated. The image data are then located in a third memory location destined for overall image accumulation. During this process of guide star location, attitude measurement and data shifting, the next time slice of the image is collected in the original memory area, until the next guide star calculation starts. The process is repeated many times during a typical frame accumulation time of the order 1000 sec. The typical X-ray observation of 10,000 secs implies that either there will be multiple such images, or different filter or window parameters will be used. During the period of the next exposure, the previous image will be compressed and transmitted to the ICU for downlink.

### ICU Specific Requirements

## Functional Requirements

### Bootstrap

The ICU bootstrap requirements are detailed in document APP-5.

Monitor

R-ICU-1/2000	The ICU shall monitor the DPU for heartbeats at 10 sec intervals. Upon failing to receive 2 successive heartbeats, it shall initiate a TBD OM recovery sequence.

R-ICU-1/2010	The ICU shall monitor the detector for breakdown/failure conditions and initiate a TBD safing sequence.

R-ICU-1/2015	Removed.

R-ICU-1/2020	The ICU shall ensure that the position of the filter wheels is consistent with safe operation of the instrument at all times

R-ICU-1/2025	Removed.

R-ICU-1/2030	The ICU shall keep a record on-board at all times of the current instrument configuration.

R-ICU-1/2035	The ICU shall monitor and maintain thermal control of the OM telescope unit.

R-ICU-1/2040	The ICU shall reset the RBI watchdog at commandable regular intervals to prevent a watchdog time-out.

R-ICU-1/2045	The ICU shall monitor packet acceptance by the spacecraft. In the event of packets no longer being accepted after 30 secs (TBC), it shall execute a TBD safing sequence.

### Command Processing

R-ICU-1/3000	The ICU shall support a telecommand rate of 2 kbits/sec

R-ICU-1/3001	The ICU shall interpret and execute commands within 1 sec of receipt.

R-ICU-1/3005	Removed.

R-ICU-1/3010	Removed.

R-ICU-1/3015	Removed.

R-ICU-1/3020	Removed.

R-ICU-1/3025	Removed.

R-ICU-1/3030	Removed.

R-ICU-1/3035	Removed.

R-ICU-1/3040	The ICU shall support all DPU commands as specified in document APP-2

R-ICU-1/3045	The ICU shall control, by means of commands specified in APP-2, the extraction and downlinking of all DPU data.

R-ICU-1/3050	The ICU shall support all detector commands as specified in document APP-11

R-ICU-1/3055	The ICU shall be able to define and load any specifiable detector window look-up table as specified in APP-11.

R-ICU-1/3060	The ICU shall be able to define and load any specifiable centroid look-up table for the detector as specified in APP-11.

R-ICU-1/3065	Removed.

R-ICU-1/3070	Removed.

R-ICU-1/3075	The ICU shall be able to command any filter wheel from any position to any other position in 10 secs.

R-ICU-1/3080	The ICU shall be able to reboot on command from ground

R-ICU-1/3085	The ICU shall verify all commands on receipt from ground.

R-ICU-1/3090	Removed

R-ICU-1/3095	Removed.

R-ICU-1/3100	This requirement removed.

R-ICU-1/3105	The ICU shall support all RBI watchdog commands as specified in APP-11.

R-ICU-1/3110	The ICU shall support all filter wheel commands as specified in APP-11.

R-ICU-1/3115	The ICU shall support all dichroic commands as specified in APP-11.

R-ICU-1/3120	The ICU shall support all heater control algorithms as specified in APP-11.

R-ICU-1/3125	The ICU shall support all TMPSU commands as specified in APP-11.

R-ICU-1/3130	The ICU shall support all time management commands as specified in APP-11.

R-ICU-1/3135	The ICU shall support all memory maintenance commands as specified in APP-11.

R-ICU-1/3135	The ICU shall support all telemetry management commands as specified in APP-11.

### Telemetry Processing

R-ICU-1/4000	The ICU shall support a telemetry rate of up to 40 kbit/sec

R-ICU-1/4005	Removed, covered in R-ICU-1/4020.

R-ICU-1/4010	The ICU shall collect DPU data and responses (as defined in APP-2) and format them for downlinking as telemetry packets (format specified by APP-11).

R-ICU-1/4015	Removed.

R-ICU-1/4020	The ICU shall provide all the telemetry as specified in APP-11.

- R-ICU-1/4025	The ICU shall provide packet data in a form suitable for downlinking by the spacecraft consistent with APP-4.

R-ICU-1/4030	The ICU shall time-stamp telemetry requiring it (as specified in APP-11) to an accuracy of 1 ms.

## Performance Requirements

As stated where necessary in the Functional Requirements

## Interface Requirements

R-ICU-3/1000	The ICU software shall handle all the interfaces according to the documents APP-1 to APP-4.

## Operational Requirements

R-ICU-4/1000	Requirement removed

R-ICU-4/1005	The ICU shall have two operational states

- Basic Mode - for support of the OM Initial State (Modes)

- Operational Mode - for support of all other OM States (Modes)

(The OM states are defined in APP-9).

R-ICU-4/1010	Code to support the ICU Basic Mode shall be stored in ROM

R-ICU-4/1015	Code to support the ICU Operational Mode shall be stored in keep-alive RAM.

R-ICU-4/1020	The Basic Mode code shall contain such code modules extracted from operational mode code as are necessary to support the following functionality.

- Moving the active filter wheels to reference positions (TBC)

- Uplinking code into ICU keep-alive RAM

- Uplinking code into DPU keep-alive RAM.

- Telemetry and Telecommands as specified by APP-9

- The ability to switch to operational mode once such code is present in keep-alive RAM.

- Basic Thermal control.

R-ICU-4/1025	The ICU shall support all mode change commands as specified in APP-11.

Verification and Validation Requirements

R-ICU-5/5000	The ICU shall provide facilities to read and write to/from memory addresses.

R-ICU-5/5005	The ICU shall support the facility of being able to update its own code

R-ICU-5/5010	The ICU shall support the facility of implementing uplinked DPU code patches.

R-ICU-5/5015	The ICU shall forward to the spacecraft any memory dumps provided to it by the DPU.

# DPU Specific Requirements

## Functional Requirements

### Data/Command I/O

R-DPU-1/1000	The DPU shall be able to receive commands and data from the ICU in the format described in the ICU -- DPU Protocol Definitions document (XMM-OM/MSSL/SP/0014 - XMM-OM/MSSL/SP/0014)

R-DPU-1/1010	The DPU shall be commandable by the ICU to collect the detector events by either of the two blue DSPs, or alternate between the two from event to event.

R-DPU-1/1015 	The DPU shall receive data from the  detectors through the Blue Detector/DPU interface.

R-DPU-1/1030 	The DPU shall be capable of accepting parameter settings and mode changes to modify its operation for any observation, based on commands received from the ICU.

R-DPU-1/1035	The DPU shall provide error detection and correction in stay-alive RAM code memory.

### White DSP Operating System

R-DPU-1/2000 	The bootstrap code for initiating the DPU shall be stored in ROM.

R-DPU-1/2005 	The DPU shall bootstrap the White DSP operating system from storage in stay-alive RAM upon command by the ICU.

R-DPU-1/2010 	The global data and address busses shall not be accessed except when any processor is enabled to do so by hardware.

R-DPU-1/2020 	The White DSP operating system shall reset the DPU to the boot-idle state if the global bus watchdog is activated.

R-DPU-1/2025 	The White DSP operating system shall support the higher level functions written in C.

R-DPU-1/2030 	This requirement to be revised once timing system has been specified. The Local Time Management System time will be available to the DSPs via an address on the global bus.

R-DPU-1/2035 	This requirement to be revised once timing system has been specified. The White DSP operating system shall read the Local Time Management System address into local memory once per 1ms period.

R-DPU-1/2045	The DPU shall transmit an alive signal (heartbeat) every 10 s (default).

R-DPU-1/2050	The DPU shall be able to echo back every command received from ICU, upon command from the ICU.

R-DPU-1/2055 	Upon reset or power on, the DPU shall be in the boot-idle state.

R-DPU-1/2060 	In the boot-idle state, the DPU shall wait for ICU command and shall be able to receive a limited set of commands from the ICU. These commands include reset, load DPUOS from stay-alive RAM to local memory, load codes sent by the ICU into stay-alive RAM, and dump RAM areas to the ICU.

R-DPU-1/2065 	Upon receiving the load DPUOS command, the DPU shall load the DPUOS from the stay-alive RAM and enter the DPUOS-idle state. This shall be the "standby" mode.

R-DPU-1/2070	In the DPUOS-idle state, the DPU shall wait for ICU commands and the full complement of DPUOS commands shall be functional.

R-DPU-1/2075	The loading of the codes into the stay-alive RAM shall only be performed in the boot-idle state.

R-DPU-1/2080 	The DPU shall respond to ICU and OM power or reset by entering the boot-idle state.

R-DPU-1/2085	The four DSPs shall in turn have access to global memory bus once every 1ms.

### Blue DSP Software

R-DPU-1/3000	The Blue DSP bootstrap code shall reside in ROM.

R-DPU-1/3005	The Blue DSP bootstrap code shall load the ``processing'' blue DSP code from stay-alive RAM upon command from the White DSP.

R-DPU-1/3010	The Blue DSP software shall receive and respond to commands from the White DSP as shall be described in APP-10.

R-DPU-1/3015	The Blue DSP shall respond to alive-inquiry from the White DSP with a latency time of no longer than 5 ms.

R-DPU-1/3020	The Blue DSP shall respond to commands from the White DSP with a latency time of no longer than 2 ms.

R-DPU-1/3025	The Blue DSP software shall calculate the memory location to increment for each MIC event received.

R-DPU-1/3030	Upon command from the White DSP, the Blue DSP software shall alternate between two sets of address calculation coefficients mapping into two separate memory areas.

R-DPU-1/3035	The Blue DSP software shall increment by 1 the calculated memory location for each MIC event received.

R-DPU-1/3040	For Blue Fast mode, the Blue DSP software shall calculate the difference of the accumulated image and the previous accumulated image at the observer specified time resolution longer than TBD ms.

R-DPU-1/3045	Upon the engineering/raw-data mode command from the White DSP, the Blue DSP software shall write 2^20 individual MIC events directly into global memory.

### Red DSP Software

R-DPU-1/4000	The Red DSP operating system shall support the higher level functions written in C.

R-DPU-1/4020	The Red DSP software should perform dedicated tasks such as the SAA.

### Tracking

R-DPU-1/5000	The high level software shall calculate the spacecraft drift and roll using successive tracking frames

R-DPU-1/5005	The DPU will perform tracking, i.e. determination of the satellite's pointing, at a regular interval. This regular interval, the frame time, shall be commandable by the observer through the ICU.

R-DPU-1/5010 	Upon completion of the tracking task, the DPU shall make the results available to other DPU processing tasks, and transmit the pointing information to the ICU for broadcast on the satellite bus.

R-DPU-1/5015	The DPU software shall be able to perform tracking based on data collected from the  detector.

### Science Modes

### Image Mode

R-DPU-1/6100	The  image mode shall consists of 0 to 6 science windows.

R-DPU-1/6105	The  image mode shall consists of no more than 1,000,000 pixels or binned pixels (no more than 1,000,000 memory locations for the tracking frame).

R-DPU-1/6110	This requirement deleted as it is covered by section 4.1.5.

R-DPU-1/6120	The  image mode shall shift successive frames according to the calculated spacecraft drift and co-add the frame into an accumulating image with binning factor specified by the observer.

R-DPU-1/6125	The  image mode shall support binning factors of 1 by 1 (no binning), 2 by 2 (bin 2 pixels in each dimension), 4 by 4 (bin 4 pixels in each dimension), 1 by 2, 2 by 1, 1 by 4, 4 by 1, 2 by 4 and 4 by 2.

#### Fast Mode

R-DPU-1/6200	The Blue DSP shall collect fast mode data, preserving the time and spatial resolution in the specified area.

R-DPU-1/6205	The Blue fast mode shall collect successive images of a designated detector area at a user-specified integration time. This integration time shall be at least 100ms (shorter sampling time is feasible if the MIC integration time is shortened).

R-DPU-1/6210	There shall be 0, 1, or 2 blue fast mode areas  pe exposure available, each fast mode set shall have a user-specified integration time.

R-DPU-1/6215	If there are two Blue fast mode set per exposure, the fast mode sets can support two different integration times.

R-DPU-1/6220	Each Blue fast mode area shall consist of a maximum of 512 pixels.

### Engineering Modes

R-DPU-1/7000	There shall be three engineering modes associated with the detector, raw data mode, full frame mode, and  detector calibration mode.

#### Raw data mode

R-DPU-1/7110	The  raw data mode shall collect the event addresses transmitted by the MIC into memory for transmission to the ground via the ICU. No processing other than the hardware parity bit check and parity bit removal shall occur.

#### Full frame mode

R-DPU-1/7220	The  full frame mode shall collect a full 2048 x 2048 frame binned 1 x 1 in memory which shall be transmitted to ground via the ICU.

R-DPU-1/7225	 full frame mode shall have a maximum image depth of 16 bits.

#### Detector calibration modes

R-DPU-1/7330	The  detector calibration mode shall acquire data and perform TBD task based on TBD algorithm to determine the MIC centroiding coefficients.

#### Red engineering modes

R-DPU-1/7400	Removed

Red Science Modes

This section removed.

## Performance Requirements

R-DPU-2/1000	The DPU shall process MIC events to memory in < 1 ms.

R-DPU-2/1005	The DPU shall provide time stamp data to < 100 ms accuracy under the fast mode (TBC).

R-DPU-2/1010	The DPU shall accept and register 2e5 events/s maximum from the  Detector.

R-DPU-2/1015	The DPU shall identify the stars in the field with the user-specified reference stars and calculate the drift and roll with respect to the frame defined by the reference stars to sub arc second accuracy.

R-DPU-2/1020	The DPU shall process the reference frame with the user-specified filter, scan and select good guide stars for subsequent tracking and perform window and memory management tasks.

R-DPU-2/1025	The DPU shall calculate drift and roll with respect to the reference frame based on the established guide stars in less than 2 s.

R-DPU-2/1030	The DPU shall perform shift-and-add of image mode data acquisition of an 1000 by 1000 to 500 by 500 area in less than TBD s. The time to perform SAA must be less than tracking frame time.

R-DPU-2/1035	The DPU shall initiate reaction/response to ICU command with a latency of no greater than 10 ms.

R-DPU-2/1040	The DPU shall transmit high priority alert to ICU with a latency time of no greater than TBD ms.

## Operational Requirements

R-DPU-3/1000	The bootstrap software shall be stored in ROM.

R-DPU-3/1005	The program software shall be stored in stay-alive RAM.

R-DPU-3/1010	All mode parameters shall be changeable by telecommand.

R-DPU-3/1025	The DPU shall transmit data to ICU as described in the ICU -- DPU Protocol Definitions document (XMM-OM/MSSL/SP/0014).

R-DPU-3/1035	On failure to find good guide stars in the field of view, select default set of poorer stars TBD.

R-DPU-3/1036	On failure to obtain an acceptable tracking solution, the DPU shall co-add the tracking frame with zero absolute pointing error and transmit an alert to the ICU.

R-DPU-3/1040	On out of bounds track solution, the DPU shall TBD.

R-DPU-3/1055	The DPU shall abort an exposure when commanded to do so by the ICU and enter the DPUOS-idle state.

R-DPU-3/1060	On watchdog time-out, the DPU shall return to the boot-idle state.

## Resource Requirements

R-DPU-4/1000	Program memory shall not be more than 66% consumed at flight software delivery

## Verification and Validation Requirements

R-DPU-5/1000	Every requirement of the SRD shall be verified by appropriate inspection analysis, demonstration or test.

R-DPU-5/1005	The DPU software environment shall provide for facilities to read and write to/from memory addresses.

R-DPU-5/1010	The DPU software shall communicate with its EGSE via an ICU/SSI interface card using the same command protocol defined for the ICU (see the ICU - DPU Protocol Definitions document, XMM-OM/MSSL/SP/0014).

R-DPU-5/1015	The DPU software shall support all functions necessary to support testing of the DPU and instrument on ground.

R-DPU-5/1020	The DPU software shall be designed for a flight duration of 10 years.

R-DPU-5/1025	The DPU software shall be tested with representative mission simulations of image data and mode combinations in a flight representative version.

R-DPU-5/1030	The duration of the mission simulation shall be at least equivalent to three orbits of operations and at least 12 different observational target plans.

# Non- Specific Requirements

## Documentation Requirements

R-OM-1/1000	Documentation of the OM S/W shall be compliant with PSS-05 Software Engineering Standards.

R-OM-1/1005	The documentation shall be provided on paper.

## Security Requirements

Not Applicable

## Portability Requirements

Not Applicable

## Quality Requirements

R-OM-4/1000	C code shall conform to the coding standard specified in APP-7

R-OM-4/1005	ADA code shall conform to the Hermes coding standard (TBC).- APP-8

## Reliability Requirements

R-OM-5/1000	The OM S/W shall be designed for a mission duration of 10 years.

## General Maintainability Requirements

These will be guided by the SOC Implementation Plan.

## Safety Requirements

R-OM-7/1000	No malfunction of the DPU or ICU software shall permanently damage or degrade the XMM-OM instrument or spacecraft hardware.

R-OM-7/1005	Hazardous Commands shall be identified during the architectural design phase. Commanding of hazardous functions shall be inhibited by default and only be possible after a preceding enable command with a special key/password. After the execution of a hazardous command it shall be inhibited again.