# XMM Optical Monitor

MULLARD SPACE SCIENCE LABORATORY

UNIVERSITY COLLEGE LONDON				 Author: Kate Olsberg, SNLA

## OM2 Redundant Mass Properties Results

DOCUMENT: XMM-OM/PENN/TR/0017R.01   26-Jun-98

CHANGE RECORD

| ISSUE | DATE | PAGES CHANGED | COMMENTS |
| --- | --- | --- | --- |
| 01 | 26-Jun-98 |  | Initial Release |

XMM / OM2

FM / REDUNDANT

Mass Properties

Test Summary

UNIT NAME:  OM2-FM (Redundant)

Mass:		5479.5 g   (12.1 lbs)

Center of Gravity:	Taken from Reference Hole Ou

XCG  =   54 mm

YCG  =   148 mm

ZCG   =   79 mm

Mass Moments of Inertia:	Taken at CG with respect to Ou

Ixx   =    5.42 e+07 g*mm2

Iyy   =    2.69 e+07 g*mm2

Izz   =    3.89 e+07 g*mm2

Mass Properties tests were performed at the Mass Properties Laboratory, Bld. 892,  Sandia National Laboratories, Albuquerque, New Mexico on 11 March, 1998.

Test Results

Results of the OM2 / FM (Redundant) unit mass properties tests show a slight decrease in mass from that of the EQM (EQM Mass = 5561 g).  Several reasons could account for this decrease: 1.) removal of material from internal shield cards due to interference fit

- slight differences in machining between the EQM and FM units  and 3.) electronic component changes on ICU between EQM and FM units..  The ICD mass (Issue E) still remains, as of this writing,  at 7094 g which includes contingency mass.  This value needs to be updated to reflect a more current status.  The change in mass also constitutes a deviation from EID-A specification of change < 1% between EQM and FM units and will be reflected in the Request for Deviation.  The CoG and MoI values currently reflected on the ICD are based on estimated values and do not reflect measured values.  These will be updated following the Mass Properties test on the FM – Prime unit and the ICD issue will reflect the correct values.

A copy of the official Mass Properties report follows.

kmo

fm_red-massprop.doc

3/18/98