# XMM Optical Monitor

MULLARD SPACE SCIENCE LABORATORY

UNIVERSITY COLLEGE LONDON				 Author: Kate Olsberg, SNLA

## OM2 Request for Deviations

DOCUMENT: XMM-OM/PENN/TR/0019.01   28-Jun-98

CHANGE RECORD

| ISSUE | DATE | PAGES CHANGED | COMMENTS |
| --- | --- | --- | --- |
| 01 | 28-Jun-98 |  | Initial Release |

XMM / OM2

Flight Models

Request for Deviations

UNIT NAME:  OM2-FM (Redundant)

The following items are requested for deviation exception:

- Mechanical Measurements as identified in Table 1 (see attachment)

2.)  Allowable mass difference from EQM unit (exceeds 10 % allowable per EID-A).  Deviation is difference between 5561 g (EQM unit) and 5479.5 g (FM / Redundant).  Allowable deviation is 55.6 g (10%).  Current difference is 81.5 g.

- Non-current ICD.  ICD drawing must be updated to reflect measured FM mass, Center of Gravity and Moments of Inertia.  Values as listed on current ICD (R39988, Issue E) reflect estimated mass (with contingency) as well as estimated Center of Gravity and Moments of Inertia.

UNIT NAME:  OM2-FM (Prime)

The following items are requested for deviation exception:

- Mechanical Measurements as identified in Table 2. (see attachment)

- Allowable mass difference from EQM unit (exceeds 10 % allowable per EID-A).  Deviation is difference between 5561 g (EQM unit) and 5479.5 g (FM / Prime).  Allowable deviation is 55.6 g (10%).  Current difference is 81.5 g.

- Non-current ICD.  ICD Drawing must be updated to reflect measured FM mass, Center of Gravity and Moments of Inertia.  Values as listed on current ICD (R39988, Issue E) reflect estimated mass (with contingency) as well as estimated Center of Gravity and Moment of Inertia.

# Table 1

MECHANICAL DEVIATIONS

OM2-Redundant

| Deviation # | Description | No. of Places | Dwg. Dimension | Tolerance (Class) | Measured Dimension | Deviation | Critical/ Non-Critical |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1a | Connector Cutouts (+X) | 4 | 55.20 | +/- .254 (Class 2) | 54.68 | .52 | NC |
| 1b |  |  |  |  | 54.66 | .54 | NC |
| 1c |  |  |  |  | 54.57 | .63 | NC |
| 1d |  |  |  |  | 54.59 | .61 | NC |
| 2 | Height w/o Screws | 1+ | 108.6 | +/- .254 (Class 2) | 108.66-108.87 | .27 | NC |
| 3a | Height w/ Screws | 2 | 109.9 | +.50 / -.00 | 109.85-110.00 | -.05 | NC |
| 3b |  |  |  |  | 109.91-110.00 | 0 |  |
| 4 | Foot Spotface Depth | 6 | .5 | +/-.127 (Class 1) | .20-.25 | .3-.25 | NC |
| 5 | Ground Lug Base Protrusion | 2 | 5.10 | +/- .127 (Class 1) | 5.15-5.33 | .23 | NC |
| 6 | Box Length w/o Screws | 1+ | 211.33 | +/- .254 (Class 2) | 210.16-210.25 | 1.17-1.08 | NC |
| 7 | Box Length w/ Screws | 1+ | 214.02 | +0.5/-.00 | 213.22-213.37 | -(0.8-.65) | NC |

## Table 2

MECHANICAL DEVIATIONS

OM2-Prime

| Deviation # | Description | No. of Places | Dwg. Dimension | Tolerance (Class) | Measured Dimension | Deviation | Critical/ Non-Critical |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1a | Connector Cutouts (+X) | 4 | 55.20 | +/- .254 (Class 2) | 54.67 | .53 | NC |
| 1b |  |  |  |  | 54.68 | .52 | NC |
| 1c |  |  |  |  | 54.58 | .62 | NC |
| 1d |  |  |  |  | 54.56 | .64 | NC |
| 2 | Height w/o Screws | 1+ | 108.6 | +/- .254 (Class 2) | 108.56-108.86 | .26 | NC |
| 3a | Height w/ Screws | 2 | 109.9 | +.50 / -.00 | 109.83-109.85 | -.0.07 | NC |
| 3b |  |  |  |  | 109.76-109.9 | -0.14 | NC |
| 4 | Ht. To Ground Lug | 2 | 50.80 | +/-.127 (Class 1) | 50.63 | 0.17 | NC |
| 5 | Ground Lug Base Protrusion | 2 | 5.10 | +/- .127 (Class 1) | 5.19-5.25 | 0.9-0.15 | NC |
| 6 | Foot Width | 6 | 16.00 | +/- .127 (Class 1) | 16.03-16.153 | 0.03-0.153 | NC |
| 7 | Box Length w/o Screws | 1+ | 211.33 | +/- .254 (Class 2) | 210.03-21.025 | 1.3-1.08 | NC |
| 8 | Box Length w/ Screws | 1+ | 214.02 | +0.5/-.00 | 213.5-213.62 | -(.52-.4) | NC |