# XMM Optical Monitor

MULLARD SPACE SCIENCE LABORATORY

UNIVERSITY COLLEGE LONDON				 Author: Kate Olsberg, SNLA

## OM2 Declared Process List (DPL)

DOCUMENT: XMM-OM/PENN/TR/0021.01   28-Jun-98

CHANGE RECORD

| ISSUE | DATE | PAGES CHANGED | COMMENTS |
| --- | --- | --- | --- |
| 01 | 28-Jun-98 |  | Initial Release |

XMM / OM2

FLIGHT MODELS

DECLARED  SPECIFICATIONS &  PROCESSES

PROCESS			SPECIFICATION		    DESCRIPTION		USE & LOCATION

ALODINE			MIL-C-5541 C		Chromate Coating		Box Housing, Frames,

Aluminum (Commercial)		Covers, Misc.

CHEMGLAZE		SS-R16205 A			Paint Application Process		Box Exterior

Z306 							Flat Black/ Non-Conductive

CONFORMAL 		SS346233 H/F/G			Conformal Coating		All PWAs,

COATING						Application PC 18-M		Motherboards

CONFORMAL 		QI-808 A			Conformal Coating		All PWAs,

COATING 						Quality Instruction		Motherbaords

CONTRACTOR		SS-S61316 D			Contractor Quality		Conformal Coating

QUALITY						Assurance Program		Quality Control for

Requirements			PWAs, Motherboards

DESIGN CHANGE	SS-S74408 B			Design Change and		SNL PWAs,

REPAIR REQUEST					Repair Request Control		Motherboards

Procedure (SNL Internal)

HANDLING ESD	SS343666 B			Handling Procedure		Internal SNL,

COMPONENTS						ESD Sensitive Devices		GTC Handling,

All Electronics

HANDLING, TESTING	DOD-HDBK-263			Handling & Testing of		Assembly Contractor

ESD COMPONENTS &					ESD Sensitive 			(L&L) for SNL

ASSEMBLIES						Components & Assemblies	PWAs

MARKING		MIL-I-43553	B		Marking of PWAs		Assembly Contractor

(L&L) for SNL

PWAs

METAL CLAD 		MIL-P-13949			Mil Spec for Circuit		SNL Printed

LAMINATE						Board Materials			Circuit Boards

PROCESS			SPECIFICATION		    DESCRIPTION		USE & LOCATION

MULTILAYER PCB		MIL-P-55110 D		Mil Standard for Board	SNL Printed

FABRICATION						Fabrication, Inspection	Circuit Boards

PWB ASSEMBLY		MIL-STD-2000 A		Mil Standard for Board	SNL  PWAs,

& REPAIR							Assembly and Repair		Motherbaords

Abbreviations / Definitions:

PWB - Printed Circuit Board

PWA - Printed Wiring Assembly (includes Printed Circuit Board and Components)

SNL - Sandia National Laboratories

GTC - General Technology Corporation (Conformal Coating Contractor for all PWAs)

L&L - Assembly Contractor for SNL PWAs

NOTE:  Processes and Specifications do not include ICU Modules prior to delivery to SNL.