# XMM Optical Monitor

MULLARD SPACE SCIENCE LABORATORY

UNIVERSITY COLLEGE LONDON				 Author: Kate Olsberg, SNLA

## OM2 Prime Vibration Re-test Results

DOCUMENT: XMM-OM/PENN/TR/0023P.01   26-Jun-98

CHANGE RECORD

| ISSUE | DATE | PAGES CHANGED | COMMENTS |
| --- | --- | --- | --- |
| 01 | 26-Jun-98 |  | Initial Release |

XMM / OM2

FM / REDUNDANT

Vibration Test Report

UNIT NAME:  OM2-FM (Redundant)

Vibration testing of the OM2 / FM Redundant Unit was conducted at the Sandia National Laboratories, Tech Area I facility, Bldg. 860 on 10 March, 1998.  Testing was performed according to the Acceptance Levels for electronic units as defined in the EID-A, Issue 6, Sec. 6.5.3.2.   Two tests, Sinusoidal and Random,  were performed in each of the three mutually perpendicular axes. Test levels were as follows:

Sinusoidal Test Levels

Axis		Frequency (Hz)		Level			Sweep Rate

All		5-22				+/- 7.3 mm		4 Oct/min

22-100				+/- 15 g

Random Test Levels

Axis		Frequency (Hz)		Level		g(RMS)	Duration

All		20-100				As 		   3.3		1 min/axis

100-500			calculated

500-2000			from modified

g Level

Test sequence was as follows:

Test 1:		Z Axis Sine

Test 2:		Z Axis Random

Remove Unit from Vibration Table -

Test 3:		Y Axis Random

Test 4:		Y Axis Sine

Test 5:		X Axis Sine

Test 6:		X Axis Random

Testing was performed using the same vibration plate, bolts, torque, etc. As that for the EQM and STM units.  Testing was performed on two different vibration tables, one for the two planar axes (Y and Z) with a second table for the vertical, X axis.  Refer to photographs for test setup.

Two in-axis accelerometers were used (identical to EQM testing), one for Control (also called Ch 1 on the test plots) and a second, located on the -Y face of the unit, above the Ou mounting foot.  Denoted as Auxiliary Ch 2 on the data plots, its function was to monitor response of the unit to the input levels.  Each test has four plots associated with it; 1.) the Control readout  2.) Control (Ch 1) overlaid with Response (Ch 2)  3.) the Response readout alone (Ch2) and a magnitude comparison between Response / Control (Ch 2 / Ch 1).  First frequency responses of the unit are seen on any Random plot with Ch 2 data.  In all axis tests, first frequency mode occurs above 350 Hz.  The highest gRMS response level seen by the unit was in the Y axis at a level of 6.34 gRMS.  No anomalies were seen during testing or post electronic testing.

All data plots are attached.

kmo

fm_red_vib.doc

3/18/98