EIS Observing Programs

Ken Dere

Web document on EIS data throughput

Operations

Data Throughput Notes

2048

512

EIS CCD

Readout Rate = 500 kHz with 2 ports per CCD

Readout Time = 2.1 s = 1s

A/D conversion at 14 bits

480 arc-sec

512 arc-sec

EIS FOV

FPP FOV

320 arc-sec

160 arc-sec

EIS Data Rates

At each KSC/DSN contact, 2.5 Gbits are downloaded and the EIS share of this telemetry is 0.125 (1/8)

Average data rate assuming on downlink per orbit:

DR = 2.4Gb / (8 * 1.5 hr) = 55 kb/s

Average data rate assuming 10 hours between contacts:

DR = 2.4 Gb / (8 * 10 hr) = 8 kb/s

Nominal maximum EIS data rate into the onboard Data Recorder

DR = 167 kb/s

Short Wave Band - Quiet Sun

| Ion | Wvl | Time to reach 100 counts S/N > 10 | Time to reach 1000 counts S/N > 30 Dv < 1 km/s | Time to reach 10000 counts S/N > 100 DvNT < 1 km/s |
| --- | --- | --- | --- | --- |
| Fe X | 184.54 | 79s |  |  |
| Fe XII * | 186.88 | 94s |  |  |
| Fe XI | 188.23 | 27s |  |  |
| Fe XII | 195.12 | 7s | 72s |  |
| Fe XII * | 196.65 | 85s |  |  |
| Fe XIII | 202.02 | 57s |  |  |

Long Wave Band - Quiet Sun

| Ion | Wvl | Time to reach 100 counts S/N > 10 | Time to reach 1000 counts S/N > 30 Dv < 1 km/s | Time to reach 10000 counts S/N > 100 DvNT < 1 km/s |
| --- | --- | --- | --- | --- |
| He II | 256.32 | 21s |  |  |
| Si X | 258.37 | 100s |  |  |
| Si VIII | 275.35 | 81s |  |  |

Observing Program:  Quiet Sun Structure and Evolution

1 Supergranular cell = 30,000 km in diameter = 40”

Exposure time: 100s

Measurements:	Fe XII intensities S/N >30

Fe XII densities

Fe XII velocities to 1 km/s

He II intensities S/N = 22

FOV: 80” x 512” (2 SG cell wide)

Raster time: 80 x 100s = 8000s = 133m = 2 hr.

Data rate:

9 spectral lines, each window 25 pixels wide

14 bit A/D

DR = 9 x 25 x 512 x 14 / (100s + 1s) = 16 kb/s

Compress by a factor of 3

DR = 5.3 kb/s

Compare to downlink rate: 8 / 55 / 167 kbps

Observing Program: Quiet Sun Dynamics

Exposure time: 20s

Measurements:	Fe XII S/N = 17 (21)

Fe XII Dv = 2 km/s

He II S/N = 10

He II Dv = 5 km/s

FOV: 512” x 40” (1 SG cell) in 1” raster steps

Raster time = 40 x (20 + 1)s = 840s = 14m

Data rate:

3 spectral lines (Fe XII 193.52, 195.12, and He II 256)

DR = 3 x 25 x 512 x 14 / (20 + 1)s = 26 kbps

Compress by 5

DR = 5.1 kbps

Compare to downline rate:   8 / 55 / 167 kbps

Observing Program: Quiet Sun dynamics with the Slot

Dl = Fe XII 193.52 - 195.12 = 1.6Å

@ 0.023 Å/pixel = 70 pixels

Dl = Fe XXIV 255.10 - He II 256.32 = 1.22Å

@ 0.023 Å/pixel = 53 pixels

Exposure time = 20s

Measurements:	Fe XII S/N = 17

He II S/N = 10

FOV : 512” x 50”

Repetition time: 20 s + 1 s = 21 s

Data rate:

3 x 50 x 512 x 14 / (20 +1 s) = 49 kbps

Compression by factor of 5

data rate = 10 kbps

Compare to downlink rate: 8 / 55 / 167 kbps

Short Wave Band - Active Region

| Ion | Wvl | Time to reach 100 counts S/N > 10 | Time to reach 1000 counts S/N > 30 Dv < 1 km/s | Time to reach 10000 counts S/N > 100 DvNT < 1 km/s |
| --- | --- | --- | --- | --- |
| Fe XII * | 186.8 | 1.74 s |  |  |
| Fe XI | 188.23 | 1.0 s |  |  |
| Fe X | 190.0 | 4 s |  |  |
| Fe XII | 193.51 | 0.41 s |  |  |
| Fe XII | 195.12 | 0.24 s | 2.4 s | 24 s |
| Fe XII * | 196.65 | 2.4 s |  |  |
| Fe XIII | 202.02 | 1.4 s |  |  |
| Fe XIII * | 203.8 | 1.6 s |  |  |

Long Wave Band – Active Region

| Ion | Wvl | Time to reach 100 counts S/N > 10 | Time to reach 1000 counts S/N > 30 Dv < 1 km/s | Time to reach 10000 counts S/N > 100 DvNT < 1 km/s |
| --- | --- | --- | --- | --- |
| He II | 256.32 | 2.7 s |  |  |
| Si X | 258.37 | 3.2 s |  |  |
| Fe XVI | 263 | 3.4 s |  |  |
| Fe XIV | 264 | 2.8 s |  |  |
| Fe XV | 284 | 0.8 s | 7.9 s |  |

Observing Program: Active Region Structure and Evolution

Medium Active region extent = 200” E-W

FOV = 512” x 200”

Exposure time: 3 s

Measurements:

Fe XII (DS) S/N = 17

Fe XII 195 S/N = 36

Dv = 1 km/s

He II 256 S/N = 10

Fe XV 284 S/N = 20

Dv = 3 km/s

Raster time: 200 x (3 + 1) s = 800 s = 13 m

Data rate:

13 spectral lines

DR = 13 x 25 x 512 x 14 / (3 + 1 ) s = 582 kb/s

Compress by factor of 5

DR = 116 kb/s

Compare to downlink rate: 8 / 55 / 167 kbps

Observing Program: Active Region dynamics with the Slot

FOV: 512” x 200”

Exposure time: 3 s

Measurements:

Fe XII (DS) S/N = 17

Fe XII 195 S/N = 36

He II 256 S/N = 10

Fe XV 284 S/N = 20

Raster time: 4 x (3 + 1) s = 16 s

Data rate:

DR = 4 x 50 x 512 x 14 / (3 + 1) s = 358 kbps

Compress by factor of 5

DR = 72 kbps

Compare to downlnk rate: 5 / 55 / 167 kbps