# Solar B - EIS

MULLARD SPACE SCIENCE LABORATORY

| UNIVERSITY COLLEGE LONDON | Author: A P Dibbens |
| --- | --- |

SOLAR B - EIS  MASS BUDGET

Document Number:  MSSL/SLB-EIS/SP/004.02    29 June 2000

Distribution:

| NRL |  | G Doschek |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | C Korendyke |  |  |  |  |
|  |  | S Myers |  |  |  |  |
|  |  | C Brown |  |  |  |  |
|  |  | K Dere |  |  |  |  |
|  |  | J Mariska |  |  |  |  |
| ISAS |  | H Hara |  |  |  |  |
|  |  | T Watanabe |  |  |  |  |
| RAL |  | J Lang |  |  |  |  |
|  |  | B Kent |  |  |  |  |
| BU |  | C Castelli |  |  |  |  |
|  |  | S Mahmoud |  |  |  |  |
| Mullard Space Science Laboratory |  | J L Culhane |  |  |  |  |
|  |  | A Smith |  |  |  |  |
|  |  | L Harra |  |  | . |  |
|  |  | A McCalden |  |  |  |  |
|  |  | C McFee |  |  |  |  |
|  |  | R Chaudery |  |  |  |  |
|  |  | P Thomas |  |  |  |  |
|  |  | R Card |  |  |  |  |
|  |  | W Oliver |  |  |  |  |
|  |  | P Coker |  |  |  |  |
|  |  | R Gowen |  |  |  |  |
|  |  | K Al Janabi |  |  |  |  |
|  |  | M Whillock |  |  |  |  |
| SLB-EIS Project Office |  | A Dibbens |  |  | Orig |  |
| Author: |  |  | Date: |  |  |  |
| Authorised By |  |  | Date: |  |  |  |
| Distributed: |  |  | Date: |  |  |  |

CHANGE RECORD

| ISSUE | DATE | PAGES CHANGED | COMMENTS |
| --- | --- | --- | --- |
| 01 | 17 April 2000 | All new |  |
| 02 | 29 June 2000 | 3 | Removal of margin |

| ITEM | MASS kg | ## ACRONYM | COMMENTS |
| --- | --- | --- | --- |
| SPECTROMETER |  |  |  |
| Structure | 25.00 | STR | Composite structure |
| Launch Lock | 1.00 | LOK | Typical for LASCO type |
| Primary Mirror | 5.61 | Optics | EM9907 : 3511C |
| Mirror Mount | 0 |  | Included in Primary Mirror |
| Scan Mechanism | 0 |  | Included in Primary Mirror |
| Grating | 1.95 |  |  |
| Grating Mount | 0 |  | Included in Grating |
| Grating Mechanism | 0 |  | Included in Grating |
| Filters | 0.20 | Filter | Typical thickness and stiffness |
| Slit Assembly | 1.21 | SLA | EM9907 : 3510C |
| Shutter | 0 | SHT | Included in Slit Assembly |
| Focal Plane Assembly | 1.00 | FPA | Estimate |
| Shielding | 0.50 |  | Estimate |
| Readout Electronics | 2.50 | ROE | Estimate |
| Radiator | 4.00 |  | Estimate |
| Comtam Monitors | 0.36 | QCM |  |
| Sensors & Heaters | 0.20 |  | Estimate |
| Multilayer Insulation | 4.00 | MLI | 15 layer blankets with alternating mesh |
| Clamshell | 3.90 | CLM |  |
| Mech/Heater Control | 3.0 | MHC | Estimate |
| Purge Harness | 2.00 | PUR | Estimate |
| Vacuum Harness | 1.00 | VAC | Estimate |
| Sub Total | 57.43 |  |  |
| INSTRUMENT CONTROL UNIT | 6.00 | ICU | Estimate |
| HARNESS | 4.00 | HAR | Scaled from LASCO |
| Total | 67.43 |  |  |