- SOLAR-B Instrument

#### EIS Baseline Thermal Design Document

- Document No. BU/SLB-EIS/TN/005.01

##### Compiled by C V Goodall

- University of Birmingham

- Astrophysics and Space Research

- 04/07/00

- Contents

- Introduction

- The Thermal Model

- Interfaces

- Boundary Temperatures

- Temperature Requirements

### Baseline Thermal Model

- Thermal Properties

- Surface Properties

- Masses and heat capacitance

- Power Dissipations

- Preliminary Results for Baseline Model EIS4

- Heaters

- Orbital Variations

- Powers Across  S/C Interface

- Conclusions

- Caveats

- APPENDIX

- EIS Baseline Thermal Design

Introduction

This document describes the baseline thermal design for EIS.  It is the result of the thermal modelling work carried out by Dr I G Butler.  It is essentially drawn from the results for EIS4 authored by him and  circulated as Document No. BU/SLB-EIS/TN/002.02.  The opportunity has been taken to make some editorial corrections to that document.

The Thermal Model

The thermal model consists of 108 Nodes, of which 5 are boundaries and 45 are arithmetic nodes. The basic design of EIS is shown below with the component names used in the following text.

Shutter

Mid-box

Grating housing

Mirror tube

Primary

mirror

CCD camera

Grating

Baffle

ROE

Inner

Clamshell

Outer

Clamshell

HCU

Figure 1. Components on the EIS instrument and their names used in the thermal model.

This model has the CCD radiator on the +Y side and the E-box radiator on the -Z-side. A small sunshield is added in front of radiator to protect the radiator from direct sunlight. Both steady-state and transient simulations have been performed.

+Y

+Z

EIS4 Radiator Position, and coldfinger.

+X

ROE Radiator

The EIS structure is assumed to be aluminium honeycomb apart from the baffle, which is formed out of 2mm thick CFRP. The whole instrument is wrapped in MLI, and a small radiator is supplied to maintain the temperature of the electronics inside the EIS instrument. The CCDs are cooled by a second radiator situated on the +Y surface of the structure above the CCD.

Thermal cases

The EIS instrument is expected to operate in the following conditions:

|  | Hot-case | Cold-case | Survival-case |
| --- | --- | --- | --- |
| Altitude of orbit Attitude | 600km +Z Pointing towards sun, +Y pointing to North-pole |  |  |
| -angle of orbit | 90o | 58.6o Mid-summer 2004 1289 0.35 216 BOL |  |
| Date used in model | 13/10/2004 |  |  |
| Solar Constant W/m2 | 1421 |  |  |
| Albedo | 0.25 |  |  |
| Infrared from Earth W/m2 | 258 |  |  |
| Thermal optical property | EOL |  |  |
| Operational Condition | Max. heat dump During observation | Min. heat dump during observation | Min. heat dump during no observation |

- Interfaces

The EIS instrument is supported off the Solar-B spacecraft via a number of GRP struts, which attach in three places on the EIS structure. The conductive coupling to the spacecraft have been defined and are expected to be less than:

| From | To | Conductive Link W/K | From | To | Conductive Link W/K |
| --- | --- | --- | --- | --- | --- |
| 302 | 9 | 0.032 | 302 | 10 | 0.045 |
| 302 | 24 | 0.02 | 302 | 25 | 0.02 |

These maximum values have been used in all of the EIS thermal models.

- Boundary Temperatures

In the three thermal cases shown above the expected spacecraft temperatures have been calculated, these are shown below:

|  | Hot-case |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Node | Description | Temperature oC | Node No. | Description | Temperature oC |
| 300 | Solar-B S/C Bus | +50 | 302 | S/C I/F points | +50 |
| 303 | OPA Door | +60 | 304 | PAS | +100 |
| 999 | Space | -269 |  |  |  |
|  | Cold-case/Survival-case |  |  |  |  |
| Node | Description | Temperature oC | Node No. | Description | Temperature oC |
| 300 | Solar-B S/C Bus | -40 | 302 | S/C I/F points | -20 |
| 303 | OPA Door | -10 | 304 | PAS | -90 |
| 999 | Space | -269 |  |  |  |

These temperatures have been used a boundary conditions for the EIS thermal model, though the +Y bus MLI has been modelled as an arithmetic node for completeness.

- Temperature Requirements

Internal to the EIS instrument various components have specified operating temperatures. The most critical of these are the electronics boxes and the optics.

The operational temperature requirements for these components are set out below:

| Component | Maximum Temperature oC | Minimum Temperature oC |
| --- | --- | --- |
| HCU | +40 | 0 |
| ROE | +40 | 0 |
| Optics | +10 | +30 |

During the survival-case it is expected that the temperature variations will increase, thus the non-operating temperature requirements for the various components are shown below:

| Component | Maximum Temperature oC | Minimum Temperature oC |
| --- | --- | --- |
| HCU | +65 | -30 |
| ROE | +65 | -30 |
| Optics | 0 oC | +40 oC |

These temperature requirements have been used as goals for the thermal model, though it would be preferable that the electronics operate in the range +25-+45 oC.

### Baseline Thermal Model

## Nodes

- Description of nodes in viewed from the sun-side of EIS.

|  | Structural Nodes |  |  |
| --- | --- | --- | --- |
| Node No. | Description | Node No. | Description |
| 1 | Mid-box rear wall L/H | 2 | Mid-box rear R/H |
| 3 | Mid-Box L/H wall rear | 4 | Mid-Box L/H wall front |
| 5 | Mid-Box R/H wall rear | 6 | Mid-Box R/H wall front |
| 7 | Mid-box front wall L/H | 8 | Mid-box front wall R/H |
| 9 | Mid-box base rear R/H | 10 | Mid-box base rear L/H |
| 11 | Mid-box base front R/H | 12 | Mid-box base front L/H |
| 13 | Mid-box top rear R/H | 14 | Mid-box top front R/H |
| 15 | Mid-box top rear L/H | 16 | Mid-box top front L/H |
| 17 | Filter | 18 | HCU |
| 19 | ROE | 20 | Grating housing L/H wall rear |
| 21 | Grating housing L/H wall front | 22 | Grating housing R/H wall rear |
| 23 | Grating housing R/H wall front | 24 | Grating housing base rear |
| 25 | Grating housing base front | 26 | Grating housing top rear |
| 27 | Grating housing top front | 28 | Grating housing end |
| 29 | Grating | 30 | Baffle R/H wall rear |
| 31 | Baffle R/H wall front | 32 | Baffle base rear |
| 33 | Baffle base front | 34 | Baffle top rear |
| 35 | Baffle top front | 36 | Baffle end |
| 40 | Mirror tube top rear | 41 | Mirror tube top middle |
| 42 | Mirror tube top front | 43 | Mirror tube base rear |
| 44 | Mirror tube base middle | 45 | Mirror tube base front |
| 46 | Mirror tube L/H wall rear | 47 | Mirror tube L/H wall middle |
| 48 | Mirror tube L/H wall front | 49 | Mirror tube R/H wall rear |
| 50 | Mirror tube R/H wall middle | 51 | Mirror tube R/H wall front |
| 52 | Mirror support/mirror tube end | 53 | Primary mirror |
| 54 | Inner Clamshell | 55 | Outer Clamshell |
| 56 | Camera | 57 | Shutter |
| 59 | CCD | 60 | E-box radiator |
| 61 | CCD Radiator | 70 | Baffle |
|  | Arithmetic Nodes |  |  |
| Node No. | Description | Node No. | Description |
| 101 | Mid-box rear wall L/H MLI | 102 | Mid-box rear R/H MLI |
| 103 | Mid-Box L/H wall rear MLI | 104 | Mid-Box L/H wall front |
| 105 | Mid-Box R/H wall rear MLI | 106 | Mid-Box R/H wall front |
| 107 | Mid-box front wall L/H MLI | 108 | Mid-box front wall R/H |
| 109 | Mid-box base rear R/H MLI | 110 | Mid-box base rear L/H |
| 111 | Mid-box base front R/H MLI | 112 | Mid-box base front L/H |
| 113 | Mid-box top rear R/H MLI | 114 | Mid-box top front R/H |
| 115 | Mid-box top rear L/H MLI | 116 | Mid-box top front L/H |
| 120 | Grating housing L/H wall rear MLI | 121 | Grating housing L/H wall front MLI |
| 124 | Grating housing base rear MLI | 125 | Grating housing base front MLI |
| 126 | Grating housing top rear MLI | 127 | Grating housing top front MLI |
| 128 | Grating housing end MLI | 130 | Baffle R/H wall rear MLI |
| 131 | Baffle R/H wall front MLI | 132 | Baffle base rear MLI |
| 133 | Baffle base front MLI | 134 | Baffle top rear MLI |
| 135 | Baffle top front MLI | 136 | Baffle end MLI |
| 140 | Mirror tube top rear MLI | 141 | Mirror tube top middle MLI |
| 142 | Mirror tube top front MLI | 143 | Mirror tube base rear MLI |
| 144 | Mirror tube base middle MLI | 145 | Mirror tube base front MLI |
| 146 | Mirror tube L/H wall rear MLI | 147 | Mirror tube L/H wall middle MLI |
| 148 | Mirror tube L/H wall front MLI | 149 | Mirror tube R/H wall rear MLI |
| 150 | Mirror tube R/H wall middle MLI | 151 | Mirror tube R/H wall front MLI |
| 152 | Mirror support/mirror tube end MLI | 62 | CCD Radiator MLI |
| 63 | Sunshield Front MLI | 64 | Sunshield Rear MLI |
| 301 | S/C +Y MLI |  |  |
|  | Boundary Nodes |  |  |
| Node No. | Description | Node No. | Description |
| 300 | Solar-B S/C Bus | 302 | S/C I/F points |
| 303 | OPA Door | 304 | PAS |
| 999 | Space |  |  |

- Thermal Properties

- Conductors

- Aluminium Honeycomb

The aluminium honeycomb structure is modelled as 10mm thick honeycomb plus two sheets of CFRP each 1mm thick. The thermal conductivity of the CFRP is assumed to be 20W/m/K, which may be conservative, but this value is an unknown until measurements are made on an in-house manufactured sheet of CFRP.

- Conductivity of honeycomb in direction of the tapes is given by

- GL=5.4×W×TL

- where

- L is the length in the direction of the tapes

W is the length of the sheet perpendicular to the tapes

T is the thickness of the honeycomb

- The conductivity of the honeycomb perpendicular direction of the tapes is given by

GL=3.6×L×TW

Bolted joints have not been included in the model as the conductivity of the honeycomb is sufficiently low as to dominate the heat flow. This may be introduced in a more detailed model.

- MLI

The MLI is treated as a temperature-variable conductor, data provided by MMS. The appropriate temperature is obtained by taking the mean of the MLI temperature and that of its corresponding wall node.  This is then used to obtain an interpolated value of the conductance from a look-up table of conductances at set temperature points (see Appendix). This value is then used iteratively in deriving steady state temperature values within the thermal programme,

All MLI around the EIS instrument is assumed to be 15-20 layers thick, apart from the radiator MLI which is taken to be a medium MLI 10-15 layers due to the large number of cut-outs.  The baffle is wrapped in a thin MLI, 5 layers (to prevent over-heating in the hot-case).  There is thick MLI on mirror tube and sun facing sides, medium on remaining side of mid-box and thin MLI on remaining sides of the grating housing.

- Baffle

The baffle is thermally de-coupled from the rest of EIS  by conductive links of  0.026W/K to the grating housing and 3.8x10-3W/K to the mid-box assembly. This should minimise the impact of sunlight scattered from the filter, though this does seem to be a design driver even taking these precautions.

- E-boxes

With the extended operating temperature ranges defined March 2000 the HCU is hard mounted to the mid-box structure. This will maintain the structure temperature in the cold-case without the need for extra operational heaters. The higher dissipation of the ROE electronics box means that it must be decoupled from the mid-box structure via insulating supports, and connected to a small radiator (area 0.049 m2) on the -Z side of the mid-box via a copper strap with conductivity 0.4 W/K.

CCD Camera

The CCDs are mounted off an aluminium structure via GRP struts, providing a conductive link of 0.0026W/K. The heat coupled into the CCD via the electrical connections is minimized by using flexible circuits. This should keep the conductive link to the electronics box to 1x10-3 W/K.

The CCDs are cooled to their operating temperature by a large radiator (area

0.11 m2) on the +Y side of the mid-box structure. A coldfinger with thermal conductivity 0.25W/K links the CCDs to this radiator. To mount the radiator the rear half of the mid-box top cover is permanently attached whilst the front half is detachable to allow access to the shutter mechanism. The radiator is mounted off 10 GRP struts each 10mm diameter and 30mm long, providing a conductive link to the top of the mid-box structure of 4x10-3W/K.

- Surface Properties

- Interior

All interior surfaces are assumed to be covered with a high emissivity Kapton tape, =0.78/=0.94 BOL and EOL  except for the right-hand wall of the grating housing facing the baffle, and the right-hand front wall of the mid-box facing the baffle, which are covered in a low emissivity tape, =0.05/=0.1 BOL and EOL.

Other components inside the EIS instrument have the following properties:

| Node | Description |  |  |  |  | Node | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BOL | EOL | BOL | EOL |  |  | BOL | EOL | BOL | EOL |
| 17 | Filter | 0.08 | 0.1 | 0.08 | 0.1 | 18 | HCU | 0.9 | 0.9 | 0.9 | 0.9 |
| 19 | ROE | 0.08 | 0.1 | 0.05 | 0.05 | 29 | Grating | 0.2 | 0.2 | 0.2 | 0.2 |
| 53 | Primary mirror | 0.2/0.02 | 0.2/0.02 | 0.2/0.02 | 0.2/0.02 | 54 | Inner Clamshell | 0.2 | 0.2 | 0.2 | 0.2 |
| 55 | Outer Clamshell | 0.2 | 0.2 | 0.2 | 0.2 | 56 | Camera | 0.08 | 0.1 | 0.05 | 0.05 |
| 57 | Shutter | 0.2 | 0.2 | 0.2 | 0.2 | 59 | CCD | 0.5 | 0.5 | 0.5 | 0.5 |

The electronics boxes are covered in a low emissivity tape to minimise radiation to the surrounding experiment. This is required in order to prevent the EIS instrument becoming too hot in the hot-case. Clamshells, shutter, mirror and grating are mounted off bare aluminium structures at the moment. More detailed modelling of these components will be required in the future.

- Exterior

All exterior MLI has a surface property of =0.34/=0.7 BOL and EOL, apart from the mirror tube which has low emissivity surface layer, =0.05/=0.1 BOL and EOL,  to reduce the heat leakage.

| Node | Description |  |  |  |  | Node | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BOL | EOL | BOL | EOL |  |  | BOL | EOL | BOL | EOL |
| 60 | E-Box Radiator | 0.09 | 0.15 | 0.9 | 0.9 | 61 | CCD Radiator | 0.09 | 0.15 | 0.9 | 0.9 |
| 61 | CCD Radiator MLI | 0.34 | 0.34 | 0.7 | 0.7 | 300 | S/C Bus | 0.32 | 0.4 | 0.57 | 0.57 |
| 301 | S/C +Y MLI | 0.32 | 0.4 | 0.57 | 0.57 | +303 | OPA Door +Z | 0.15 | 0.2 | 0.05 | 0.05 |
| -303 | OPA Door -Z | 0.15 | 0.2 | 0.9 | 0.9 | +304 | PAS +Z | 0.65 | 0.78 | 0.8 | 0.8 |
| -304 | PAS -Z | 0.9 | 0.9 | 0.76 | 0.76 | 300 | +Y Radiator | 0.08 | 0.16 | 0.78 | 0.78 |

- Masses and heat capacitance

To compute the transient temperature variations around the orbit the mass and heat capacity of all nodes need to be included. The values used are shown below:

| Node | Description | Mass | Cp | Node | Description | Mass | Cp |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | kg | J/kg/K |  |  | kg | J/kg/K |
| 1 | Mid-box rear wall L/H | 0.254 | 850 | 2 | Mid-box rear R/H | 0.137 | 850 |
| 3 | Mid-Box L/H wall rear | 0.37 | 850 | 4 | Mid-Box L/H wall front | 0.37 | 850 |
| 5 | Mid-Box R/H wall rear | 0.37 | 850 | 6 | Mid-Box R/H wall front | 0.37 | 850 |
| 7 | Mid-box front wall L/H | 0.254 | 850 | 8 | Mid-box front wall R/H | 0.637 | 874 |
| 9 | Mid-box base rear L/H | 0.418 | 850 | 10 | Mid-box base rear R/H | 0.418 | 850 |
| 11 | Mid-box base front L/H | 0.418 | 850 | 12 | Mid-box base front R/H | 0.418 | 850 |
| 13 | Mid-box top rear R/H | 0.418 | 850 | 14 | Mid-box top front R/H | 0.418 | 850 |
| 15 | Mid-box top rear L/H | 0.418 | 850 | 16 | Mid-box top front L/H | 0.418 | 850 |
| 17 | Filter | 8.9x10-6 | 880 | 18 | HCU | 2.5 | 880 |
| 19 | ROE | 6.0 | 880 | 20 | Grating L/H wall rear | 0.35 | 850 |
| 21 | Grating L/H wall front | 0.35 | 850 | 22 | Grating R/H wall rear | 0.35 | 850 |
| 23 | Grating R/H wall front | 0.35 | 850 | 24 | Grating base rear | 0.35 | 850 |
| 25 | Grating base front | 0.35 | 850 | 26 | Grating top rear | 0.35 | 850 |
| 27 | Grating top front | 0.35 | 850 | 28 | Grating end | 0.141 | 850 |
| 29 | Grating | 1.95 | 850 | 30 | Baffle R/H wall rear | 0.35 | 850 |
| 31 | Baffle R/H wall front | 0.35 | 850 | 32 | Baffle base rear | 0.35 | 850 |
| 33 | Baffle base front | 0.35 | 850 | 34 | Baffle top rear | 0.35 | 850 |
| 35 | Baffle top front | 0.35 | 850 | 36 | Baffle end | 0.089 | 850 |
| 40 | Mirror top rear | 0.38 | 850 | 41 | Mirror top middle | 0.38 | 850 |
| 42 | Mirror top front | 0.38 | 850 | 43 | Mirror base rear | 0.38 | 850 |
| 44 | Mirror base middle | 0.38 | 850 | 45 | Mirror base front | 0.38 | 850 |
| 46 | Mirror L/H wall rear | 0.35 | 850 | 47 | Mirror L/H wall middle | 0.35 | 850 |
| 48 | Mirror L/H wall front | 0.35 | 850 | 49 | Mirror R/H wall rear | 0.35 | 850 |
| 50 | Mirror R/H wall middle | 0.35 | 850 | 51 | Mirror R/H wall front | 0.35 | 850 |
| 52 | Mirror end | 0.164 | 850 | 53 | Primary mirror | 5.61 | 880 |
| 54 | Inner Clamshell | 1.0 | 880 | 55 | Outer Clamshell | 1.0 | 880 |
| 56 | Camera | 3.0 | 880 | 57 | Shutter | 1.2 | 880 |
| 59 | CCD | 0.5 | 880 | 60 | E-box radiator | 0.73 | 880 |
| 61 | CCD Radiator | 0.73 | 880 | 70 | Baffle | 0.141 | 850 |

- Power Dissipations

The main dissipations within the EIS structure are from the camera electronics (ROE) and the heater control unit (HCU).  The power dissipations used in the thermal model are shown below:

| Node | Description | Power/W | Node | Description | Power/W |
| --- | --- | --- | --- | --- | --- |
| 18 | HCU | 5 | 19 | ROE | 7.1 |
| 59 | CCD | 1.0 |  |  |  |

- Preliminary Results for Baseline Model EIS4

- The temperatures shown in this section are the average orbital temperatures, using the average heat fluxes calculated by the Thermica analysis package.

- Hot-case Temperatures

27.7oC

29.7oC

HCU 33.2oC

ROE 23.4oC

Grating 27.3oC

27.3oC

Filter 101.9oC

21.6oC

18.6oC

Mirror 20.7oC

Shutter 31.5oC

29.3oC

26.5oC

1W

Other Components

CCD Radiator  -63.3 oC

CCD  -54.7oC

E-Box Radiator 5.0 oC

Inner Clam Shell 30.0oC

Outer Clam Shell 26.5 oC

S/C i/f +50 oC.

## Cold-case Temperatures

EIS temperature control 4.5W (Baseline design 15W)

(NB. No CCD control power)

1W

11.8oC

12.2oC

HCU 20.6oC

ROE 3.4oC

Grating 11.7oC

14.8oC

Filter 75.6oC

16.2oC

-23.3oC

Mirror 13.6oC

Shutter 11.6oC

10.9oC

11.1oC

2.5W

1W

Other Components

CCD Radiator. -91.7 oC

CCD -86.4 oC

E-Box Radiator -15.8oC

Inner Clam Shell 11.8 oC

Outer Clam Shell 0.2 oC

S/C i/f  -20.0 oC

- Survival Temperatures

- It is assumed that during the survival mode the spacecraft maintains its attitude with respect to the sun. Therefore the filter and front of the instrument will remain warm.

- Heater Dissipations

CCD control heater 11W

EIS thermal control 9.5W + CCD control 11W. (Baseline design 15W)

2W

0.7 oC

4.0 oC

HCU 3.8

ROE -34.2 oC

Grating 11.2oC

17.8oC

1W

Filter 72.4oC

9.0oC

-21.3oC

Mirror 6.4oC

Shutter 4.0oC

2.3 oC

1.4 oC

2.5W

4W

Other Components

CCD Radiator. -36.1 oC

CCD 7.93oC

E-Box Radiator -43.8oC

Inner Clam Shell 2.9 oC

Outer Clam Shell -5.2 oC

S/C i/f  -20.0 oC

Heaters

Additional heaters are required to maintain the instrument temperature during the various operating phases. Powers are derived from performing a steady-state analysis with the average heat flux from the sun and Earth and are included in the ESATAN N.TAN file.  A summary of the power requirements is shown below:

|  | Hot-case |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Node | Description | Power/W | Node | Description | Power/W |
| 52 | Mirror Support | 1 |  |  |  |
|  | Total | 1 |  |  |  |
|  | Cold-case |  |  |  |  |
| Node | Description | Power/W | Node | Description | Power/W |
| 52 | Mirror Support | 2.5 | 28 | Grating Support | 1 |
| 13 | MBX Base | 1 |  |  |  |
|  | Total | 4.5 |  |  |  |
|  | Survival-case |  |  |  |  |
| 52 | Mirror Support | 2.5 | 29 | ROE | 1 |
| 10 | Mid-box base rear R/H | 1 | 11 | Mid-box base front L/H | 1 |
| 12 | Mid-box base front R/H | 1 | 9 | Mid-box base rear L/H | 1 |
| 28 | Grating Support | 2 |  |  |  |
|  | Sub-Total | 9.5 |  |  |  |
| 59 | CCD | 11 |  |  |  |
|  | Total | 20.5 |  |  |  |

Orbital Variations

The plot (Option D) shows the orbital variation of the CCD temperature for the hot-case.  The variation can be described as

steady state temperature  3º C

Powers Across  S/C Interface

The EIS instrument is supported off the Solar-B spacecraft via a number of GRP struts which attach in three places on the EIS structure. The conductive coupling to the spacecraft has been defined and are expected to be less than:

| From | To | Conductive Link W/K | From | To | Conductive Link W/K |
| --- | --- | --- | --- | --- | --- |
| 302 | 9 | 0.032 | 302 | 10 | 0.045 |
| 302 | 24 | 0.02 | 302 | 25 | 0.02 |

These maximum values have been used in all of the EIS thermal models, and the calculated power flows are as follows:

Hot-case

| From | To | Power Flow W | From | To | Power Flow W |
| --- | --- | --- | --- | --- | --- |
| 302 | 9 | 0.64 | 302 | 10 | 0.87 |
| 302 | 24 | 0.42 | 302 | 25 | 0.45 |

Total power flow 2.38W

Cold-case

| From | To | Power Flow W | From | To | Power Flow W |
| --- | --- | --- | --- | --- | --- |
| 302 | 9 | -1.52 | 302 | 10 | -1.25 |
| 302 | 24 | -0.77 | 302 | 25 | -0.77 |

Total power flow from S/C 4.3W

Survival-case

| From | To | Power Flow W | From | To | Power Flow W |
| --- | --- | --- | --- | --- | --- |
| 302 | 9 | -2.1 | 302 | 10 | -1.57 |
| 302 | 24 | -0.86 | 302 | 25 | -0.78 |

Total power flow from S/C -5.3W

Conclusions

The temperature requirements of the EIS instrument are meet by the design EIS4.  The relaxation of the E-box temperature limits has allowed the HCU to be hard mounted to the structure. This has lead to a reduction in the operational heater power budget. Also the lower survival temperature limit for the ROE has dropped the survival case power budget.  The difficulty is controlling the CCD temperature. At the moment large powers are required to warm the CCD to 0oC. This could be reduced with the introduction of a thermal impedance in the coldfinger, but this would have a detrimental effect on the operational CCD temperature.

Caveats

The coldfinger plus supports has not been modelled and  it is conceivable that there could be a 5oC increase in temperature at the CCDs in both of these models.

The issue of a thermal impedance in the CCD coldfinger could be of importance in this analysis,  though with the careful design of an enclosure for the CCDs the radiative and conductive couplings into the CCD could be minimized, thus reducing the temperature drop across an impedance during operation.

#### APPENDIX

###### MLI Efficiency Tables

0.0299

0.061

0.105

0.154

273

0.0338

0.069

0.119

0.175

293

0.071

0.149

0.257

0.378

393

0.081

0.171

0.295

0.434

413

0.0385

0.079

0.136

0.200

313

0.0085

0.017

0.029

0.042

93

Gap

Large (L)

Medium (M)

Small (S)

Temperature (K)

0.0447

0.092

0.159

0.234

333

0.0193

0.039

0.067

0.098

213

0.0085

0.017

0.029

0.042

0

Efficiency W/m2/K