Solar B - EIS

MULLARD SPACE SCIENCE LABORATORY

| UNIVERSITY COLLEGE LONDON | Author: A P Dibbens |
| --- | --- |

SOLAR B - EIS  ICD DOCUMENT

Document Number:  MSSL/SLB-EIS/SP003.04    5 July 2000

Distribution:

| NRL |  | G Doschek |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | C Korendyke |  |  |  |  |
|  |  | S Myers |  |  |  |  |
|  |  | C Brown |  |  |  |  |
|  |  | K Dere |  |  |  |  |
|  |  | J Mariska |  |  |  |  |
| NAOJ |  | H Hara |  |  |  |  |
|  |  | T Watanabe |  |  |  |  |
| RAL |  | J Lang |  |  |  |  |
|  |  | B Kent |  |  |  |  |
| BU |  | C Castelli |  |  |  |  |
|  |  | S Mahmoud |  |  |  |  |
| Mullard Space Science Laboratory |  | J L Culhane |  |  |  |  |
|  |  | A Smith |  |  |  |  |
|  |  | A James |  |  |  |  |
|  |  | L Harra |  |  |  |  |
|  |  | A McCalden |  |  | . |  |
|  |  | C McFee |  |  |  |  |
|  |  | R Chaudery |  |  |  |  |
|  |  | P Thomas |  |  |  |  |
|  |  | R Card |  |  |  |  |
|  |  | J Tandy |  |  |  |  |
|  |  | W Oliver |  |  |  |  |
|  |  | P Coker |  |  |  |  |
|  |  | R Gowen |  |  |  |  |
|  |  | K Al Janabi |  |  |  |  |
|  |  | M Whillock |  |  |  |  |
| SLB-EIS Project Office |  | A Dibbens |  |  | Orig |  |
| Author: |  |  | Date: |  |  |  |
| Authorised By |  |  | Date: |  |  |  |
| Distributed: |  |  | Date: |  |  |  |

CHANGE RECORD

| ISSUE | DATE | PAGES CHANGED | COMMENTS |
| --- | --- | --- | --- |
| 01 | 29 February 2000 | All new |  |
| 02 | 17 April 2000 | All | Major update following the engineering meeting in Japan, 6-9 March 2000. |
| 03 | 16 June 2000 | 3,4,6,7,8 | Par 4.3.2 units of CLA added. Paras 8.3 & 8.4 added. Par 5.3 updated to reflect the larger ICU base area in contact with the S/C bus. Par 5.4 added. Co-planarity added in par 4.3.2. Par 4, drawing references to structure and templates updated; also mass, M of I, c of g and stiffness properties updated. Par 9, Power Budget updated. References to cables and connectors added to par 8.1 and Appendix 7 added. Par 4.3.3 changed to reflect 20mm dia shear pins. |
| 04 | 05 July 2000 | All | Major revision in preparation for the EIS UK PDR. |

Contents

1	INTRODUCTION	6

2	OVERVIEW	6

2.1	Description	6

2.2	System Hierarchy	6

2.3	Block Diagram of EIS	7

3	APPLICABLE DOCUMENTS	8

4	FILE REFERENCES	8

5	Spacecraft Resource Summary	8

5.1	Mass	8

5.2	Power	9

6	SPECTROMEter	9

6.1	Structure	9

6.2	Stiffness	10

6.3	Mechanical interface	10

6.3.1	Mechanical details	10

6.3.2	Specification of attachment surfaces	10

6.3.3	Attachment Fastening	10

6.3.4	Template	10

6.3.5	Launch Lock	11

6.4	Mass properties	11

6.4.1	Centre of Gravity	11

6.4.2	Moments of Inertia	11

6.5	Motors	11

6.6	Field of View and Exclusion Zone	12

6.7	Disturbances	14

Disturbances to the spacecraft can be caused by both translating and rotational mechanisms.	14

Translating Mechanisms	14

Rotating Components:	15

Conclusions	16

6.8	Provisional EIS Co-alignment	16

6.9	Thermal interface	17

6.9.1	Attached Area	17

6.9.2	Heat Dissipation Across Attachment Points	17

6.9.3	Heat Capacity	18

6.9.4	Acceptable Temperature Ranges	18

6.9.5	Positions of Temperature Measurement	18

6.9.6	Temperature Sensors	19

6.9.7	Interface Temperatures	19

6.9.8	Properties of Outer Surfaces	19

6.9.9	Operational Heaters	19

6.9.10	Survival Heaters	20

7	INSTRUMENT CONTROL UNIT (ICU)	20

7.1	Mechanical Interface	20

7.1.1	Mechanical details	20

7.1.2	Specification of attachment surfaces	20

7.1.3	Attachment Fastening	20

7.2	Mass Properties	20

7.2.1	Centre of Gravity	20

7.2.2	Moments of Inertia	21

7.3	Thermal Interface	21

7.3.1	Attached Area	21

7.3.2	Heat Dissipation Across Attachment Points	21

7.3.3	Heat Capacity	22

7.3.4	Acceptable Temperature Ranges	22

7.3.5	Positions of Temperature Measurement	22

7.3.6	Temperature Sensors	22

7.3.7	Interface Temperatures	22

7.3.8	Properties of Outer Surfaces	22

8	ELECTRICAL INTERFACES	23

8.1	MDP—EIS-ICU Electrical Interface	25

8.2	Overview of MDP— EIS-ICU Electrical Interface	25

8.3	Passive Bi-level Interface	26

8.4	Serial Command Interface	27

8.5	Serial Status Interface	28

8.6	Mission Data Interface	29

8.7	Electrical Pin-outs	30

8.8	System Voltage List	30

8.9	Frequency List	30

9	POWER Distribution	31

10	SOFTWARE INTERFACES	31

10.1	TC packets	31

10.2	Status data	31

10.3	Mission data	32

11	INSTRUMENT MODES	32

12	Contamination Control	33

12.1	Contamination Tests	34

13	Environmental Tests	34

13.1	Mechanical Tests	34

13.1.1	Test Matrix(sub-system)	34

13.1.2	Test Matrix (Equipment within EIS)	34

13.1.3	Test Matrix(system)	34

13.1.4	Test Levels	34

13.2	Thermal Vacuum Tests	34

13.2.1	Test Matrix (System)	34

13.2.2	System Test Levels	34

13.2.3	Test Matrix (Sub-system)	35

13.2.4	Sub-system Test Levels	35

13.2.5	Test Matrix (Equipment within EIS)	35

13.3	EMC Tests	35

13.3.1	Test Matrix	35

13.3.2	Test Levels	35

14	FAIRING ACCESS REQUIREMENTS FOR CLAMSHELL	35

14.1	General	35

14.2	Vacuum Requirement	36

14.3	Fairing Access	36

14.4	Access Hatch Location	36

14.5	Expected Launch Pad Operations	37

15	Nitrogen Purge	37

16	ACRONYMS	37

APPENDICES	1.  Structure GA

2.  Mounting Template

3.  ICU Interface

4.  Electrical Block Diagram

5.  Grounding Scheme

6.  Power Distribution

7.  Cables and Connectors

# INTRODUCTION

Solar-B will study the connections between fine magnetic field elements in the photosphere and the structure and dynamics of the entire solar atmosphere.

The mission will perform three basic types of observation with high spatial, spectral and temporal resolution :

Determination of the photospheric magnetic vector and velocity fields.

Observation of the properties of the resulting plasma structures in the transition region and 
corona.

Measurement of the detailed density, temperature and velocity of these structures.

The EUV imaging spectrometer (EIS) will obtain plasma velocities to an accuracy of <= 10 km s-1 along with temperatures and densities in the transition region and corona at <2 arc sec resolution.

# OVERVIEW

## Description

EIS consists of a multi-layer coated single mirror telescope, and a stigmatic imaging spectrometer incorporating a multilayer coated diffraction grating.  The image produced by the primary mirror is imaged onto an entrance slit/slot and the light which passes through this spectrometer aperture is dispersed and re-imaged at the focal plane of the CCD detectors.

A separate electronics box (ICU) provides the instrument control functions and interface with the spacecraft.

For details of the system definition see RD 3

## System Hierarchy

EUV Imaging Spectrometer

Structure

Mirror Assembly

Grating Assembly

Slit-Slot Assembly

Clamshell Assembly

Camera Assembly

Sensors and Heaters

Mechanism and Heater Control Unit

Thermal Blanket

Instrument Control Unit

Processor

Camera Buffer

Camera Mechanism Controller

Power Conditioner

Harness

## Block Diagram of EIS

# APPLICABLE DOCUMENTS

RD 1   NAO/SLB-EIS/SP/MDP001		MDP-EIS-ICU Electrical Interface

RD 2	MSSL/SLB-EIS/SP/004   		Mass Budget

RD 3	MSSL/SLB-EIS/SP/011   		EIS System Definition

RD 4	MSSL/SLB-EIS/PA/003  		Cleanliness Control Plan

RD 5	MSSL/SLB-EIS/PA/002  		PA Plan

RD 6	SLB-124				Environmental Conditions for Solar B

RD 7	SR 8189

RD 8	Solar B Electrical Design Standards (Japan)

# FILE REFERENCES

The following files are available at the ftp site indicated

FR1	SOLARB-8193.dxf	EIS Spectrometer GA drawing	Birmingham

FR2	SR8154-B.dxf		Mounting Template Drawing		Birmingham

FR3	SR8224.dxf		Interface drawing for Launch Lock	Birmingham

FR4				Provisional ICU Interface		MSSL

Site addresses:

Birmingham	ftp://cad8.sr.bham.ac.uk/pub/solarb/mech

MSSL		TBA

NRL		TBA

# Spacecraft Resource Summary

The following sections provide the high level status of the mass and power budgets. No contingencies are held within the EIS project but rather they are held by the ISAS team and are available through a process of justifiable request.

## Mass

| Subsystem | Mass (kg) |
| --- | --- |
| Spectrometer | 57.43 |
| ICU | 6.0 |
| Harness | 4.0 |
| Total | 67.43 |

The Instrument Mass Budget is shown in RD 2.

## Power

| Mode | ICU | MHC | CAM | Mechs Pwr | Av Power | Pk Power |
| --- | --- | --- | --- | --- | --- | --- |
| Off | Off | Off | Off | Off | 0.0 | 0.0 |
| Boot | On | Off | Off | Off | 14.2 | 17.1 |
| Standby | On | Off | Off | Off | 14.2 | 17.1 |
| Emergency Safe | On | Off | Off | Off | 14.2 | 17.1 |
| Manual | On | On | On | On | 39.8 | 55.3 |
| Auto | On | On | On | On | 39.8 | 55.3 |
| Engineering | On | On | On | Off/On | 39.8 | 55.3 |
| Bake-out | On | Off | Off | Off | 44.2 | 44.2 |

Note 1. All values are in Watts and refer to primary power.

Note 2. When the operational heaters are on the power is pulse width modulated. This excludes the CCD heater which is listed separately

Note 3. The design is such that operational heaters will be switched off while mechanisms are moved.

Note 4. Survival power is not included.

Note 5 – The CCD heater will be used to decontaminate the CCD and will be used with other power systems switched down

# SPECTROMEter

## Structure

The subsystems of the spectrometer are supported by a composite structure. This structure consists of a single base plate which performs the function of an optical bench. The optical elements are mounted directly (or near directly) from inserts within this composite base. The Spectrometer enclosure is formed by side and top composite panels which are held together with titanium inserts. The upper panel is divided into two parts, one of which is removable to provide access to the grating and slit-slot assemblies. Access to the mirror is via the associated end panel. The structure also provides for optical baffling.

The mechanical structure of EIS is shown in the drawing GA Proposal, SR8193, see Appendix 1 (file reference FR 1).

## Stiffness

The lowest characteristic frequency is 75Hz about the mounting legs.

## Mechanical interface

### Mechanical details

Details of the mechanical interface of EIS with the spacecraft are shown in the drawing GA Proposal, SR8193, see Appendix 1 (file reference FR 1).

### Specification of attachment surfaces

Attachment surface is titanium insert within a  molded carbon fibre composite

Surface roughness = 1.6m CLA (Centre-Line-Average)

Co-planarity = ± 0.05mm

### Attachment Fastening

There are three holes with 3 x 5/16” Unified tapped thread holes with 2xConcentric dia. 16.0 H7 (B0 & C0) and 1xConcentric dia.20.0 H7 (A0) holes for special shear

bushes.  The bolts are 3 x 5/16” Unified tapped thread with shear bushes 32mm

long, concentrically positioned.

Fastener Torque: TBD

The required drawing are:

For A0 hole: drawing SR8217

For C0 hole: drawing SR8218

For B0 hole: drawing SR8219

### Template

The details of the interface template are shown in the drawing Mounting Template, SR 8154, see Appendix 2 (file reference FR 2).

### Launch Lock

While the present baseline does not include a Launch Lock it has been deemed prudent to provide an appropriate interface if one is introduced later. This interface is shown in drawing SR 8224 (see Appendix ?).

## Mass properties

The spectrometer mass is provided in section 5.1.

### Centre of Gravity

The centre of gravity is at:

x = -0.254m

y = 0.112m

z = 1.69m.

The coordinate system is defined from a local origin in EIS.

### Moments of Inertia

Ixx = 2.29 kg.m²

Iyy = 54.3 kg.m²

Izz = 55.3 kg.m²

## Motors

Electric motors are used in the EIS instrument to move mechanisms.  The following table summarizes their characteristics:

Table 6.5 Mechanism Characteristics

| Mechanism Subassembly | Translation | Actuator | Encoder | Average Duty Cycle | Peak Internal Power | Average Power |
| --- | --- | --- | --- | --- | --- | --- |
| MIR Primary Mirror Subassembly | Coarse Position | Size 16, 4 phase stepper motor | Resolver | 2 (20 sec) operations per day | 20 W | 0.0092 W |
|  | Fine Position | Piezoelectric Transducer | Strain gauge | 0.5 V step per five seconds | 0.29 W | <0.05 W |
| SLA Slit/Slot Subassembly | Slit/Slot Exchange | Size 12, 4 phase stepper motor | Resolver | 2 operations per hour | 6 W | 0.0084 W |
|  | Shutter | Brushless DC motor | Optical encoder | 1 operation every 5 seconds | 2.65 W | 0.0122 W |
| GRA Grating Subassembly | Focus Mechanism | Size 16, 4 phase stepper motor | Optical encoder | 2 (20 sec) operations per month | 20 W | 0.0092 W |
| NOTE: Duty cycle, peak internal power, and average dissipated power values are preliminary estimates. |  |  |  |  |  |  |

## Field of View and Exclusion Zone

The EIS instrument views the Sun through a front aperture at the end of a rectangular baffle tube. This baffle tube extends sunward beyond the thin aluminum filters. The angular size of the Sun is 0.5 degree, and the baffles and aperture openings are sized to accommodate this angle plus a 2 mm margin all around.

Figure 6-6a. EIS Entrance Aperture

The front aperture is actually an oval to accommodate the 10 mm X translation of the primary mirror, but is assumed to be circular here for simplicity and to be 200.2 mm in diameter.

While the Sun only occupies a 0.5 cone angle, the front portion of the baffle tube serves to protect the thin aluminum filters from micrometeorites, orbital debris, and contamination. The most likely source of contamination or damage is from components of the Solar-B spacecraft itself. Outgassing from warm surfaces and particulates such as paint flakes will be very damaging to the filters. For this reason, the front baffle tube has been designed so that the filters have no direct line of sight to other components of the spacecraft. In the present design, there is a light baffle midway between the filter and the entrance aperture. A zone of exclusion in front of the EIS aperture is configured in front of EIS such that no straight-line path within this zone can reach beyond the middle baffle. Such an exclusion zone has a full angular extent of 48 about the S/C Z direction. The entrance aperture center point location in S/C coordinates and footpoint B0 are given as S/C coordinates in Table 6-6. The coordinates of B0 are as supplied by the System (see file fairing.pdf from H. Hara dated 1/18/00), and the EIS aperture has been calculated from this point, should B0 move, the EIS aperture will move with it.

Figure 6-6b. EIS Baffle Tube and Exclusion Zone

Table 6-6. Location of EIS Entrance Aperture

| S/C Coordinate | Footpoint B0 (mm) | Center of EIS Aperture (mm) |
| --- | --- | --- |
| X | 0.0 | 104.9 |
| Y | 605.0 | 721.0 |
| Z | 2767.8 | 3161.5 |

## Disturbances

## Disturbances to the spacecraft can be caused by both translating and rotational mechanisms.

## Translating Mechanisms

- Primary Mirror (MIR): The primary mirror translates back and forth in the X direction by  8mm. This coarsely positions the image of the region of interest on the spectrometer slit. The selection of a single mirror telescope forces us to move the 2.3kg mirror assembly instead of a much smaller secondary mirror to do this. It is driven by one of the stepper motors mentioned above through a gearhead, which turns a ball screw. At its top speed of 200 steps/sec, the mirror can move 16 mm in 28.63 sec. It has already been determined that this is UA, but a speed reduction to 100 steps/sec brings us to a CA angular momentum, and further reductions can be made as necessary. Cell F7 of the spreadsheet contains the time required for a 16 mm translation. The 16mm range is worst case, and corresponds to a slew from the East limb of the sun to the West limb. A mirror translation will be required whenever a new target is selected for EIS. This might occur on the order of once a day.

The cumulative angular momentum for the mirror translation is independent of the speed of the motion, depending only on the mass of the object, the distance from the S/C CG, and the distance traveled. For the EIS Primary mirror, the full range travel results in two UA values. These come into the CA range when moving smaller distances (on the order of 3mm), but we expect that since the motion is slow (taking ~1 min or more) the ACS can keep up with the disturbance. It is expected that re-pointing EIS will only occur when a new target is chosen for all Solar B instruments and the move can be made while fine pointing control is not required. If necessary, EIS can delay its move until the other instruments are finished exposing. It may be necessary to provide advance information to the ACS system to permit it to anticipate the disturbance. The S/C ACS engineers in Japan are studying this.

It is important to note that the range of this motion is limited to 8mm in the X direction, so the cumulative angular momentum cannot grow beyond the maximum reported value. Any movement to an extreme position must be followed by a movement in the opposite direction. In the long term, the EIS targets will be randomly located on the sun and the cumulative angular momentum from the primary mirror will be zero.

- Grating Focus (GRA): A grating focus mechanism is included, and incorporates the same motor and lead screw combination that is used for the primary mirror. Focus adjustments move the grating in the X-Z plane along a line making a 4.3 angle with the Z-axis. The disturbance was calculated for a 1mm movement of the grating, but normal movements are expected to be on the order of 0.25mm or less. The movement was assumed to take 60 sec. It is expected that several focus movements will be made during the commissioning of EIS, and once a best focus is attained, this mechanism will rarely be used. The speed of this movement could easily be increased, as the angular momentum is small.

## Rotating Components:

There are three components that have rotational motions within EIS. The pivoting of the primary mirror during fine scan mode and the rotation of the shutter blade both are operational modes that involve continuous periodic motion. The third, the slit/slot interchange is an intermittent motion.

- Mirror Fine Scan (MIR): The primary mirror fine scan is a rotation of the primary by up to 4 arc min about an axis located at the vertex of the mirror and oriented in the S/C Y direction. During the fine scan, the mirror moves very slowly, for example in steps of ~1 arcsec/sec. At the end of a scan, the mirror flies back to the starting position in about 2 sec. This “flyback” disturbance is calculated here (worst case). However, the sum of the fine scan and the flyback result is zero cumulative angular momentum for each scan cycle. The CG of the mirror is 3.81cm from the axis of rotation, so this is treated as a “static imbalance” disturbance. The disturbances are small since the angular travel is extremely small.

- Shutter (SLA): The shutter blade is directly driven by a Kolmorgen 1” size stepper motor. The shutter is a thin disk with a sector cut out for the open position, so we have a static imbalance. The missing mass of this cutout is the source of this imbalance. Q for this item is 3.89E-6 Kg-m, and the worst cast rotational speed (15.7 rad/sec) is assumed for a 180 rotation in 0.2 sec in the shortest exposures. The rotation axis of the shutter is in the X-Z plane and makes a 4.3 angle with the Z-axis. Reversing the shutter direction periodically possibly after every exposure can null the cumulative angular momentum.

EIS’s shutter executes one complete cycle for each exposure, and the cadence can be as high as 1 Hz. This may be close to a vibration mode of the solar panels. The solar panel mode frequencies are TBD and must be determined on orbit. Exposure cadences must be chosen after launch to avoid exciting these resonances.

- Slit/Slot Mechanism (SLA): The slit/slot mechanism is used to bring different sized apertures to the slit position. There are four apertures located 90 apart on a paddle wheel-like holder. A 90 rotation brings another slit into position. The mechanism is driven by a size 12 (0.75”) CDA Astro stepper motor. A reducing gearhead is used to give the positioning accuracy required. The rotation axis is in the X-Z plane, nearly parallel to X. 14 sec are required to make the 90 movement, and intermittent operations several times per hour might occur in some observing plans. The device is tiny and moves slowly putting its disturbance into the “A” category.

## Conclusions

All the disturbance torques except those associated with retargeting EIS by moving the Primary Mirror were found to be Acceptable (A) or Conditionally acceptable (CA). Steps that will be taken to minimize the UA effects on cumulative angular momentum are as follows: 1) The ACS system will study compensations for the EIS Primary Mirror movements. 2) Primary movements will be scheduled, not autonomous, and will occur within periods where fine pointing is not required by other instruments. 3) Movements of the Primary Mirror will be of duration consistent with the time constants of the ACS control system. 4) EIS will keep the moving mass to the practical minimum consistent with mechanical and optical constraints

## Provisional EIS Co-alignment

The Solar B instrument complement must be sufficiently well co-aligned to have reasonable overlap of the three instrument’s fields of view.  To accommodate co-alignment at the spacecraft level, the EIS instrument will have an optical alignment cube.  The alignment cube will be aligned to the EIS telescope optical axis to within 20 arc-seconds on an optical bench in the laboratory.  The cube will be utilized to co-align the EIS optical axis with the axes of the spacecraft sun sensor and the other instruments.  Along the north-south direction, the EIS field of view is 1024 arc-seconds and along the east-west direction the coarse mirror motion corrects for up to +/- 800 arc-second.  Even with an additive 1 arc-minute tilt of the primary and grating, the EIS field still maintains complete coverage of the SOT field of view and is still well centered within the XRT field of view.

Table 10.  Preliminary internal EIS co-alignment error budget (significant contributors only)

| Subassembly | Significant error budget contributor |
| --- | --- |
| Alignment cube | 10 arc-seconds transfer error to cube front face 20 arc-seconds mechanical variation total: 30 arc-seconds optical axis error note: cube face tolerance of <5 arc-seconds expected |
| Parabolic mirror | 30 arc-seconds optic/mount tilt 30 arc-seconds structure tilt total: 60 arc-seconds of tilt, 120 arc-seconds equivalent solar error |
| Grating | 30 arc-seconds optic/mount tilt 30 arc-seconds structure tilt total: 60 arc-seconds of tilt, 62 arc-seconds equivalent solar error |
| RSS total: | 138 arc-seconds |

Note:  Additive error budgeting at the component interface level has been utilized to simulate a cross-coupled error.  The gross errors in each component were root sum squared to produce the total.

## Thermal interface

### Attached Area

Area of attachment points are:

A0: 7.1 cm2

B0: 8.2 cm2

C0: 8.2 cm2

Power flow across attachment point -5 to +5W

### Heat Dissipation Across Attachment Points

|  | Off | Typical operating | Peak | Survival |
| --- | --- | --- | --- | --- |
| Absolute W A0 B0 C0 | Fill in etc. <2.5 <2.0 <2.0 | <2.5 <2.0 <2.0 | 2.5 2.0 2.0 | 2.5 2.0 2.0 |
| Density W/cm2 A0 B0 C0 | <0.4 <0.3 <0.3 | <0.4 <0.3 <0.3 | 0.4 0.3 0.3 | 0.4 0.3 0.3 |

Note that this is required to be < 5W per attachment point in either direction

### Heat Capacity

The heat capacity of the Spectrometer is 40900 W/K.

### Acceptable Temperature Ranges

| Critical Item | Operating | Survival |
| --- | --- | --- |
| Mirror | +10 to +30oC | 0 to +40oC |
| Piezo actuator | +10 to +30oC | 0 to +40oC |
| Grating | +10 to +30oC | 0 to +40oC |
| CCD | -55 to +30oC | -100 to + 60 oC |
| Clamshell filter | TBC | TBC |
| Secondary filter | TBC | TBC |
| Slit-Slot | +10 to +30oC | 0 to +40oC |

### Positions of Temperature Measurement

The following sensors are provided by the spacecraft at the locations indicated.

| Sensor/Position | X | Y | Z |
| --- | --- | --- | --- |
| Primary Mirror support | 105.0 | 814.38 | 204.05 |
| E-box doubler plate | -261.0 | 668.25 | 1929.30 |
| Grating Support | -114.2 | 633.4 | 3180.0 |
| B/H Mounting clam shells | 105.0 | 827.5 | 2366.5 |
| CCD Temperature monitor | -116.75 | 735.0 | 1736.20 |
| Mid-box base plate | 0.0 | 626.0 | 1966.25 |
| Clamshell actuator | 67.30 | 750.0 | 2406.65 |

These positions are shown in SR 8225 and the co-ordinates are spacecraft.

The coordinate system is defined in RD 7.

### Temperature Sensors

Type: PRT100 (integral with heater pads)

### Interface Temperatures

| Location | Operating | Survival |
| --- | --- | --- |
| A0 | +10oC to +30oC | -30oC to +40oC |
| B0 | +10oC to +30oC | -30oC to +40oC |
| C0 | +10oC to +30oC | -30oC to +40oC |

### Properties of Outer Surfaces

|  | Emissivity |  | Solar Absorptivity |  | IR Specularity | Solar Specularity |
| --- | --- | --- | --- | --- | --- | --- |
|  | BOL | EOL | BOL | EOL |  |  |
| EIS MLI | 0.7 | 0.7 | 0.34 | 0.34 | 0.0 | 0.0 |
| Radiators | 0.92 | 0.92 | 0.09 | 0.16 | 0.0 | 0.0 |

### Operational Heaters

| Location | Maximum power (W) |
| --- | --- |
| Mirror Assembly | 2.5 |
| Grating Assembly | 2 |
| Mid Box | 1.5 |
| CCD (see note 1) | 20 |

Note 1 – The CCD heater will be used to decontaminate the CCD and will be used with other power systems switched down

### Survival Heaters

| Location | Power (W) | Port # |
| --- | --- | --- |
| Mirror Assembly | 3.0 | 1 |
| Grating Assembly | 2.0 | 1 |
| Mid Box | 4.0 | 2 |
| Readout Electronics | 1.0 | 2 |
| CCD | 5.0 | 3 |

# INSTRUMENT CONTROL UNIT (ICU)

## Mechanical Interface

### Mechanical details

Details of the mechanical interface of the ICU with the spacecraft are shown in the drawing, ICU Interface A1 5275 300-3, see Appendix 3 (file reference pal-5275-300-3).

### Specification of attachment surfaces

The attachment point of the ICU is aluminium

Surface roughness = 1.6m CLA (Centre-Line-Average)

### Attachment Fastening

The four mounting holes have a diameter of 5.40 +0.30 - 0.05mm, to suit 10-32 UNF mounting bolts.  The interface details are referenced in par 7.1.1.

The fastener torque is 35.7 - 39.1 kgf-cm (standard MS 16996 - D/N)

## Mass Properties

The mass of the ICU is given in section 5.1

### Centre of Gravity

x = 163.697mm

y = 108.034mm

z = 56.006mm

These values are calculated with respect to a reference datum identified on the drawing, ICU Interface A1 5275 300-3, see Appendix 3 (file reference pal-5275-300-3).

### Moments of Inertia

Ixx = 0.1364 kg.m²

Iyy = 0.2551 kg.m²

Izz = 0.3448 kg.m²

These values are calculated with respect to a reference datum identified on the drawing, ICU Interface A1 5275 300-3, see Appendix 3 (file reference pal-5275-300-3).

## Thermal Interface

### Attached Area

The base of the ICU will be machined flat but for the purpose of this section it is assumed that each of the attachment bolts will provide TBD cm2 useful thermal contact area.

Total area of attachment of the base of the ICU is 766.42 cm2

### Heat Dissipation Across Attachment Points

|  | Typical operating | Peak | Survival |
| --- | --- | --- | --- |
| Absolute W | 21.3 | 25.3 | 0 |
| Density W/cm2 | 0.028 | 0.033 | 0 |

### Heat Capacity

The heat capacity of the ICU is TBD W/K.

### Acceptable Temperature Ranges

| Operating | Survival |
| --- | --- |
| -20oC to +50oC | -30oC to +65oC |

### Positions of Temperature Measurement

The following sensors are provided by the spacecraft at the locations indicated.

| Sensor/Position | X | Y | Z |
| --- | --- | --- | --- |
| Near base datum | 0 | 0 | 0 |
| Top of front panel | 295mm | 121mm | 0 |

These values are calculated with respect to a reference datum identified on the drawing, ICU Interface A1 5275 300-3, see Appendix 3 (file reference pal-5275-300-3).

### Temperature Sensors

Type: PRT100

### Interface Temperatures

| Operating | Survival |
| --- | --- |
| -20oC to +50oC | -30oC to +65oC |

### Properties of Outer Surfaces

|  | Emissivity |  | Solar Absorptivity |  | IR Specularity | Solar Specularity |
| --- | --- | --- | --- | --- | --- | --- |
|  | BOL | EOL | BOL | EOL |  |  |
| EIS MLI | 0.7 | 0.7 | 0.34 | 0.34 | 0.0 | 0.0 |

# ELECTRICAL INTERFACES

There are five electrical interface between EIS-ICU (Interface Control Unit) and Solar-B system components:

- ICU – DIST (Distributor)

- ICU – TCI-B (Telemetry Command Interface B)

- ICU – HKU (House Keeping Unit)

- ICU – HCE (Heater Control Electronics)

- ICU – MDP (Mission Data Processor)

There are two electrical interfaces between EIS electrical components:

- ICU – ROE (Read Out Electronics)

- ICU – MHC (Mechanism and Heater Controller)

EIS-STR

HCE

DIST

TCI-B

EIS-ICU

MDP

survival heater

#1- #3

PIM

DHU

HKU

T-sensor

#1 - #7

T-sensor

#8 - #9

DC/DC

signal line

primary power on/off

CMD+TLM

28V primary power line

secondary power line

bus structure

PIM

PIM

MHC

ROE

Figure 3.1 Overview of electrical interfaces around EIS-ICU

## MDP—EIS-ICU Electrical Interface

The grounding scheme is shown in  MSSL/SLB-EIS/DD002, see Appendix 5.

The scheme for cables and connectors is shown in MSSL/SLB-EIS/DD008, see Appendix 7.

## Overview of MDP— EIS-ICU Electrical Interface

EIS-ICU

MDP

Mission Data

Serial Command

Serial Status

Passive Bi-level Status

Data

Enable

Clock

Data

Enable

Clock

Data

Enable

Clock

Data

Busy

4 ch

1

Figure 4.1 Overview of MDP-ICU electrical interface

1 line/component

MDM-25P (MDP-11)

MDM-37P (MDP-13)

There are four types of electrical interfaces between MDP and EIS-ICU.

[1] Bi-level status interface

Telemetry with bi-level status interface can be transferred to MDP even when MDP-CPU is not running. There are four passive bi-level status lines between MDP and ICU.

[2] Serial command interface

Commands from the ground, DHU, and MDP to ICU are transferred to ICU with this interface.

- Serial status interface

EIS HK status information is sent to MDP with this interface.

[4] Mission data interface

Mission data interface is a high-speed serial interface, which delivers science data from ICU to MDP.

## Passive Bi-level Interface

VDD: +5V  TBD V

U1: CMOS 4049 / 4050

Circuit Type TB-E-R

Circuit Type TB-E-T

C1: 220pF  TBD %

R1: 27 k  TBD %

R2: 10 k( ( TBD %

EIS-ICU

MDP

VDD

U1

R1

C1

R2

DATA 0

DATA 4

・

・

・

Figure 4.2 Interface circuit for passive bi-level status interface

U1

R1

C1

VDD

R2

Serial Command Interface

Point of definition for timing chart

Driver device: HS-26C31

Receiver device: HS-26C32

ENABLE

CLOCK

DATA

100  10%

100  10%

4.7k  10%

receiver

4.7k  10%

driver

No resistance

+

-

100  10%

100  10%

4.7k  10%

receiver

4.7k  10%

driver

No resistance

+

-

100 10%

100  10%

4.7k  10%

receiver

4.7k  10%

driver

No resistance

+

-

EIS-ICU

MDP

insertion of

choke coil

Figure 4.3-1 Interface circuit for serial command interface

Clock signal comes only when serial command data come out from MDP.

・

・

・

・

・

・

T1

T4

ENABLE

CLOCK

DATA

100%

50%

T3

T2

T: 1/f (f=62,500 Hz)

T1 : T/2  10 %

T2 : T   10 %

T3 : T/2  10 %

T4 : T/2  10 %

Figure 4.3-2 Timing chart for serial command interface

## Serial Status Interface

+

-

100  10%

100  10%

4.7k  10%

receiver

4.7k  10%

driver

No resistance

Receiver device: HS-26C32

Driver device: HS-26C31

ENABLE

+

-

100  10%

100  10%

4.7k  10%

receiver

4.7k  10%

driver

No resistance

CLOCK

+

-

100  10%

100  10%

4.7k  10%

receiver

4.7k  10%

driver

No resistance

DATA

EIS-ICU

MDP

point of definition for timing chart

insertion of

choke coil ?

Figure 4.4-1 Interface circuit for serial status interface

Clock signal comes only when serial status data come out from ICU.

・

・

・

・

・

・

T1

T4

ENABLE

CLOCK

DATA

100%

50%

T3

T2

T: 1/f (f=62,500 Hz)

T1 : T/2  10 %

T2 : T   10 %

T3 : T/2  10 %

T4 : T/2  10 %

Figure 4.4-2 Timing chart for serial status interface

## Mission Data Interface

position of definition for timing chart

insertion of

choke coil ?

ENABLE

CLOCK

DATA

+

-

33  10%

33  10%

4.7k  10%

receiver

4.7k  10%

driver

100  10%

+

-

33  10%

33  10%

4.7k  10%

receiver

4.7k  10%

driver

100  10%

+

-

33  10%

33  10%

4.7k  10%

receiver

4.7k  10%

driver

100  10%

33  10%

33  10%

4.7k  10%

driver

4.7k  10%

-

+

100  10%

BUSY

receiver

EIS-ICU

MDP

The grounding scheme is shown in  MSSL/SLB-EIS/DD002, see Appendix 5.

The scheme for cables and connectors is shown in MSSL/SLB-EIS/DD008, see Appendix 7.

## Electrical Pin-outs

TBA

## System Voltage List

EIS takes 28V (max 30V) from the spacecraft and generates +5V, +8V, +15V, +36V, -8V & -15V for sub-system use. +150V is generated in the MHC for drive of the mirror fine positioner.  No detector high voltage is necessary.

Transient voltages due to switching of inductive loads will be reduced to less than 1V above the appropriate sub-system rail voltage.

Voltages used by sub-systems:

ICU:

+5V, +15V, -8V, -15V.

MHC:

+8V, +15V, -8V, -15V, -20V, +150V.

CAM:

+8V, +15V, +36V, -8V, -15V.

## Frequency List

Main power converter (ICU)

Operation at 200kHz (TBC).  Total power is about 30W.

Converter for fine mirror positioner driver (MHC)

Frequency TBD.  Total power about 2.4W.

Converter for 36V CAM bias supply

Frequency TBD, phase locked to CAM clock.  Total power about 800mW.

CAM Clock

In the range 16 - 32MHz.

ICU Processor Clock

20MHz.

MHC Processor Clock

9.816MHz.

# POWER Distribution

The power budget is given in paragraph 5.2.

Power distribution is shown in  MSSL/SLB-EIS/DD003.01, see Appendix 6.

# SOFTWARE INTERFACES

The ICU exchanges three types of data with the MDP.  These are as follows:

## TC packets

Telecommand packets which consist of a command identifier (one byte) followed by up to 132 bytes, as illustrated below:

| CMD ID | Command Parameters |
| --- | --- |
| Command ID (BC0) | Data Area |
| 8 bits | Max. 132 bytes |

The proposed EIS commanding structure is as follows:

| CMD-IDs | Function |
| --- | --- |
| 04 - 09 | Memory dump (defined by MDP side - TBC) |
| E0 - EF | Memory uplink (defined by MDP side - TBC) |
| 20 - 2F | Mode control commands |
| 30 - 3F | PSU commands |
| 40 - 4F | MHC commands |
| 50 - 5F | CAM commands |
| 60 - 6F | Flare operation commands |
| 70 - 7F | Sequence table operations |
| 80 – FF | Spares (excluding E0 – EF) |

## Status data

Status data packets (instrument HK) consist of a 4 bytes header followed by up to 2 kbytes of status data.  The general format is as follows:

| Header Area |  | Data Area |
| --- | --- | --- |
| Data Type | Packet Size | Status Data |
| 8 bits | 24 bits | Max. 2 kbytes |

The following are the EIS status allocations:

Status type 1 - ICU status

| Header Area |  | Data Area |
| --- | --- | --- |
| Data Type | Packet Size | EIS Status-1 Data |
| 8 bits | 24 bits | 100 bytes |

Status type 2 - ICU + PSU + CAM status

| Header Area |  | Data Area |  |
| --- | --- | --- | --- |
| Data Type | Packet Size | EIS Status-1 Data | EIS Status-3 Data |
| 8 bits | 24 bits | 100 bytes | 150 bytes |

Status type 3 - ICU + MHC status

| Header Area |  | Data Area |  |
| --- | --- | --- | --- |
| Data Type | Packet Size | EIS Status-1 Data | EIS Status-3 Data |
| 8 bits | 24 bits | 100 bytes | 150 bytes |

## Mission data

Science data packets consist of a data header, followed by image data.  The maximum size of mission data packet is 256 kpixels (16 bit pixel).  For practical reasons a mission data packet is sent as a series of 4 kbyte sub-packets, as illustrated below:

| header | Image data 1 |  | image data 2 |  | image data 3 | . . . | image data N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| sub-packet (e.g. 4Kbytes) |  |  | sub-packet (e.g. 4Kbytes) |  | sub-packet (e.g. 4Kbytes) |  | sub-packet (e.g. < 4Kbytes) |

Mission data parameters are still under discussion with ISAS.

# INSTRUMENT MODES

The instrument modes are shown in the following diagram:

The following table defines the instrument mode transition commands:

| Command ID | Command Parameter | Mode |
| --- | --- | --- |
| 20 | 01 | Standby |
| 20 | 02 | Manual |
| 20 | 03 | Auto |
| 20 | 04 | Emergency safe |
| 20 | 05 | Bake-out |
| 20 | 06 | Engineering |

# Contamination Control

Details for the contamination control of the EIS instrument in its assembly, integration and testing and commissioning phases are identified in the Cleanliness Control Plan, RD 4.

## Contamination Tests

TBA

# Environmental Tests

## Mechanical Tests

### Test Matrix(sub-system)

| Test/Model | MTM/TTM | PM | FM |
| --- | --- | --- | --- |
| Quasi-static load test 1 | QT | N/A | N/A |
| Acoustic test 1 | QT | N/A | PFT 2 |
| Random vibration test 1 | QT | N/A | PFT 2 |
| Low frequency shock test | QT | N/A | N/A |
| Pyrotechnic shock test 1 | TBD | N/A | N/A |

1 Baseline is random vibration.  Quasi static load test, acoustic test and pyrotechnic shock test will only be performed if these loads are found to be dominant in the design.  Acoustic may be substituted for random, if this is found to be dominant.

2 A protoflight test of either acoustic or random will be performed, whichever is considered dominant.

### Test Matrix (Equipment within EIS)

To be included

### Test Matrix(system)

| Test/Model | MTM/TTM | PM | FM |
| --- | --- | --- | --- |
| Quasi-static load test | TBD | N/A | TBD |
| Acoustic test | QT | N/A | PFT |
| Random vibration test | TBD | N/A | TBD |
| Low frequency shock test | QT | N/A | PFT |
| Pyrotechnic shock test | QT | N/A | PFT |

### Test Levels

The test levels are defined in RD 6.

## Thermal Vacuum Tests

### Test Matrix (System)

| Test/Model | MTM/TTM | PM | FM |
| --- | --- | --- | --- |
| Thermal vacuum cycle | QT | N/A | AT |
| Thermal balance | QT | N/A | N/A |

### System Test Levels

The temperature range for the QT is -30°C to +60°C

The temperature range for the AT is -10°C to +50°C

The vacuum shall be better than 10 -5 mm Hg

The Thermal Balance Test shall be conducted at lower extremes of temperature than the QT (TBD) and for a duration TBD.

Five cycles shall be performed for the thermal vacuum cycle test (TBC).

### Test Matrix (Sub-system)

| Test/Model | MTM/TTM | PM | FM |
| --- | --- | --- | --- |
| Thermal vacuum cycle | N/A | N/A | AT |
| Thermal balance | N/A | N/A | TBD |

### Sub-system Test Levels

The temperature range for the AT is -10°C to +50°C

The vacuum shall be better than 10 -5 mm Hg

Five cycles shall be performed for the thermal vacuum cycle test (TBC).

### Test Matrix (Equipment within EIS)

To be included

## EMC Tests

### Test Matrix

| Test/Model | MTM/TTM | PM | FM |
| --- | --- | --- | --- |
| Conducted and radiated emission and susceptibility | N/A | NONE | ALL |

### Test Levels

The test levels are identified in the Solar B Electrical Design Standard, RD 8.

# FAIRING ACCESS REQUIREMENTS FOR CLAMSHELL

## General

The EIS clamshell (CLM) is a vacuum compartment that protects the EIS entrance filter from harmful environmental conditions (acoustics, contamination, debris, humidity, air gusts, etc.) before and during Solar-B launch.  The filter is a very large thin aluminum filter and as such is quite fragile, an inadvertent touch or rush of air will almost certainly destroy it.  The only safe way to launch such items is under a vacuum of <~1 torr.  The CLM is a special chamber for this purpose, and has two doors that open when the instrument reaches orbit.  It will remain under vacuum from the time of EIS integration until safely deployed in orbit.

## Vacuum Requirement

The CLM is constructed to high vacuum standards and should be evacuable to less than 0.001 Torr.  Use of vacuum greases and lubricants are avoided and organic materials are minimized.  It is expected that the chamber can maintain a vacuum of < 1 torr for up to 21 days.  In all such vacuum systems without active pumping, a gradual rise in pressure is expected with time due to outgassing of material from internal surfaces, diffusion of gas through seals, virtual leaks and real (but tiny) leaks.  This means that the CLM will need to be reattached to a pumping station periodically.  Repumping should be done whenever the pressure in the CLM approaches 1 torr.  The frequency of this operation must be determined during the commissioning of the CLM by plotting the pressure versus time.  Careful cleaning vacuum baking and leak testing will be done to obtain the longest possible interval between pumping operations.

A portable GSE pumping station has been designed to safely perform this operation with the flight filters installed in the CLM.  It can perform both evacuation and backfilling operations and will follow the CLM throughout the EIS development program and launch cycle.  It is necessarily very slow in pumping and backfilling since the filters would not survive any rush of air into or out of the CLM.  The pump connects to the CLM seal-off valve by a long copper tube (flexible) and Swagelok connectors.

The seal-off valve is required to be an integral part of the CLM to avoid having long pieces of tubing become part of the CLM vacuum compartment.  The CLM also provides a rugged support for the valve body.

One or more vacuum gauges will be integral to the CLM so the pressure can be monitored during the launch cycle.  It is possible that one gauge might be read by the MHC so the pressure could be found in the telemetry whenever EIS is interrogated.  A main vacuum gauge will be required that reads out into an EGSE monitor.  A [TBD] connector will be provided for this purpose.  Also integral to the CLM are a Photodiode and LED pair that allow checking the filter for light tightness.  Their EGSE unit can probably be combined with the vacuum gauge EGSE.

## Fairing Access

Using Astro-E as a model, fairing close-out is expected to occur on the order of 12 days prior to launch.  While this is less than the expected 21-day hold time of the CLM, we must be prepared for situations where the hold time is exceeded.  Removing the fairing is much too difficult to contemplate, but a hatch could provide the needed access to re-pump the CLM through the fairing.  The hatch opening would need to be on the order of 200 mm in diameter to permit manual connection of the vacuum hose and operation of the seal-off valve.

The access hatch cover should be left open until 3 days before launch to allow continuous pressure monitoring via the EGSE cable.  A data logging pressure monitor will be provided.  A temporary hatch cover that passes the cable can be used to protect the S/C.  For reliable pressure measurements, the same gauge, cable, and monitor should be used at all times.

## Access Hatch Location

The CLM is presently located at Z=+2334mm relative to the origin of the S/C coordinate system. It has been suggested that the CLM seal-off valve be located on the +Y facing surface of EIS at X=~105.  In this case, the center of the hatch should be at X=105, Y=~1100, Z= 2334 (S/C coordinates) in the fairing skin.  At this point there should be about 250mm clearance between the EIS +Y panel and the fairing, an easy reach for an operator’s hand.  If necessary, small “finger wrenches” can be used in tight places.  These can be tied to the operator’s hand to prevent their being lost in the fairing.

## Expected Launch Pad Operations

There should be near continuous monitoring of the CLM pressure from the time of the last pumpdown until three days  (or less) prior to launch.  The CLM pressure will be plotted as a function of time and extrapolated to the end of the launch window.  Should the pressure be predicted to rise above a predetermined limit, it will be necessary to re-pump the CLM.

This decision should be made [TBD] days before scheduled launch to avoid last-minute chaos.  A final pumpdown within a week of the expected launch window should suffice in any case.  The launch preparation plan should include a critical decision point where a pump/no pump decision is made that allows the pumping work to be carried out with the minimum disruption of the rocket preparations.

The pumping process will require bringing the GSE pumping station to the rocket building, removing the hatch cover, connecting a vacuum hose, evacuating the hose, verifying vacuum hose integrity, opening the CLM valve and pumping until the desired pressure [TBD] is reached.  The process is reversed at the end.  The pumping time depends on the size and length of the hose and can be baselined during CLM development.  Estimated working time should be about two hours to start and two hours to close out after reaching the required pressure.  The CLM pressure should be monitored as long as possible after valve closure to verify that a proper seal has been obtained.  Two experienced vacuum operators should be on hand to conduct this operation safely.

# Nitrogen Purge

During integration and testing of the flight model with the spacecraft, the EIS instrument will be required to be purged with clean, dry nitrogen gas (specification TBC).  This will be a common requirement with the other instruments.  Consideration should be given to a common supply with a combined purge manifold for the instruments.

# ACRONYMS

| Acronym | Meaning |
| --- | --- |
| CCD | Charge Coupled Device |
| CLA | Centre-Line-Average |
| CLM | Clamshell |
| CNP | Connector Panel |
| EIS | EUV Imaging Spectrometer |
| FPA | Focal Plane Assembly |
| GA | General Assembly |
| ICU | Instrument Control Unit |
| MHC | Mechanism and Heater Controller |
| ROE | Read Out Electronics |
| TBC | To be confirmed |
| TBD | To be decided |