# Solar B - EIS

MULLARD SPACE SCIENCE LABORATORY

| UNIVERSITY COLLEGE LONDON | Author: A P Dibbens |
| --- | --- |

EIS  SYSTEM DEFINITION

Document Number:  MSSL/SLB-EIS/SP011.01    29 June 2000

Distribution:

| NRL |  | G Doschek |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | C Korendyke |  |  |  |  |
|  |  | S Myers |  |  |  |  |
|  |  | C Brown |  |  |  |  |
|  |  | K Dere |  |  |  |  |
|  |  | J Mariska |  |  |  |  |
| NAOJ |  | H Hara |  |  |  |  |
|  |  | T Watanabe |  |  |  |  |
| RAL |  | J Lang |  |  |  |  |
|  |  | B Kent |  |  |  |  |
| BU |  | C Castelli |  |  |  |  |
|  |  | S Mahmoud |  |  |  |  |
|  |  | G Simnett |  |  |  |  |
| Mullard Space Science Laboratory |  | J L Culhane |  |  |  |  |
|  |  | A Smith |  |  |  |  |
|  |  | A James |  |  | . |  |
|  |  | L Harra |  |  |  |  |
|  |  | A McCalden |  |  |  |  |
|  |  | C McFee |  |  |  |  |
|  |  | R Chaudery |  |  |  |  |
|  |  | P Thomas |  |  |  |  |
|  |  | W Oliver |  |  |  |  |
|  |  | P Coker |  |  |  |  |
|  |  | R Gowen |  |  |  |  |
|  |  | K Al Janabi |  |  |  |  |
|  |  | M Whillock |  |  |  |  |
| SLB-EIS Project Office |  | A Dibbens |  |  | Orig |  |
| Author: |  |  | Date: |  |  |  |
| Authorised By |  |  | Date: |  |  |  |
| Distributed: |  |  | Date: |  |  |  |

CHANGE RECORD

| ISSUE | DATE | PAGES CHANGED | COMMENTS |
| --- | --- | --- | --- |
| 01 | 29 June 2000 | All new |  |

CONTENTS

1.   INTRODUCTION

2.   SCOPE

3.   DOCUMENTS, GLOSSARY AND ACRONYMS

4.   INSTRUMENT REQUIREMENTS

5.   SYSTEM BREAKDOWN

6.   SYSTEM DETAILS

APPENDIX  A (Instrument Block Diagram)

1.   INTRODUCTION

Solar-B will study the connections between fine magnetic field elements in the photosphere and the structure and dynamics of the entire solar atmosphere.

The mission will perform three basic types of observation with high spatial, spectral and temporal resolution :

Determination of the photospheric magnetic vector and velocity fields.

Observation of the properties of the resulting plasma structures in the transition region and 
corona.

Measurement of the detailed density, temperature and velocity of these structures.

The EUV imaging spectrometer (EIS) will obtain plasma velocities to an accuracy of <= 10 km s-1 along with temperatures and densities in the transition region and corona at <2 arc sec resolution.

2.  SCOPE

The purpose of this document is to reflect the design of the EIS instrument that will satisfy the requirements, as stated in Section 4.

3.   DOCUMENTS, GLOSSARY AND ACRONYMS

3.1  Applicable Documents

AD 1	MSSL/SLB-EIS/SP007		EIS Science Requirements

AD 2 	MSSL/SLB-EIS/SP003 		EIS Interface Control Document (ICD)

3.2  Reference Documents

RD 1

3.3  Glossary and Acronyms

AD	Applicable Document

Bfn	Baffles

BU	Birmingham University

CAM	Camera

CCD	Charge Coupled Device

CLM	Clamshell

CUB	Alignment Cube

EEPROM	Electrically Erasable PROM

EIS	E-UV Imaging Spectrometer

ENC	Enclosure

EUV	Extreme Ultra-Violet

EW	East West

FFA	Front Filter Assembly

FOV	Field Of View

FPA	Focal Plane Assembly

FPF	Focal Plane Filter assembly

GRA	Grating assembly

GSE	Ground Support Equipment

ICD	Interface Control Document

ICU	Instrument Control Unit

ISAS	Institute of Space and Astronautical Science (Japan)

LOK	Launch Lock

MHC	Mechanisms and Heater Controller

MIR	Mirror assembly

MLI	Multilayer Insulation

MSSL	Mullard Space Science Laboratory

NAOJ	National Astronomy Observatory, Japan

NRL	National Research Laboratory, USA

NS	North South

PROM	Programmable Read Only Memory

PUR	Instrument Purge harness

QCM	Quartz Control Monitor

RAD	Radiator

RAL	Rutherford Appleton Laboratory, U.K.

RAM	Random Access Memory

RD	Reference Document

RMS	Root Mean Square

ROE	Readout Electronics

SHT	Shutter assembly

SLA	Slit assembly

SLB	Solar-B

STR	Spectrometer

S-PUR	Spacecraft purge harness

S-VAC	Spacecraft vacuum harness

S-WIR	Spacecraft wiring harness

TBC	To Be Confirmed

TBD	To Be Defined

VAC	Instrument Vacuum harness

WIR	Instrument Wiring harness

XRT	X-Ray Telescope (instrument on Solar-B)

4.  INSTRUMENT REQUIREMENTS

4.1  Science Requirements

These are defined in AD1.

4.2  Performance requirements

The performance requirements for EIS are summarized in the following table:

| Requirement | Value |
| --- | --- |
| Effective Area | 0.42 cm2 @ 270 Å |
| Wavelength ranges | 250 - 290 Å |
|  | 170 - 210 Å |
| Spatial Resolution on sun | 2 arc secs |
| Equivalent Spectral resolution | 20 km/s |
| Systematic measurement error | <1% |
| Field of View | 4 arc minutes |
| Scan Range | 4 x 2 arc minute |
| Maximum misalignment | +/- 1 arc minute |
| Observing Modes |  |
| Primary | Slit spectroscopy with Line selection |
| Secondary | Monochromatic Imaging |
| Data Rate | 64 kbps max |
| Data Volume | 384 Mbits/orbit |
| Mass | <70 kg |
| Power | <60 Watts |
| 1st Resonance Frequency | >70 Hz |

4.3  Interface Requirements

Spacecraft interface details are specified in the EIS ICD (AD 2).

5.  SYSTEM BREAKDOWN

A block diagram of the instrument is shown in Appendix A.

A list of the system hardware is shown below and references are made to the system software which will be described in separate documents.

5.1 Flight Equipment

5.1.1 Spectrometer

ENC	Enclosure

BFn	Baffles

LOK	Launch lock

CLM	Clamshell

FFA	Front filter assembly

MIR	Mirror assembly

SLA	Slit assembly

FPF	Focal plane filter assembly

GRA	Grating assembly

SHT	Shutter assembly

CUB	Alignment cube

CAM	Camera

FPA	Focal plane assembly

ROE	Readout electronics

QCM	Quartz control monitor

MHC	Mechanisms and heater controller

WIR	Instrument wiring harness

PUR	Instrument purge harness

VAC	Instrument vacuum harness

MLI	Multilayer insulation

5.1.2  Instrument Control Unit

The ICU is sited on the spacecraft bus and interfaces with the MDP on the spacecraft and the ROE and MHC on the Spectrometer.

5.1.3  Spacecraft Harnesses

S-WIR		Spacecraft wiring harness (ICU to spacecraft and ICU to Spectrometer)

S-PUR		Spacecraft purge harness  (clean dry nitrogen line for purging the 
 			Spectrometer)

S-VAC	Spacecraft vacuum harness (vacuum line for the Clamshell)

5.2  Non-flight hardware

These items may be installed in the instrument during integration and test, but must be removed before launch.  Typically they would consist of such things as:

Contamination witness plates

Aperture cover

Test interface connectors

Connector savers

Lifting attachments

Installation jigs

Shims

Alignment mirrors

5.3  Ground Support Equipment (GSE)

The following GSE would be provided:

Purge gas supply

Vacuum pump and accessories

Alignment jigs, light sources, targets etc

Electrical GSE

Quick-look data analysis workstations

Operational data center workstations

5.4  Logistics Elements

For logistical support, the following will be required:

Shipping containers

Support frames

Clean tents

Toolkits

Material supplies

6.  SYSTEM DETAILS

6.1  Spectrometer General

EIS consists of a multi-layer coated single mirror telescope, and a stigmatic imaging spectrometer incorporating a multilayer coated diffraction grating.  The image produced by the primary mirror is imaged onto an entrance slit/slot and the light which passes through this spectrometer aperture is dispersed and re-imaged in the focal plane of the CCD camera.  The spectra will be focused on two CCD detectors.

There are two reflections in the system and the two wavelength bands are:

170-210Å and 250-290Å, as limited by the grating

180-204Å and 250-290Å useful ranges, as limited by the multilayer coatings

6.2  Field of View (FOV)

Extent of  EW FOV			2080 arcsec on sun  (given by sum of coarse and fine 
 					telescope positioning)

Max EW scan range, fine only	±240 arcsec

Max extent of NS FOV		512 arcsec

Spectral scale

Spectral resolution

Spatial scale

Spatial resolution

Effective area

Scattered light

6.3  Filters

The primary incidence filter is an aluminium foil on a support mesh and fitted into a quadrant frame. The material for the focal plane filter is the same.

Transmission

Stray light throughput

Obscuration

6.4  Telescope Mirror

Type					Off-axis paraboloid

Aperture				15 cm clear

Area					88.4 cm2 per band

Focal length				1934 mm

Off-axis distance			70 mm

Plate scale at focus (slit)		9.37 microns/arcsec

Multilayer coating			Mo/Si

Number of coats			20

Layer period, short wavelength	105 Å

Layer period, long wavelength	145 Å

Coarse position method		Stepper motor

Range ±			800 arcsec solar image motion

Precision

Repeatability

Speed

Fine position method			Piezo electric actuator

Range ±			240 arcsec solar image motion

Precision			1/3 arcsec per step

Repeatability

Speed

6.5  Slits (SLA)

Number				4

Sizes	1				1”

2				40”

3				TBD

4				TBD

Slit change time			TBD

Alignment				During integration

Exchange mechanism step size	15 arcmin

Tolerance perpendic. to optical axis	1 micron

Tolerance parallel to optical axis	13 microns

6.6  Shutter (SHT)

Type					Rotary vane

Position in optical system		Adjacent to slits

Maximum exposure time		Could be infinite

Minimum exposure time		50ms

Repeatability of exposure time	5% of minimum

Lifetime				6x107 operations

6.7  Grating (GRA)

Mount					Magnifying

Slit distance				1m

Detector distance			1.4m

Diameter of optic			100mm

Figure					Toroid

Radius in Saggital		1182.940mm

Radius in Tangential		1178.280mm

Substrate surface roughness		5Å RMS

Ruling density				4200 lines/mm

Ruling type				Holographic with straight laminar grooves, uniform line 
 					spacing

Groove depth				58Å

Multilayer				Mo/Si

Number of coats			20

Layer period, short wavelength	105Å

Layer period, long wavelength	145Å

6.8  Focal Plane Assembly (FPA)

Number of detectors			2

Detector technology			Back-illuminated CCD

Manufacturer				Marconi (EEV)

Type					CCD-42-20

Format - columns (spectral)		2048

Format - rows 	(spatial)		1024

Orientation of readout register 	Parallel to rows

Image mode 				Full frame, not frame transfer

Pixel size				13.5 microns

Minimum read noise 			5 electrons

Maximum binned capacity 		>500,000 electrons

Full well capacity - image		90,000 electrons

Full well capacity - image 		5,000 photons (190Å)

Full well capacity - image 		6,570 photons (250Å)

Anti-blooming structures		None

Quantum Efficiency			0.8

Hot pixels				<100

Column defects 			0

Operating temperature 		-55 C (TBD)

Position resolution FWHM		16 microns

Dark Current				0.1 electron/pixel @ -55 C

Electrons per photon @ 190Å		18

Electrons per photon @ 250Å		13.7

Shielding				15 mm Al equivalent

6.9  Read-out Electronics (ROE)

Digitisation Level			14 bits

Gain					>5.5 electrons/DN

Number of readout ports		2 per CCD

Read-out time				Variable

Number of ADC-chains		4

Simultaneous CCD operation		Yes

Output links to ICU			2

Number of windows			2 per CCD

Restrictions of windows		Same height on each CCD (TBC)

6.10  Mechanism and Heater controller (MHC)

6.10.1  MHC Hardware

6.10.2  MHC Software

This is described in a separate document

6.11  Instrument Control Unit (ICU)

6.11.1  ICU Hardware

CPU					Temic TSC21020F

CPU speed				20 MHz

Instruction per cycle			1

Instruction word length		48 bits

SEU rate				TBD

1  PROM - fixed program and boot	TBD

2  EEPROM - re-writable program	TBD

3  RAM for running program		TBD

4  RAM data memory			TBD

5  Mass memory - CCD data		TBD

6.11.2  ICU Software

This is described in a separate document

6.12  EIS Internal Electrical Interfaces

6.12.1  High Speed Link

6.12.2  Control Link