| SWIFT-UVOT UNIVERSITY COLLEGE LONDON MULLARD SPACE SCIENCE LABORATORY | H. E. Huckle |
| --- | --- |

UVOT ICU Software USERS MANUAL

Document: MSSL/SWT-UVOT/ML/001.2

Distribution:

| Penn. State |  | J. Nousek |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | P. Roming |  |  |  |  |
|  |  | P. Broos |  |  |  |  |
|  |  | S. Koch |  |  |  |  |
| GSFC |  | D. Bundas |  |  |  |  |
|  |  | R. Borelli |  |  |  |  |
| Mullard Space Science Laboratory |  | R. Card |  |  |  |  |
|  |  | M. Carter |  |  |  |  |
|  |  | M. Cropper |  |  |  |  |
|  |  | B. Hancock |  |  |  |  |
|  |  | M. Hailey |  |  |  |  |
|  |  | H. Huckle |  |  |  |  |
|  |  | K. Mason |  |  |  |  |
|  |  | P. Smith |  |  |  |  |
|  |  | A. Smith |  |  |  |  |
|  |  | P. Thomas |  |  |  |  |
|  |  | J. Tandy |  |  |  |  |
| SWIFT-UVOT Project Office |  | T. Kennedy |  |  | Orig |  |
| Author: |  |  | Date: |  |  |  |
| Authorised By: |  |  | Date: |  |  |  |
| Distributed: |  |  | Date: |  |  |  |

| REVISION SUMMARY |  |  |  |
| --- | --- | --- | --- |
| Version | RELEASE DATE | BRIEF DESCRIPTION/REASON FOR CHANGE | EFFECTED PAGES |
| 01 | 7-Jul-2004 | Initial Version | All |

TABLE OF CONTENTS

1.	INTRODUCTION	9

1.1	Purpose	9

1.2	Scope	9

1.3	Document Overview	9

1.4	Definitions, Acronyms and Abbreviations	10

2.	ICU Software	11

2.1	ICU Overview	11

2.2	ICU States	11

2.3	Design Method	12

2.4	Ada Overview	12

2.4.1	Basic Definitions	12

2.4.2	File Naming Conventions	13

2.4.3	Task Scheduling	13

2.4.4	Identifier Naming Conventions	13

2.5	Software Components for Basic and Operational	16

2.5.1	Object Mapping	16

2.5.2	Definitions	17

2.5.3	Library Routines	17

2.6	Task Priorities	18

2.7	Interrupts	19

2.8	Ada Exception Handling and Debugging	20

2.8.1	Overview	20

2.8.2	Exception Handling	20

2.8.3	Reserved Locations	21

2.9	ICU Delay Adjustment	22

2.9.1	Conversion from TICKS to DURATION'SMALL	22

Calculation	22

2.9.2	TICKS in one and two days	23

3.	Detailed Component Description	24

3.1	Introduction	24

3.2	Bootstrap and Loader Code	24

3.2.1	Bootstrap Code	24

3.2.2	Loader 1	24

3.2.3	Loader 2	24

3.3	Package algebraic_f_exceptions	24

3.3.1	Summary	24

3.4	Package algebraic_functions	25

3.4.1	Summary	25

3.5	Package astro	25

3.5.1	Summary	25

3.5.2	Description	25

3.6	Package chcksum	25

3.6.1	Summary	25

3.6.2	Description	25

3.7	Package command_distributor	25

3.7.1	Summary	25

3.7.2	Description	25

3.8	Package crc	26

3.8.1	Summary	26

3.8.2	Description	26

3.9	Package dcs	26

3.9.1	Summary	26

3.9.2	Description	26

3.10	Package debug	34

3.10.1	Summary	34

3.11	Package detanalog	34

3.11.1	Summary	34

3.11.2	Description	34

3.12	Package detdigital	37

3.12.1	Summary	37

3.12.2	Description	37

3.13	Package detector	37

3.13.1	Summary	37

3.14	Package detsafety	38

3.14.1	Summary	38

3.14.2	Description	38

3.15	Package dpu	38

3.15.1	Summary	38

3.15.2	Description	38

3.16	Package eeprom	40

3.16.1	Summary	40

3.16.2	Description	40

3.17	Package elementary_f_exceptions	40

3.17.1	Summary	40

3.18	Package elementary_functions	40

3.18.1	Summary	40

3.19	Package float_rep	40

3.19.1	Summary	40

3.20	Package heater	41

3.20.1	Summary	41

3.20.2	Heaters and Thermistors	41

3.20.3	Description	41

3.21	Package hk	42

3.21.1	Summary	42

3.21.2	Description	43

3.22	Package icb	43

3.22.1	Summary	43

3.22.2	Description	43

3.23	Package icb_driver	44

3.23.1	Summary	44

3.23.2	Description	44

3.24	Main Program icu.ada	44

3.24.1	Summary	44

3.25	Package icu_mem_manager	44

3.25.1	Summary	44

3.25.2	Description	44

3.26	Package ieee	44

3.26.1	Summary	44

3.26.2	Description	45

3.27	Package importance	46

3.27.1	Summary	46

3.28	Assembler Routine INTVEC.ASM	46

3.28.1	Summary	46

3.29	Package limit	46

3.29.1	Summary	46

3.29.2	Description	46

3.30	Package math_constants	47

3.30.1	Summary	47

3.31	Package mechanism	47

3.31.1	Summary	47

3.31.2	Filter Wheel	47

3.31.3	Dichroic Mechanism	48

3.31.4	Description	48

3.32	Package memloc	50

3.32.1	Summary	50

3.33	Package mem_manager	50

3.33.1	Summary	50

3.34	Package modeman	50

3.34.1	Summary	50

3.35	Package monitor	50

3.35.1	Summary	50

3.36	Package mutex	50

3.36.1	Summary	50

3.36.2	Description	50

3.37	Package nhk	51

3.37.1	Summary	51

3.38	Package packet	51

3.38.1	Summary	51

3.39	Package peek_poke	51

3.39.1	Summary	51

3.39.2	Description	51

3.40	Packages pps_ih and pps2_ih	51

3.40.1	Summary	51

3.40.2	Description	51

3.41	Package primitive_functions	51

3.41.1	Summary	51

3.42	Package reset	52

3.42.1	Summary	52

3.42.2	Description	52

3.43	Package safetyport	52

3.43.1	Summary	52

3.44	Package sc1553	52

3.44.1	Summary	52

3.44.2	Description	52

3.45	Package sc1553_ih	52

3.45.1	Summary	52

3.46	Package ssi	52

3.46.1	Summary	52

3.46.2	Description	52

3.47	Package ssi_ih	53

3.47.1	Summary	53

3.48	Package starcat	53

3.48.1	Summary	53

3.48.2	Description	53

3.49	Package support_functions	57

3.49.1	Summary	57

3.50	Package taskman	57

3.50.1	Summary	57

3.50.2	Description	57

3.51	Package task_report	57

3.51.1	Summary	57

3.52	Package tcq	57

3.52.1	Summary	57

3.53	Description	57

3.54	Package tc_verify	58

3.54.1	Summary	58

3.54.2	Description	58

3.55	Package time	58

3.55.1	Summary	58

3.55.2	Description	58

3.56	Package timer_a_ih	58

3.56.1	Summary	58

3.56.2	Description	58

3.57	Package tmpsu	59

3.57.1	Summary	59

3.57.2	Description	59

3.58	Package tmq	60

3.58.1	Summary	60

3.58.2	Description	60

3.59	Package types	60

3.59.1	Summary	60

3.60	Package uvotman	60

3.60.1	Summary	60

3.60.2	Description	60

3.61	Assembler Routine USERDEFS.ASM	67

3.61.1	Summary	67

3.62	Package version	67

3.62.1	Summary	67

3.63	Package watchdog	67

3.63.1	Summary	67

3.63.2	Description	67

Appendix A	Ada Summary	A-1

A.1	Introduction	A-1

A.2	Ada Procedure and Function Notation.	A-1

A.3	Ada Task and Entry Notation	A-1

A.4	Use of Ada Pragma’s	A-2

A.5	ELABORATE	A-2

A.6	FOREIGN_BODY	A-2

A.7	INLINE	A-2

A.8	LINKAGE_NAME	A-2

A.9	OPTIMIZE	A-2

A.10	PACK	A-2

A.11	PRIORITY	A-2

A.12	SHARED	A-2

A.13	SUPPRESS	A-3

- INTRODUCTION

## Purpose

This document describes design of the software contained within the Instrument Control Unit (ICU) of the Ultra Violet Optical Telescope (UVOT) instrument onboard the SWIFT.

It’s purpose is to provide an understanding of the design of the software..

The intended readership is includes:-

- The technical development team for this software, in order to aid clarification of the software structure and show top-level compliance with the requirements.

- Other UVOT team members, including PI, project manager, system engineers, software management, PA, test managers, EGSE & operations personnel, COI’s, and others to whom requirements, schedule, interfaces, and quality are relevant.

- Anyone else who is interested, including other UVOT experimenters & users.

## Scope

The scope of this document is limited to a description of the ICU onboard software associated with the UVOT instrument. The ICU is primarily concerned with providing overall system control; spacecraft interface data handling, and instrument monitoring.

It does not include UVOT onboard DPU software. The DPU software is primarily responsible for the scientific data collection, processing and forwarding to the spacecraft.

## Document Overview

The code is fully commented. However, the comments tend to be limited to that of the module in which they appear and to concentrate of specifics of a problem local to that module. It is therefore the purpose of this document to be an ‘Explanatory Supplement’ and thus will concentrate on matters of a higher-level nature. When read in conjunction with the comments in the code of a particular module, it will (hopefully) allow the reader to obtain an overall comprehension of the context in which that module functions.

Note: Those comments in the code beginning with a ‘--+’ are used to indicated principle points. All other comments are used for subsidiary information.

## Definitions, Acronyms and Abbreviations

| CCD | Charge Coupled Device (detector) |
| --- | --- |
| CONFIG | CONFIGuration |
| DEM | Digital Electronics Module |
| DMA | Direct Memory Access |
| DPU | Digital Processing Unit |
| DSP | Digital Signal Processor |
| EEPROM | Electrically Eraseable ROM |
| EGSE | Electrical Ground Support Equipment |
| FIFO | First-In First-Out (queue) |
| FOV | Field Of View |
| HK | Housekeeping (data/information) |
| ICB | Instrument Control Bus |
| ICU | Instrument Control Unit |
| I/O | Input-Output |
| MACSbus | Modular Attitude Control System bus |
| NHK | Non-periodic Housekeeping |
| PSU | Power Supply Unit |
| RAM | Random Access Memory |
| ROM | Read Only Memory |
| S/C | Spacecraft |
| S/W | Software |
| SSI | Serial Synchronous Interface |
| TBA | To Be Added |
| TBC | To Be Confirmed |
| TBD | To Be Defined |
| TC | Telecommand queue |
| TM | Telescope Module |
| TM | TeleMetry queue |
| TMPSU | Telescope Module Power Supply Unit |
| UV | Ultra-Violet |
| UVOT | Ultra-Violet Optical Telescope |

# ICU Software

## ICU Overview

The instrument controller provides the overall instrument function. Its main software functions are as follows:-

- Configuring the instrument.

- Monitoring for breakdown/failure conditions (and safing if required).

- Controlling and monitoring status of, the detector, the telescope power supply and the DPU

- Incorporating new or modified code modules for itself

- Collecting and telemetering, instrument housekeeping and engineering packets.

- Interfacing with the spacecraft for data and commands.

- Monitoring and controlling the thermal environment.

The ICU software consists of 3 programs:-

| BOOTSTRAP | This resides in ROM and is copied into RAM for execution. It is responsible for bringing up the ICU in a known safe state after turn on or reset. It also copies the basic state software from ROM to RAM. |
| --- | --- |
| BASIC | This resides in ROM and is copied into RAM for execution. Basic will be responsible for loading the up-linked ICU operational mode code into EEPROM and copying it into RAM and then executing it. It is also responsible for housekeeping, limit checking and basic thermal control and for ensuring the telescope module in a safe state. |
| OPERATIONAL | This is up-linked and will reside in EEPROM and copied into RAM for execution. Operational provides the full functionality of the ICU. |

## ICU States

The purpose and activities of each state are summarized below.

Basic: The ICU enters this state on turn-on. The code executed in this state is resident in ROM and therefore cannot be updated. It ensures that the filter wheel is in the block position and that the high voltages are at zero. It maintains housekeeping, on-board autonomous limit checking, thermal control and enables the watchdog. These latter processes continue to run in all other states. On command to go to the Safe state (note, the basic=>safe transition is not supported in operational code), it first loads the operational code from EEPROM into RAM and then performs the transition. All subsequent states are supported only by the operational code.

Safe: As its name implies, in this state the instrument is configured to be least susceptible to damage. The filter wheel is in a blocked position and the high voltages (HVs) are set to zero.

Idle: The UVOT is configured to be ready to observe but is awaiting the next slew. As a safety precaution the cathode voltage is held down at zero and the filter wheel may be placed in the blocked position until observations commence.

Slewing: In this state the UVOT cleanly shuts down any current observation, prepares the instrument for slewing and informs the spacecraft when it is ready, then selects the next exposure and performs various safety calculations

Settling: After a new GRB has entered the UVOT’s field of view but before the spacecraft has settled, it is observed by collecting an event list. As the target will be moving rapidly, it is not possible to collect an image.

Finding Chart: If the target is a new GRB, then once the spacecraft has settled, a 100 second exposure is made in a standard filter to produce a finding chart that will be sent to the ground. In many cases, by the end of the exposure, the XRT will have reported an improved position for the source. The ICU uses that information, and the GRB brightness information supplied by the BAT, when it configures for the subsequent Automated Target exposures.

Automated Target (AT): During the period whilst a GRB has no pointing constraints – this is referred to as a snapshot – the ICU will configure and run a series of exposures in this state..

Planned Target (PT): The ICU will configure this type of exposure when the FOM informs the ICU, via the FONEXTOBSINFO record, that the spacecraft is performing a planned observation.

Safe Pointing (SP): The ICU will configure this when informed that the spacecraft is slewing to a safe pointing. This type of exposure occurs when no automated or planned targets are available.

South Atlantic Anomaly (SAA): In this configuration, the instrument is configured to be safe whilst passing through the SAA. In particular, the MCP bias voltage is held at 70% of its nominal value and the cathode voltage is at zero.

## Design Method

The design methodology is “object-based”, i.e. most modules in the system denote an object identified in the system (e.g. ‘real’ objects such as the detector, or ‘soft’ objects such as an on-board virtual CPU). The design aim was that objects are implemented as Ada packages (see next section), using the Tartan compiler (version 5.1). For convenience, it was sometimes appropriate to split the object into several e.g. the object representing the Blue Processing Electronics (BPE) was split into two, one representing the digital aspects, the other the analogue. When speed of execution is a requirement, the language used is 1750 Assembler.

Note: By object, we mean an entity that has

- a state ( i.e. a value or values)

- actions it suffers or can apply to other objects

There are also a number of packages that are of a ‘library’ nature - i.e. they provide constants, definitions and routines of a general nature with no specific ‘real-word’ object in mind.

## Ada Overview

### Basic Definitions

A full description of the Ada language is beyond the scope of this document, but the following summary of some Ada features may be useful in understanding the descriptions in the following sections.

Program Units - An Ada program is composed of one or more program units. Program Units consist of Subprograms, Task, Packages and Generic Units.

- Specifications identify the information visible to a client (i.e. the caller) of that program unit.

- Bodies contain the implementation details and will be hidden from the client.

Subprograms are either Procedures or Functions and express a sequential action. A function is the same as a procedure, except that its primary purpose is to return a calculated value. (A Main Program is a special case of a Subprogram that is called directly when the code starts running. It can also be regarded as a separately running task)

- Tasks defines an action that is executed in parallel with other tasks.

Packages are a collection of computational resources, encapsulating data types or instances thereof, subprograms, tasks or other packages.

All Ada packages have a similar two-part structure, consisting of a Specification and a Body. It is standard UVOT ICU practice to compile these separately.

Generic Units are templates for subprograms and packages and serve as the primary mechanism for building reusable software components. Note – for UVOT ICU, the only generic packages used were Tartan supplied.

### File Naming Conventions

| filename.Ada | - a file containing a 'main' program |
| --- | --- |
| filename.ads | - a file containing an Ada specification |
| filename.adb | - a file containing an Ada body |
| filename.asm | - a file containing MSSL written 31750 assembler code |
| filename.ASM | - a file containing Tartan supplied, MSSL modified, assembler code |

- The filename for a specification (.ads) is the same as the corresponding package body (.adb) name.

filename is always the name of the package specification and/or body or main program included in the file.

### Task Scheduling

At a given time during the execution of an Ada program, there are sets of tasks that are eligible for execution. The process of choosing a subset of tasks to actually run is called scheduling. The code uses the Tartan implementation of this process.

Scheduling has two parts:

- Scheduler

- Dispatcher

The scheduler is responsible for:

- adding a task to the set of executable tasks

- removing a task from the set of executable tasks

- selecting a task from the set to be the next task executed

The scheduler is implemented as a strictly priority ordered queue. The task with the highest priority is selected for execution. New tasks are inserted after tasks of equal priority within priority levels.

When the dispatcher is invoked, it causes the currently executing task to be suspended and replaced by the next task selected by the scheduler. If the current task is also the task selected to be the next by the scheduler, no change occurs.

The dispatcher is invoked whenever an Ada tasking or delay operation causes execution of the current task to be blocked, or when a high priority task becomes ready (for instance, by the expiration of a delay) and pre-empts a task with lower priority. This method of pre-emptive scheduling minimises the amount of time a high-priority task must wait for execution after a delay operation.

### Identifier Naming Conventions

In common with standard Ada coding practice, lower case letters indicate reserved words, whilst UPPER CASE letters indicate identifiers

.

BASIC CODE – PRINCIPAL PACKAGES

OPERATIONAL CODE - PRINCIPAL PACKAGES

## Software Components for Basic and Operational

### Object Mapping

The diagrams above illustrate the key control and data flows between the main software objects for both basic and operational code. The table below shows how the component files ‘map’ to those s/w objects. A brief explanation of each component is also given.

This table is valid for both basic and operational mode code provided, of course, that the named s/w object is present.

| Object | Consists of Filename(s) | Description |
| --- | --- | --- |
| 1553 | sc1553.ad[sb] | Routines to read/write packets to the interface |
|  | sc1553_ih.asm | Interrupt handler that counts the telecommands as they arrive. |
| 1PPS | pps_ih.ads, pps_ih.asm | 1 PPS interrupt handler for 1pps side A |
|  | pps2_ih.ads, pps2_ih.asm | 1 PPS interrupt handler for 1pps side B |
| BRIGHT OBJECTS | astro.ad[sb] | Calculates Sun, Moon and planetary positions |
| MANAGER | starcat.ad[sb] | Scans on-board catalogue for stars in field of view |
| COMMAND DISTRIBUTION | command_distributer.ad[sb] | Distributes all commands (either internally generated or external) to appropriate packages, |
|  | taskman.ad[sb] | Validates and distributes commands to the telescope module, watchdog and dcs.adb |
|  | detanalog.ad[sb] | Routines to control and monitor the analogue functions of the detector electronics. |
|  | detdigital.ads | Routines to control and monitor the digital functions of the detector electronics |
| DETECTOR | detector.ads | Provides single interface for all detector routines |
|  | detsafety.ad[sb] | Provides routines to control and monitor the detector safety circuit. |
|  | safetyport.ads | Format specifications for Safety Circuit ports |
| DPU MANAGER | dpu.ad[sb] | Sends commands and receives data from the DPU. |
| HK | hk.ad[sb] | Obtains HK items, constructs the HK packet and sends result to the TM queue. |
| ICB | icb.ad[sb] | Provides mutex controlled access to the ICB interface code in icb_driver. |
|  | icb_driver.ad[sb] | Provides the routines to perform I/O on the ICB interface |
| LIMIT CHECKING | limit.ad[sb] | Monitors data for limit exceptions, requests recovery action if detected. |
| MECHANISMS | mechanism.ad[sb] | Code to control the filter wheel and dichroic. |
|  | timer_a_ih.ad[sb] | Routines to handle timer A interrupts. |
|  | icu_mem_manager.ad[sb] | Routines to perform ICU memory load and dump |
| MEMORY MANAGER | mem_manager.ad[sb] | Validates memory management packet and calls appropriate routines |
|  | peek_poke.ads/asm | Examines addresses in various memory modes of ICU RAM. |
| OBSERVATION MANAGER | uvotman.ad[sb] | Controls all aspects of UVOT operation that involve state changes. |
|  | ieee.ad[sb] | Provides routines to convert 32 and 64 bit floating point data to Tartan format floats. These are present in packets sent by the spacecraft |
|  | reset.ads, reset.asm | Reloads the ICU oper code from EEPROM when in Safe OR in Basic, initiates switch to oper. |
| MODE MANAGER | modeman.ad[sb] | Manages state transitions (Basic Only) |
| RTS MANAGER | dcs.ad[sb] | Implements virtual machine on which RTSs run |
| SSI | ssi_ih.ads, ssi_ih.asm | Routine to handle SSI interrupts |
|  | ssi.ad[sb] | Routines to read/write packets to the interface |
| THERMAL | heater.ad[sb] | Routines to control heaters |
| TIME MANAGER | time_man.ad[sb] | Obtains Mission Elapsed Time, corrected factor spacecraft time and processes time-tone packets |
| TELEMETRY QUEUE | nhk.ad[sb] | Constructs non periodic HK packets and places them in telemetry queue |
|  | task_report | Procedure PUT constructs and places a Task Parameter Report packet in the telemetry queue |
|  | tcq.ads[sb] | Extracts packets and send then to command_distributor AND sends RT-to-RTs. |
|  | tcq_verify.asb | Verifies incoming packets |
|  | tmq.ad[sb] | Places a packet in the telemetry queue |
| TMPSU | tmpsu.ad[sb] | Routines for low level TMPSU control and monitoring |
| WATCHDOG | monitor.ads | Defines tasks to be monitored for aliveness by the watchdog |
|  | watchog.ad[sb] | Watchdog code |

### Definitions

The following table indicate which files are used for definitions only. They contain only specifications. There are no corresponding files containing a package body.

| Filename(s) | Description |
| --- | --- |
| types.ads | Defines additional Ada types |
| packet.ads | Defines the packet structure |
| importance.ads | Defines Ada task priorities |
| memloc.ads | Defines key memory locations |
| INTVEC.ASM | Linkage and Service Pointers for Interrupts |
| USERDEFS.ASM | Timer correction factor + Ada run time constants and masks |
| version.ads | S/W version number |

### Library Routines

The following table contain routines that do not map to a single object but are instead used by many. They are therefore classed as ‘libraries’.

| Filename(s) | Descriptions |
| --- | --- |
| algebraic_f_exception.ads | Defines exception code used by algebraic_functions |
| algebraic_functions.ad[sb] | Provides algebraic functions of general utility on floating point types, e.g. square, cube etc. |
| crc.ad[sb] | CRC calculation routines |
| checksum.ad[sb] | Packet checksum routines |
| debug.ad[sb] | Debugging utility routines |
| eeprom.ad[sb] | Routines to read and write to EEPROM |
| elementary_f_exception.ads | Defines exception code used by elementary_functions |
| elementary_functions.ad[sb] | Single-precision implementation of elementary functions, e.g. sqrt, log, cos, sin etc |
| float_rep.ads | Types and functions which are needed by primitive_functions and are dependent on the floating point hardware. |
| math_constants.ads | Defines mathematical constants such as Pi, e, etc |
| mutex.ad[sb] | Used to provide mutually exclusive access to various resources |
| primitive_functions.ad[sb] | Functions on floating point types whose implementation depends on access to the mantissa/exponent of the floating point number |
| support_functions.ad[sb] | a single-precision-only package of functions supporting elementary_functions. |

## Task Priorities

- Ada tasks are allocated to actions that are executed in parallel with other actions. In order to ensure that the behaviour of tasks is a deterministic as possible, their priorities (defined in the package IMPORTANCE) are allocated as follows:

| Task | Package | Basic | Oper. | Description |
| --- | --- | --- | --- | --- |
| DPU_DATA_MANAGER | dpu | 81 | 82 | Processes message from the DPU |
| FEED_1553 | 1553 | 130 | 130 | Sends telemetry to S/C. |
| HK_LIMIT_MONITORING | LIMIT | 89 | 94 | Checks HK for out-of-range conditions |
| HK_PROCESS | HK | 90 | 90 | Acquires Housekeeping. |
| HV_RAMP_TASK | detanalog | 96 | 96 | Ramps the HV voltages up or down. |
| ICU_MEM_MANAGER | ICU_MEM_MANAGER | 80 | 78 | Performs memory load and dumps. |
| LOAD_CENTROID_TABLE_TASK | detdigital | - | 79 | Loads the Blue Electronics centroid look-up table. |
| LOAD_WINDOW_TABLE_TASK_TYPE | detdigital | - | 80 | Loads the Blue Electronics window table. |
| RTS_TASK | DDC | - | 97 | Executes RTSs. |
| SSI_PUT | DPU | - | 195 | Sends command to the DPU. |
| STAR_CAT | STARCAT | - | 81 | Scans star catalogue. |
| TCQ | TCQ | 186 | 186 | Monitors the Tele-command stream. |
| THERMAL_CONTROL | heater | 92 | 92 | Monitors and controls the telescope module heaters. |
| TIMER_A | mechanism | 133 | 133 | Controls the mechanisms. |
| WATCHDOG | rbi | 190 | 190 | Controls the watchdog. |

## Interrupts

The interrupts for the ICU is as follows:-

| Number | Description |
| --- | --- |
| 0 | Power Down (cannot be masked or disabled)1 |
| 1 | Machine Error (cannot be disabled) |
| 2 | SSI Receive Interrupt |
| 3 | Floating Point Overflow |
| 4 | Fixed Point Overflow |
| 5 | Executive Call (cannot be masked or disabled) |
| 6 | Floating Point Underflow |
| 7 | Timer A4 |
| 8 | Spare |
| 9 | Timer B3 |
| 10 | Diagnostic Port |
| 11 | Spare |
| 12 | Input/Output Level 1 |
| 13 | 1553 Interrupt |
| 14 | Input/Output Level 2 |
| 15 | 1 PPS |

Notes.

- Interrupt Number 0 has the highest priority. Priority decreases with increasing interrupt number.

- All Interrupts are as per the 1750 standard except 2, 10, 13 and 15. These ‘spare’ interrupts that have been allocated as above for the ICU.

- Used by the Tartan Kernel to derive times for e.g. the delay facility

- Used by the ICU code to produce a series of pulses to control the speed of the mechanisms (filter wheel and dichroic).

(The following is a summary of section 8.5.5.4 of the Tartan Compilation System Manual). There are five configurable interrupt masks that control the behaviour of the runtimes with respect to interrupts. They are defined in the linker control file tlink17.lcf. The purpose of each mask is as follows:

| ART_MASK | Used when executive space runtime code is executing |
| --- | --- |
| ARTELAB_MASK | Used when executive space main program is being elaborated |
| ARTTASK_MASK | Used when executive main program or user tasks are executing |
| PREEMPTER_MASK | Blocks all interrupts that might cause task pre-emption |
| CONNECT_MASK | Determines if a task entry may be directly connected to a particular hardware interrupt |

For both basic and operational code, the interrupts are enabled as follows:

| MASK | Value | Interrupt (1 = ENABLED) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (hex) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ART_MASK | D5E4 | 1 | 1 | - | 1 | - | 1 | - | 1 | 1 | 1 | 1 | - | - | 1 | - | - |
| ARTELAB_MASK | DD40 | 1 | 1 | - | 1 | 1 | 1 | - | 1 | - | 1 | - | - | - | - | - | - |
| ARTTASK_MASK | DDE4 | 1 | 1 | - | 1 | 1 | 1 | - | 1 | 1 | 1 | 1 | - | - | - | - | - |
| PREEMPTOR_MASK | DC00 | 1 | 1 | - | 1 | 1 | 1 | - | - | - | - | - | - | - | - | - | - |
| CONNECT_MASK | 0100 | - | - | - | - | - | - | - | 1 | - | - | - | - | - | - | - | - |

## Ada Exception Handling and Debugging

### Overview

The package DEBUG provides a number of facilities that can be helpful in the event of an unexpected Ada exception or ‘hang’ of the code. In addition, a series of fixed memory locations (defined in package MEMLOC) are provided in which debugging information may be written. A combination of the two will usually indicate the problem area of the code.

### Exception Handling

In the event of an Ada exception

- a code of the form ‘Package Code’ + ‘Exception Code’ is written to the reserved location. This is done using the procedure EXCEPTION_REPORT in package DEBUG. The ‘Package Code’ indicates the package in which the exception occurred. The ‘Exception Type’ indicates which type of Ada exception occurred.

- A non-periodic housekeeping (NHK) engineering exception packet is issued containing 3 parameters. The first parameter contains the first ‘Package code + Exception Code’ to have occurred. The second parameter is of the same form but for the current exception. The third parameter is of the form ‘Package Code’ + ‘Code Region’. The ‘Package Code’ indicates which package was executing just prior to the exception. The ‘Code Region’ indicates within which region of package code the exception occurred. This parameter is contained within reserved location FIRST_PROGRESS of package DEBUG. Its contents are set up using calls to procedure PROGRESS in package DEBUG.

The Package Codes (defined in package DEBUG) are as follows:

| Package | Code |
| --- | --- |
| ICU | 0x1000 |
| LIMIT | 0x1100 |
| ICU_MEM_MANAGER | 0x1200 |
| MEM_MANAGER | 0x1300 |
| SC1553 | 0x1500 |
| WATCHDOG | 0x1600 |
| ICB | 0x1C00 |
| ICB_DRIVER | 0x1D00 |
| IEEE | 0x1E00 |
| TIME | 0x2100 |
| HK | 0x2300 |
| MODEMAN | 0x2400 |
| UVOTMAN | 0x2400 |
| NHK_PACKAGE | 0x2500 |
| TASK_REPORT | 0x2700 |
| TASKMAN | 0x2800 |
| TC_VERIFY | 0x2B00 |
| TCQ | 0x2C00 |
| TMPSU | 0x2E00 |
| TMQ | 0x2F00 |
| MECHANISM | 0x4000 |
| TIMER_A_IH | 0x4600 |
| MUTEX | 0x4400 |
| HEATER | 0x4500 |
| SSI | 0x5500 |
| SAFING | 0x5A00 |
| RTS_SCRIPT | 0x5C00 |
| DEBUG_TEXT | 0x6000 |
| TEXT_PACKETS | 0x6100 |
| ASTRO | 0x8500 |
| BOOT_DUMP | 0xBD00 |
| CRC | 0xC200 |
| CHCKSUM | 0xC500 |
| STARCAT | 0xCA00 |
| COMMAND_DISTRIBUTOR | 0xCD00 |
| DPU | 0xD000 |
| DETSAFETY | 0xD500 |
| DETANALOG | 0xDA00 |
| DEBUG | 0xDB00 |
| DCS | 0xDC00 |
| DETDIGITAL | 0xDD00 |
| TM_MAN | 0xE500 |
| EEPROM | 0xEE00 |

The exception types are as follows:

| Ada Exception | Exception Type |
| --- | --- |
| Constraint Error | 0 |
| Program Error | 1 |
| Storage Error | 2 |
| Tasking Error | 3 |
| Other | 4 |

### Reserved Locations

The package MEMLOC defines the following reserved locations for debugging purposes.

| Location Name and Description | Address |
| --- | --- |
| FIRST_PROGRESS_ADDR | 0x600 |
| LAST_PROGRESS_ADDR | 0x601 |
| FIRST_EXCEPTION_ADDR | 0x602 |
| LAST_EXCEPTION_ADDR | 0x603 |
| SSI_ERROR_COUNT_ADDR | 0x604 |
| BOOT_TYPE_ADDR | 0x605 |
| SSI_HEARTBEAT_COUNT_ADDR | 0x606 |
| SSI_INT_COUNT_ADDR | 0x607 |
| TIME_INT_COUNTER_ADDR | 0x608 |
| TIME_INT_COUNTER2_ADDR | 0x609 |
| SC1553_INT_COUNTER_ADDR | 0x60A |
| DEBUG_INT_CTR_ADDR | 0x60B |
| SSI_OVERFLOW_ADDR | 0x60C |
| SSI_HFF_ADDR | 0x60D |
| DEBUG_LED_MEM_LOC_ADDR | 0x60E |
| SC1553_INTERRUPT_ADDR | 0x60F |
| SC1553_IH_TC_COUNTER_ADDR | 0x610 |
| SC1553_ERROR_ADDR | 0x611 |
| TC_PACKET_COUNTER_ADDR | 0x612 |
| SC1553_OVERFLOW_IMMINENT_ADDR | 0x613 |
| SC1553_OVERFLOW_NEXT_ADDR | 0x614 |
| TC_GOOD_ADDR | 0x615 |
| TC_BAD_ADDR | 0x606 |
| TM_GOOD_ADDR | 0x617 |
| TM_BAD_ADDR | 0x618 |
| RT2RT_GOOD_ADDR | 0x619 |
| RT2RT_BAD_ADDR | 0x61A |
| SSI_GOOD_ADDR | 0x61B |
| SSI_BAD_ADDR | 0x61C |
| SSI_SENT_ADDR | 0x61D |
| PPS_SYNCHRONIZE_ME_ADDR | 0x61E |
| WATCHDOG_TASK_STATUSES_ADDR | 0x61F |
| TIME0_ADDR | 0x622 |
| TIME1_ADDR | 0x623 |
| SYNC_TIME0_ADDR | 0x624 |
| SYNC_TIME1_ADDR | 0x625 |
| DEBUG_F00C_ADDR | 0x626 |
| DEBUG_C0DD_ADDR | 0x627 |
| ICB_ERROR_COUNT_ADDR | 0x628 |
| FW_POSITION_ADDR | 0x629 |
| ACS_COUNTER_ADDR | 0x630 |
| TIMETONE_COUNTER_ADDR | 0x631 |
| TOP_LEVEL_RTS_ADDR | 0x632 |
| BOTTOM_LEVEL_RTS_ADDR | 0x633 |
| RTS_LINENO_ADDR | 0x634 |
| RTS_DEPTH_ADDR | 0x635 |
| CURRENT_EVENT_ADDR | 0x636 |
| TIMEOUT_COUNT_ADDR | 0x637 |
| DELAY_TIME_ADDR | 0x638 |
| TC_COUNTER_ADDR | 0x639 |
| RT_TO_RT_COUNTER_ADDR | 0x63A |
| TC_CODE_ADDR | 0x63B |
| COMMAND_CODE_ADDR | 0x63C |
| CURRENT_EXEC_ADDR | 0x63D |
| HK_PACKET_ADDR | 0x640 |

## ICU Delay Adjustment

The standard 31750-processor specification requires a timer B clock of 100 khz. However, for the UVOT-ICU a 93.75KHz clock is used. As this timer is used to control any requested delays, this will produce an error factor in the delay statements.

The Tartan run time library (mAdart.tlib) was modified as described below so that an appropriate correction is automatically applied to the handling of Timer B interrupts.

### Conversion from TICKS to DURATION'SMALL

The default conversion factor is 1.6384/2, which is

DATA	068DB,08B01,0AC71

in 1750 long floating point data format (see 1750 doc ...)

| 48-bit format |  |
| --- | --- |
| 0 | Sign bit (0 for positive, 1 for negative) |
| 1-23,32-47 | Mantissa (m) |
| 24-31 | Exponent (e) |

So the number is .m * 2^e

The 1750 and Tartan Compiler are expecting a tclock of 100 kHz. Swift has a tclock of 93.75 kHz (=6/2^6 MHz). So the conversion factor must be multiplied by 100/93.75

- Calculation

.8192/2*100/93.75 = .6FD9 1D84 BCDA * 2^1

so the conversion factor in hex is:

6FD9 1D01 84BD

so the DATA line in USERDEFS.asm should be

DATA 06FD9, 01D01, 084BD

### TICKS in one and two days

Ticks in one and two days should be measured in real hardware ticks not expected ticks.

The default value for ARTONEDAY in USEDEFS.asm is

DATA	0337F,09800

where 16#337f9800# = 10#864 000 000# using a tclock of 100kHz  (timer B 10kHz)

This must be divided by 100/93.75 if our tclock is a factor of 93.75/100 too slow to give 864_000_000 * 93.75/100 = 3047 9E80h so the DATA line in USERDEFS.asm should be

DATA 03047, 09E80

The default value for ticks in two days is

DATA 066FF, 03000

This must be divided by 1.6 if our tclock is a factor of 93.75/100 too slow to give 864_000_000 * 2 * 93.75/100 = 608F 3D00h so the DATA line in USERDEFS.asm should be

DATA 0608F, 03D00

# Detailed Component Description

## Introduction

This section contains:

- A description of the Bootstrap and Loadaer code.

- A set of sections, each describing the software components of the Basic and Operational code. These components are presented in alphabetical order.

Unless stated otherwise, all components are coded in Tartan Ada version 5.1.

## Bootstrap and Loader Code

### Bootstrap Code

- Sets up the interrupt vectors.

- Defines whether it is running as prime or redundant.

- Define the PROM version number.

- Copies bootstrap code  (which includes loader1 and loader 2) from PROM to RAM instruction and operand space.

- Looks for activity on the debug interface and accepts commands if available to, for example, load code.

- If no debug commands after 1 second, times out.

- It then proceeds to run Loader 1.

### Loader 1

- This reads, a block at a time, Ada Basic code from PROM into the appropriate location in RAM above the bootstrap.

- Each block contains details on the start address, whether it is instruction or operand space, the length of the data to be loaded, the data itself and a checksum of the block.

- The blocks are terminated with a checksum of all the blocks.

- If there is a checksum error, it leaves a message in memory, enables the watchdog, force a hang and waits for the watchdog to reboot.

- It then shuts off the PROM (which was enabled on power-on or reset)

### Loader 2

It loads or reloads the Ada Operational code. It is called (using the reset procedure in the reset package) when a request is issued to go to the Safe state

- from the Basic state

- when already in the Safe state.

- Its initial structure is similar to Loader 1, except that it reads from where the operational code is store i.e. EEPROM rather than PROM.

- It saves debug information into reserved areas of memory (which is extremely important in the event of a watchdog reboot of Ada exception.

- Zeroes the lower part of memory.

- Jumps to the start of the Ada code at 0x300.

## Package algebraic_f_exceptions

### Summary

Defines an exception code used by package algebraic_functions. It has no body.

## Package algebraic_functions

### Summary

Provides algebraic functions of general utility for floating point types e.g. square, cube etc.)

## Package astro

### Summary

This provides a set of routines to calculate Sun, Moon and planetary positions.

### Description

The package astro calculates the positions of the Sun, Moon, and planets) to approx 1 arc minute allowing for nutation. Calculations for asteroids were originally included but later removed due to memory constraints. There are also functions for angular distance, obliquity of the Ecliptic and coordinate conversions. The positions are calculated using formulae, algorithms and data given in

- U. S. Naval Observatory, The Astronomical Almanac, Government Printing Office, 2001.

- P. Kenneth Seidelmann (Editor), “The Explanatory Supplement to the Astronomical Almanac”, University Science Books, 1992.

- Peter Duffet-Smith, Practical Astronomy with Your Calculator - 3rd Edition, Cambridge University Press, Cambridge, 1979.

## Package chcksum

### Summary

This provides a suite of routines to calculate or verify packet checksums.

### Description

The checksum calculated is simply the modulo 65536 addition of each octet of the CCSDS packet header, secondary header and application data fields, but excluding the 2 octets of the checksum field, as required by the Swift 1553 Bus Protocol ICD (Document 1143-EI-S19121).

The procedure calc calculates the checksum required for internally generated outgoing commands. It also generates for telemetry packets. Although the above ICD does not require this, the resulting checksum word is added to the end of the telemetry data field. However, the checksum is only calculated for that application data field and does not include the header.

The procedure check validates the checksum for all incoming commands.

## Package command_distributor

### Summary

As its name implies, this package, in conjunction with the taskman package, examines all incoming commands, validates parameters and distributes them to the appropriate package.

### Description

All packets are acknowledged with a verification message except for those broadcast messages whose acknowledgement would cause a flood of telemetry or are not relevant to ICU operation. For example, the SISCATTITUDE and SITIMETONE records are only acknowledged by incrementing HK counters but the SISLEWWARNING and SISLEWABORT are acknowledged. Any error messages resulting from the rejection of a package will have been issued by the relevant package with the exception of those caused by an unrecognized APID or Function Code, which are issued by this package.

Note that this package handles both any incoming external commands and internally generated RTS commands.

The taskman package is used separately to check a block of commands (dealing largely with TM commands) for code heritage reasons. The two packages could have been merged if the effort had been considered worthwhile.

## Package crc

### Summary

This provides the suite of routines to calculate CRCs for various situations.

### Description

The underlying algorithm used is that of the heritage instrument XMM-OM. It is documented in Appendix B of the ESA packet Telecommand Standard ESA-PSS-04-107. It is implemented in the routine CLC.

Function CALC returns the unsigned 16 bit integer checksum of the first NUMBER locations in unsigned byte array DATA.

Function CALC_MEM is used to calculate a checksum for a large block of data on the assumption that not all the data will be available once. Therefore, it uses the CURRENT_CRC value returned by a prior call as input to the current call and then calculates the CRC of the NO_WORDS 16-bit words of data contained in MEM. The result is the CRC all blocks of data supplied (NOTE: the sequence is restarted by supplying a value of all binary ones for CURRENT_CRC).

Procedure CALC_EEPROM is used to calculate a checksum for a very large block of data stored in EEPROM B. It takes a pointer to the first word of data, and another to the last word of the data. It then reads the data one smaller block at a time and calls CALC_MEM for each smaller block, as above, using the CRC returned from one call as input to the next. Also, as this function may take a long time AND be called from an RTS, it optionally monitors the RTS Abort Flag to see if it should prematurely exit. If so, OK = False.

Function CALC_VARIABLE_TABLE is used to calculate a checksum for a very large block of data stored in EEPROM B in which the 1st word contains the overall length of the table, including the checksum - thus the result ought to be zero.

## Package dcs

### Summary

Implements the virtual machine to execute the RTS. It also manipulates the associated RTS index file in EEPROM.

### Description

declare_event: This procedure is used to clear or set event flags that control the progress of an RTS. When a request is encountered by an RTS to wait for a particular event, if it is not already set, the RTS will loop, polling the flag once every 0.2 second until it becomes set, or until a specified time-out is reached. Depending on the parameters associated with the event, the RTS then 1) proceeds to the next instruction 2) switches (‘chains’) to another RTS 3) exits 4) returns to the calling RTS if present or exits if not.

The event flags are as follows:

| Event Flag Name | Code | Description |
| --- | --- | --- |
| NORMAL_DELAY | 0 | Set when a delay instruction expires |
| AT_REQUESTED_HV | 1 | Set when an HV ramp terminates |
| AT_REQUESTED_FILTER_POSITION | 2 | Set when filter wheel move ends |
| DPU_MODE_READY | 3 | Set when a DPU Mode Ready message received |
| DPU_ACK | 4 | Set when a DPU Ack message is received |
| CENTROID_TABLE_LOADED | 5 | Set when a BPE centroid table load ends |
| WINDOW_TABLE_LOADED | 6 | Set when a BPE window table load completes |
| NEXTOBSINFO | 7 | Set when a NEXTOBSINFO record is received |
| WITHIN_10_ARCMIN | 8 | Set when the WITHIN_10ARCMIN flag is set. |
| SETTLED | 9 | Set when the settled flag is set |
| CHAIN_CODE | 10 | Dummy flag – used by chaining mechanism |
| SLEW_START | 11 | Set when a slew starts |

kill: This procedure is used to abort any RTS running. It sets an ‘abort’ flag which is polled by the RTS virtual CPU within 0.2 seconds. By providing a non-zero code number (and its address in EEPROM) for an RTS, that current RTS will be aborted and the new RTS run in its place. As there is the possibility of the abort flag being manipulated by other routines, the manipulation is performed in a critical region. Checks are also performed in case more than one ‘kill’ is in progress, in which case new RTS with the same or higher priority is used.

enable_trace: This routine in called by the /rtstrace command. If trace is enabled, the virtual CPU will issue a debugging NHK event message as it starts executing code corresponding to the start of a new line in the original RTS script. This message will trace the line number and RTS being run.

run_rts: This procedure first uses find_proc to see if the requested RTS exists and if so, what its address and priority is. If that RTS is already running or is of a higher priority, the requested is ignored. Otherwise, if an RTS is already running it is aborted using kill and run in its place. If there is no RTS already running, then the RTS code number is supplied when the virtual CPU is started with a call to the execute entry point to the RTS task.

The RTS task

The RTS tasks main job is to validate parameters before calling the execute_proc procedure and then to examine the information returned by this procedure to decide whether it should run another RTS. The execute_proc implements the virtual cpu. It also perform a failsafe check performed every second whilst the virtual CPU is waiting to be started. This is just to avoid a possible abort of the current RTS in favour of another RTS that might have been missed because the RTS was exiting just as the request came in. The additional poll will run that new RTS if it finds one.

execute_proc executes the RTS requested by the supplied RTS code. For code numbers less than 16 hex, these are in fact references to Ada code fragments (the so-called ‘hardwired-RTS’) and these are executed instead. All other code numbers are references to an RTS stored in EEPROM and these are executed on a virtual CPU that forms the main body of this procedure.

An RTS consists of a standard area of EEPROM containing a set of command words generated by a translator from a C-like human-readable language. These are best considered as analogous to assembler instructions encountered by a simple hardware CPU. The on-board code is a virtual CPU that, from within a simple loop, reads a command word from EEPROM and then calls a routine that performs the instruction corresponding to that word. It thus 'executes' the command word, analogous to a hardware CPU executing an assembler instruction. For example, if the command word means ‘push value onto stack’, then a code that performs the necessary instructions to do just that would be executed. The virtual CPU normally obtains the next instruction from the next location in EEPROM. However, branch instructions modify that value according to a true or false value found at the top of its internal stack.

Each of the routines or code fragments called is straightforward in what it does. This enabled them to be thoroughly tested prior to launch. It is intended that they will not require modification on orbit.

The design uses a stack to hold all temporary values. This is primarily to exploit the nested nature of the RTS language, for example, in allowing the passing of parameters to potentially deeply nested calls to RTSs. It also permits a simpler code design. All commands that produce a result place the values on the top of the stack and commands requiring results of prior operations always find them on the top of the same stack. In order to fully exploit this structure, there are ‘push value onto stack’ or ‘push number at a RAM location onto stack’ instructions available to the virtual CPU.

The flow diagram below illustrates the simple structure of the virtual CPU. For clarity, not all possible command codes are shown.

A summary of each of the command words now follows. In the code, the tests for which command word is being executed are performed in order of likely frequency of occurrence, with most probable first.

| Command Word Name | Value (hex) | Description |
| --- | --- | --- |
| NOOP_CODE | F000 | Code is read and discarded - i.e. Do Nothing operation |
| CMD_CODE | F001 | Treats next word in EEPROM as an index identifying a command whose description is stored in the command database, also stored in EEPROM. Using any values on stack as arguments to the command and the information stored in the database, it constructs the command and sends it to either command_distributor or, if it is an RT2RT, to tmq.put. The format of the database is described in the UVOT User Manual. |
| RTS_CODE | F002 | Treats next word in EEPROM as code number of RTS and sets up and performs recursive call to execute_proc to run that RTS using any items on stack as arguments to the RTS. May force exit of RTS if return code abnormal. |
| PUSH | F003 | Pushes the next word in EEPROM onto the stack |
| PUSHLONG | F004 | Pushes the longword located in next two words in EEPROM onto stack. The first word is treated as the most significant. |
| PUSHARG | F005 | Treats next word in EEPROM as an argument number (first one =1 ). Copies relevant argument from lower in the stack onto top of stack. |
| DELAY_CODE | F006 | Treats item on top of stack as delay time in units of 0.2 s. Sets up appropriate delay which virtual machine then obeys. It is implemented as a special WAITFOR with no recovery action on timeout. |
| WAITFOR_CODE | F007 | Sets up a waitfor event that the virtual CPU will then poll until the event happens or times out. The event flags are described above under declare_event. The value at the top of the stack is taken as the timeout value in units of 0.2 s. The next location in EEPROM is used to specify the event. The following EEPROM location, if set to 1 or greater, specifies that any recovery action will take place when the event times out. If set to zero, the recovery action will take place when the event occurs. The location after that indicates the recovery action to be taken: 0=noaction, 1=return without error,2=exit without error,3=autostate transition, 4=>11 exit with error code, 0x8000+RTS_CODE=chain to RTS with code RTS_CODE. The value from the next location in EEPROM |
| EXIT_CODE | F008 | Virtual machine shuts down RTS stack cleanly. |
| RETURN_CODE | F009 | RTS returns to calling RTS. If top-level RTS, virtual machine exits. |
| PUSHRAM | F00A | Treats next word in EEPROM as word address in RAM. Pushes contents of that address onto the stack. |
| PUSHRAM_LONG | F00B | Treats next word in EEPROM as longword address in RAM. Pushes contents of that address and the next onto the stack, assuming first word is the most significant. |
| EXEC_CODE | F00E | Uses next word in EEPROM to select which built-in Ada function to run, as follows: 0; Uvotman.check_catalog_sources.. 1; Uvotman.moving_bright_sources. 2; Uvotman.zero_accumulated_counts. 3; Uvotman.start_accumulate_counts. 4; Uvotman.end_accumulate_counts. 5; Uvotman.get_at_config. 6; Uvotman.get_pt_config. 7; Uvotman.get_sp_config. 8; Uvotman.update_flag. 9; Uvotman.inc_fc_attempts. 10; Uvotman.get_settling_config. 11; Uvotman.get_fc_config. 12; Uvotman.starting_exposure. 13; Uvotman.ending_exposure. 14; Uvotman.calculate_sun_moon_planets. 16; Detsafety.alert. Any other values result in the RTS exiting with a DCS_INVALID_EXEC_TOKEN return condition. |
| IF_CODE | F00F | If the next location in EEPROM is zero, then this is treated as a ‘JUMP IF FALSE’ command. If that location is greater than zero, then this instruction is treated as a ‘JUMP IF TRUE’ instruction. The next EEPROM location after gives the number of words to jump if the instruction is obeyed. As the number is treated as a signed integer it is possible to jump backwards, as required by conditional loops. The value at the top of the stack is the condition tested, with greater than zero considered as TRUE. |
| JUMP_CODE | F010 | Treats the next word in EEPROM as the number of words to jump. As the number is treated as a signed integer it is possible to jump backwards, as required by loops. |
| LINENO_CODE | F011 | Stores next word in EEPROM as current line number. Used in message reporting and issues message when command /irtstrace has enabled line trace. |
| EQ_CODE | F012 | Compares the top two items on the stack. After popping the stack twice, the result of the comparison (with greater than zero representing TRUE) is pushed onto the stack. |
| NOT_CODE | F013 | Replaces the item on the top of the stack with its logical NOT – greater than zero represents TRUE. |
| AND_CODE | F014 | Logically ANDs the top two items on the stack. After popping the stack twice, the result of the operation (with greater than zero representing TRUE) is pushed onto the stack. |
| OR_CODE | F015 | Logically ORs the top two items on the stack. After popping the stack twice, the result of the operation (with greater than zero representing TRUE) is pushed onto the stack. |
| GT_CODE | F016 | Performs a ‘Greater Than’ comparison of the 2nd item on the stack with the 1st, i.e. that item on the top (note the order). After popping the stack twice, the result of the operation (with greater than zero representing TRUE) is pushed onto the stack. |
| LT_CODE | F017 | Performs a ‘Less Than’ comparison of the 2nd item on the stack with the 1st, i.e. that item on the top (note the order). After popping the stack twice, the result of the operation (with greater than zero representing TRUE) is pushed onto the stack. |
| POKE_CODE | F018 | Pops the value off the top of the stack and stores it in RAM at the word address given by the next location in EEPROM. Note that it only supports word, not longword, operations. |

find_proc: This procedure scans the separate RTS index stored in EEPROM to determine if it the requested RTS exists and if so, returns its priority and address.

peek: There are two versions of this routine, one for EEPROM B and one for RAM.  The virtual CPU uses them internally. They simply return the datum at the address specified.

inc_stackp: Used internally by the virtual CPU to increases its own stack pointer and check for stack overflow.

find_dbx: This procedure is used internally by the virtual CPU. One of the latter’s main jobs is to construct and send internally an ICU commands. The routine finds the location in EEPROM of the relevant database entry. The format of that database is described in UVOT Users Manual.

autoread: Used internally by the virtual CPU. It simply obtains a datum for the requested RAM address, then increments that address by one.

Summary of RTS Scripts -

This list is for release 9.

| RTS Name | Code | Priority | Description |
| --- | --- | --- | --- |
| autostate | 19 | 1000 | Requests ICU to autonomously decide and switch to the next state. |
| blocked_failsafe | 1283 | 1000 | Moves f/w to blocked position, transitions to Idle or SAA state (if s/c present in SAA), with HVs set accordingly. |
| blocked_tdrss | 1284 | 1000 | Same as ‘blocked_failsafe’ but also issues TDRSS message. |
| bright_planets | 1007 | 0 | Not used. |
| change_state | 513 | 0 | ‘Pokes’ locations in ICU Ram with state code and issues NHK message to say State Change has taken place. |
| controlled_off | 257 | 32765 | Contingency RTS. Goes to Safe state then requests the S/C to turn off UVOT. |
| controlled_tmoff | 258 | 32764 | Contingency RTS. Goes to Safe state then requests the S/C to turn off UVOT Telescope Module only. |
| diagnostic | 1280 | 0 | Diagnostic RTS. Sends NHK event message with supplied 3 parameters inserted into it. |
| do_exp | 48 | 0 | Starts exposure (using ‘start_exp’), waits for exposure length (plus DPU ‘slack’) then ends exposure (using ‘end_exp’). |
| do_fc_exp | 1012 | 1000 | Performs finding chart exposure. If allowed to go to completion, flags that the finding exposure has been achieved. |
| do_pt_exp | 1014 | 1000 | Performs Planned Target exposures, including Null or Idle type exposures. |
| do_slew | 770 | 1000 | Runs during slew. Allows any centroid table load to complete. Switches to appropriate exposure at start of settling/settled period. |
| do_sttlng_exp | 1011 | 1000 | Performs settling exposure, switches to finding chart exposure when settled flag goes ‘true’. |
| emergency_hvoff | 18 | 32766 | Contingency RTS. Sets all HVs to zero without controlled ramp-down. |
| end_exp | 50 | 0 | Stops BPE events, failsafe turn-off flood LEDs, awaits completion of any centroid table load, signals ICU to stop counting events. |
| fastsafe | 769 | 32764 | Goes to state Safe quickly (F/w to blocked, Vcathode set to zero, other HVs ramped down in 5 seconds each). |
| fc_to_at | 837 | 1000 | Called at end of finding chart exposure. Performs multiple AT exposures until next slew. |
| get_at_config | 997 | 0 | Not used. |
| get_fc_config | 996 | 0 | Not used. |
| get_pt_config | 998 | 0 | Not used. |
| get_sp_config | 999 | 0 | Not used. |
| get_sttling_config | 995 | 0 | Not used |
| gotobasic | 768 | 32763 | HVs set down quickly (Vcathode set to zero, other HVs ramped down in 5 seconds each). Goes to Basic state/ |
| gotosaa | 776 | 1000 | Performs transition to SAA state. Any exposure is tidily interrupted and HVs set appropriately. No F/W movement performed. |
| gotosafe | 17 | 32763 | Performs nominal transition to Safe state from all states in operational code. |
| gotosafe_tdrss | 1537 | 32763 | Same as gotosafe except that a DCS Timeout message is also sent to TDRSS as it is called after timeout on waitfor instruction. |
| h_11 | 11 | 32767 | Placeholder – not used. |
| h_12 | 12 | 32767 | Placeholder – not used. |
| h_13 | 13 | 32767 | Placeholder – not used. |
| h_14 | 14 | 32767 | Placeholder – not used. |
| h_15 | 15 | 32767 | Placeholder – not used. |
| h_basic | 2 | 32767 | Implemented in Ada rather than RTS. Causes transition to Basic state. |
| h_emergency_safe | 3 | 32767 | Implemented in Ada rather than RTS. Performs emergency fast transition to Safe state (HVs slugged to zero, F/W to datum |
| h_tm_off | 4 | 32767 | Implemented in Ada rather than RTS. Requests s/c to turn off UVOT Telescope Module. |
| h_uvot_off | 5 | 32767 | Implemented in Ada rather than RTS. Requests s/c to turn off UVOT. |
| hv231 | 1033 | 101 | Ramps up Vmcp23 and Vmcp1 to nominal. |
| hvcathode | 1032 | 101 | Ramps up Vcathode to nominal. |
| hvidle_or_saa | 1031 | 101 | Ramps down Vcathode to zero then, if we are in the SAA, ramps down Vmcp23 and Vmcp1 to SAA levels. |
| hvnominal | 1028 | 101 | Ramps up Vmcp23, Vmcp1and Vcathode to nominal. |
| hvoff | 1027 | 101 | Ramps down Vcathode, Vmcp1 and Vmcp23 to zero. |
| hvofffast | 1030 | 101 | HVs ramped down quickly (Vcathode set to zero, other HVs ramped down in 5 seconds each). |
| hvon | 1026 | 100 | Not used. |
| hvsaa | 1029 | 101 | Ramps HVs down to SAA levels. |
| idpumode | 33 | 1 | Sends DPU mode command and performs verification as described in the ICD for ICU/DPU protocol. |
| interrupt_exp | 51 | 0 | Interrupts current exposure – send stop command to DPU, stops events to DPU, tells ICU to stop counts received calculation. |
| line | 1 | 0 | Diagnostic – not used. |
| noaction | 0 | 0 | The RTS equivalent of a NOOP command – does nothing. |
| notallowed | 32766 | 0 | Reserved for ICU internal use. |
| obs_to_idle | 1018 | 1000 | Performs transition to Idle state from AT, PT, Finding, Settling and Safe Pointing states. |
| process_slew_warn | 20 | 32768 | Processes SISLEWWARNING – sends SISLEWWARNING after Vcathode =0 , then optionally calls do_slew. |
| pt_to_pt | 870 | 1000 | Performs multiple PT exposures until next slew. |
| retry_fw | 1282 | 0 | In the event of a F/W movement failure, attempts recovery by going to datum then trying again or goes to Safe state if fails again. |
| saa_to_idle | 906 | 1000 | Performs SAA to Idle state transition. |
| safe_to_idle | 794 | 1001 | Performs Safe to Idle state transition – note, may stop in SAA state if spacecraft is in the SAA. |
| safep_to_safep | 887 | 1000 | Not used. |
| send_slewsafe | 994 | 0 | Sends SACSLEWSAFEREPLY to spacecraft. |
| settling_to_fc | 820 | 1000 | Performs Settling to Finding Chart state transition. |
| slew_to_at | 805 | 1000 | Performs Slew to AT state transition. |
| slew_to_fc | 804 | 1000 | Performs Slew to Finding chart state transition. |
| slew_to_idle | 810 | 1000 | Performs Slew to Idle state transition. |
| slew_to_pt | 806 | 1000 | Performs Slew to PT state transition. |
| slew_to_safep | 807 | 1000 | Performs Slew to Safe Pointing state transition. |
| slew_to_settling | 803 | 1000 | Performs Slew to Settling state transition. |
| slewabort | 21 | 1000 | Processes SISLEWABORT command. |
| start_exp | 49 | 0 | Starts and exposure (sets up BPU, commands DPU, starts events, tells ICU to start counting events) |
| test | 32 | 0 | Not used. |
| testmode | 1281 | 0 | Not used. |
| undefined | 32767 | 0 | Not used |

RTS Statements

A RTS may contain any of the following statements:

Any UVOT telecommand: The arguments supplied may be constants, references to the contents of standard memory locations or arguments given to the calling RTS. A secondary table, directly derived from the ITOS database, contains the formatting information for constructing the command on-board.

An RT-to-RT telecommand: These are system-to-system commands, for example the UVOT, after receiving a request to slew, acknowledges to the spacecraft that it is ready to slew using the SACSLEWSAFEREPLY message.

A call to another RTS: This is analogous to a subroutine call. The arguments supplied may be constants, references to the contents of standard memory locations or arguments given to the calling RTS.

A delay statement: This allows a RTS to delay its next action for a number of clock-ticks, where a clock-tick is 0.2 seconds. The delay may be a constant, a reference to the contents of standard memory locations or an argument given to the calling RTS.

A wait for event statement: This causes the RTS to wait for a specified number of seconds – the timeout – for a significant event to happen. An example of such an event is the successful completion of a filter wheel rotation to a commanded filter position. Should that event fail to occur by the end of the timeout, the RTS could, for example, shut down and replace itself with another RTS using the chain statement (see below).

A chain statement: This is effectively an unconditional ‘wait for event’ with zero as the value for the timeout. It is used to shut down the current RTS and run a complete replacement.

Flow control statements: The “if … then … else”, “repeat … until” and “while” constructs are supported. Blocks of statements are conditionally executed depending on the values of certain Boolean flags maintained by the underlying Ada code. For example, it is possible to respond differently if the instrument is in the SAA or when the ‘wait for event’ statement described above exceeds its time limit. The flow control statement also supports Boolean (for example ‘and’, ‘or’) and relational (for example ‘greater than’, ‘less than’) operators.

Internal calls: These are commands to procedures supplied by the underlying Ada code. They therefore act as calls to built-in functions. An example of this is a call to the procedure that loads the selected exposure configuration into RAM from EEPROM.

Return: In the case of a top-level RTS, this causes it to stop executing. For a lower-level RTS, it causes it to return control to the calling RTS.

Exit: The current RTS, and any RTSs calling it, stops executing.

Messages: A facility similar in concept to the Unix echo command is provided. This allows appropriate diagnostic or error messages to be sent to the ground.

Symbolic Constants and Comments: These are provided to make the scripts more human-readable.

The RTS Converter

The above RTS statements must be converted to command words understood by an on-board command interpreter. This is performed on the ground with a utility generated using the GNU tools flex and bison. Flex generates the lexical analyzer (or ‘lexer’) layer while bison generates the parser layer of the code. The lexical analyzer converts a character stream into a syntactically meaningful string of text tokens. For example, the stream of characters “if true then” is analyzed and split up into the meaningful keywords “if”, “true” and “then”. These are then sent to the parser which recognizes the combination of those keywords in that order to be the start of an “if … then else” construct. It then generates the appropriate branching instructions. The instructions are written into a file in a format suitable for uploading into EEPROM via ITOS. This type of file is referred to as an “image” file.

It was found convenient to use this generator – referred to as dcsgen – to produce all EEPROM loadable tables. This was for two reasons. First, even for those tables that made no reference to a RTS, the lexical and parsing capabilities described above, together with the symbolic constants and comments, were useful in checking and generating the loadable files from a human readable text source. Secondly, those tables referred to RTSs require knowledge of their code numbers, which are only readily available within the converter.

The above process is summarized in the diagram. A detailed description of flex and bison on the web,

## Package debug

### Summary

This provides a suite of routines used for debugging and error handling. See “Ada Exception Handling and Debugging”, section 2.8.

## Package detanalog

### Summary

This provides a suite of routines to command and monitor the analogue side of the detector electronics.

### Description

Set_fine_position_sensor_current controls the current flowing in illuminating LED of the filter wheel fine sensor. Values higher than the maximum value of 15 are set to 15. However, this value is not used until fine_sensor is called.

Fine_sensor enables or disables the fine sensor illuminating LED. If the argument is TRUE , i.e enable, the value supplied by Set_fine_position_sensor_current is used.

Fine_sensor_current returns the current flowing in the illuminating LED of the filter wheel fine sensor as defined by Set_fine_position_sensor_current. It is used to return the values for housekeeping purposes.

Fine_position_sensed is simply a pointer, for coding convenience, of the identically named routine in package timer_a_ih.

Set_flood_led_bias_current controls the current flowing in flood LEDs. For detector safety reasons, it is rejected if it is called with the filter wheel at any other position than blocked.

In order that the detector may be calibrated in flight, four flood-LED’s are provided. They are located off-axis close to the detector. They are positioned so that their focused emission falls on the side of the filter facing the detector. The blank filter is used which then acts as a defocused ‘screen’ providing the flat field. They are green LED’s but with emission in the UV range.

Their intensity is controlled via ICB commands routed from the Blue Detector analogue control card to a 4-bit port. There are thus 16 possible levels. They are driven in such a way that if one should fail the remaining LED’s will remain fully functional. Values higher than the maximum value of 15 are set to 15 by the software.

Flood_led_bias_current returns the current flowing in the flood LEDs as defined by Set_flood_led_bias_current. It is used to return the value for housekeeping purposes.

Set_hv_enable enables or disables the HV control circuit. As this bit shares a port with the Vmcp1 value control, the code ensures that the value is zero when being enabled, in order to avoid the possible detector damage consequence of that value being set instantly to a high value.

Hv_enabled returns TRUE if the HV control circuit is enabled.

Set_hv sets a particular high voltage instantly to the specified value. In practice, this must be used directly with care to avoid damage to the detector. It is usually only used to set a value to zero (or internally by hv_ramp which performs a controlled ramp). As the Vmcp23 and Vcathode values share the same control port, the code must take care to resend the shared value as it was commanded the last time. Similarly, as the enable HV bit shares a port with the Vmcp1 value control, that value must be carefully repeated each time the Vmcp1 voltage is commanded.

Hv_ramp starts a controlled ramp from the current high voltage value to the specified value. It does this by calling the Start entry point for the perform_ramp task (or returns a busy error conditions if that task is not immediately waiting at that point).

Perform_task performs the HV ramp. The code is structured as a simple state machine (see diagram). The purpose of each state is as follows.

STOPPED

In this state it awaits a call to the START entry point. Parameters are verified using validate_hv_params. If correct, it proceeds to the STARTING state.

STARTING

The current value of the voltage to be commanded is determined using get_converted_hv.  Similarly the value of any voltage that shares that port is read back. It determines for both whether the value is the same as the last value commanded, within an EEPROM tabulated noise tolerance. If outside that tolerance, the value read back is adopted as the new value.  Otherwise the last commanded value is used. This ensures that correct values for all HVs involved are used.

It checks that the values commanded are consistent with any specific ramp direction specified (proceeding to the STOPPING state if it is not). It determines the increment or decrement of the HV to be performed each step of the ramp, based on EEPROM tabulated values. If the read back current voltage is within the noise tolerance of the requested voltage, it also proceeds to STOPPING. It will also go to that state if we are attempting to ramp up any HV with the filter wheel not in the blocked position.

Otherwise, it proceeds to the COMMANDING state.

COMMANDING

It increments or decrements the HV value being commanded by one step. If that value exceeds the anticipated voltage range of the ramp, the ramp is deemed to have finished and the value set to the target voltage. The HV value is then commanded (along with any shared HV values determined in the STARTING state).  It then proceeds to the POLLING state.

POLLING

In this state, the voltage being ramped is monitored every 1 seconds to see if it has reached the value commanded for the current step, to within a rather wide EEPROM tabulated tolerance (typically of the order of 100 volts). This continues for up to 10 seconds when it is deemed to have failed and it goes to the STOPPING state. If it has reached the commanded voltage, it returns to the COMMANDED state unless that commanded voltage was the target voltage, in which case it proceeds to the STOPPING state.

STOPPING

This state issues any event or error message required indicating success or failure. It then proceeds to the STOPPED state where it awaits the next ramp command.

Note – it is also possible to proceed to the STOPPING state from any state should the STOP_FLAG be set by hv_ramp_stop, as it is polled in all states.

Hv_ramp_stop aborts the ramp being performed by hv_ramp by setting the STOP_FLAG to TRUE.

Validate_hv_params is used internally by hv_ramp to validate the supplied parameters. If deemed incorrect, the ramp does not take place. It simply checks if the voltage requested is in a valid range.

Safe_to_slew returns whether it is OK to send the /SIACSLEWSAFEREPLY message. It is no longer used, this decision is now performed by an RTS.

Convert_hv_to_bits is used internally by hv_ramp to convert a particular HV value to the required bit pattern to send to the ICB interface. It uses EEPROM tabulated coefficients to perform the conversion.

Get_converted_hv obtains the monitored HV level and converts it from the digitized value to volts. It uses EEPROM tabulated coefficients to do this. It is used internally by hv_ramp as this works in units of volts.

Get initiates an analogue to digital conversion, waits for it to complete and returns the value. This is used internally by hv_ramp and for limit checking and HK purposes.  As this involves two ICB ports (the first to initiate the A-to-D on a particular channel, the second to read the value on that channel, a mutex is set up prior to using the first port and only removed after the use of the second port. This is to avoid a ‘race’ condition between two different tasks (the HK and limit checking task for example) in which the channel being polled by one task is changed by the other task. This is in addition to the mutex applied internally in the icb package, which is called to perform the interrogation of the ICB ports.

## Package detdigital

### Summary

This provides a suite of routines to command and monitor the digital side of the detector electronics.

### Description

Set_table_boundaries specifies the table boundaries that will be used to generate the centroid table into the detector RAM.

Load_centroid_table starts or stops the generation and loading of the centroid table into detector RAM. Should a load be already in progress, it returns a ‘Busy’ condition.

Set_window_description specifies the window parameters that will be used to generate and load the window bitmap table into detector RAM.

Table_address and Table_data are used internally by the centroid table-loading task to generate the address and data required.

Load_centroid_table_task loads the centroid table using the algorithm described in “Software Setup of the Blue Detector Electronics”, XMM-OM/MSSL/SP/0077.

Load_window_table starts or stops the generation and loading of the window bitmap table into detector RAM. Should a load be already in progress, it returns a ‘Busy’ condition.

Map_address is used internally by the window table-loading task to generate the addresses required.

Load_window_table_task loads the window table using the algorithm described in “Software Setup of the Blue Detector Electronics”, XMM-OM/MSSL/SP/0077.

Mic_out: Both the centroid table and window table-loading tasks perform the ICB accesses required using Mic_out internally.  Both processes involve both a writing followed by a reading and verification stage and this code distinguishes the latter operations, allowing the higher level code to simply perform two passes.

Set_acquisition_mode does what its name suggests for the detector electronics!

Set_event_threshold set the value for which events greater than or equal to this will be accepted by the detector.

Get_event_threshold returns the value supplied by an earlier call to set_event_threshold. This is used for housekeeping purposes.

Disable_frame_tags controls the insertion or deletion of frame tag words into the data stream sent to the DPU – note the inversion of the usual logic i.e. TRUE = do NOT insert flags.

Reset_Camera_head_electronics does what its name suggests.

Camera_running returns TRUE if the camera is started. If FALSE (=’standby’) it is possible to load the window bitmap table. It is used for HK purposes.

Status returns the status word of the detector electronics for HK purposes.

## Package detector

### Summary

This is simply a ‘wrapper’ specification-only package that, for programming convenience, combines the specifications of the detanalog and detdigital packages.

## Package detsafety

### Summary

This provides a suite of routines to command and monitor the safety circuit of the detector electronics.

### Description

Configure is used to command the safety circuit. This includes the threshold values as well as the control bits. However, as a safety precaution, the following checks are performed if the circuit has been previously tripped. The UVOT must be in the Safe state, not performing a state transition, not running a RTS and the HVs must be disabled. If the checks are successful, the command is obeyed. Otherwise the command is refused and an error message issued.

Monitor is used to return the status of the circuit for HK. Note that as the information is multiplexed on the ICB, several reads of the relevant ICB sub-addresses must be performed until all the information is acquired.

Alert determines whether the safety circuit has fired. Note that as the safety circuit status information is multiplexed, more than one read of the relevant sub-addresses may be required before the necessary information is acquired. Not also that the alert flag is only considered valid when the flags indicating the enabling of both the cathode control and safety circuit are true.

Reset uses configure to command a full reset of the safety circuit. The sequence of 3 commands used is derived from the ITOS sequence of commands tested on the spacecraft. In particular, it handles the requirement that the OPRST bit (which initiates a reset when set to True) should be returned to false subsequently to allow for proper operation of the circuit. Being based on the script, it also only changes one item at a time.

Disable uses configure to command a disable of the safety circuit. The sequence of 3 commands used is derived from the ITOS sequence of commands tested on the spacecraft. . In particular, it handles the requirement that the OPRST bit (which initiates a reset when set to True) should be returned to false subsequently to allow for proper operation of the circuit.

## Package dpu

### Summary

This package forwards DPU commands sent externally, or generated internally, to the DPU. It also receives messages from the DPU and forwards or processes them appropriately.

### Description

The function heartbeats returns a count of the heartbeat records received (wrapped around at 65536) since turn-on. This count is maintained by the DPU_DATA_MANAGER task

Status returns the status and housekeeping words contained in the DPU heartbeat. It is used for HK and Limit checking purposes. Their contents are defined in the ICD for the ICU-DPU protocol document.

Ssi_feed - This task feeds commands to the DPU via the SSI interface.

Commands to be sent are input into a small FIFO queue of 5 entries by the REQUEST function. This queue is polled every 0.5 second to see if any commands are present. When detected, the first command is extracted and the task attempts to send it to the DPU via the SSI interface. It then follows the lower levels of the ICU-DPU protocol handshake.

For all commands, if a NAK is received, it will resend the command once more. If that is also rejected with a NAK, an error message is sent and it proceeds to the next command in the queue. Otherwise, the dpu_ack semaphore is set to allow any waiting RTS to proceed.

For all non-Mode commands, if no acknowledgement is received after one second, it will resend the command once more. If that is also times-out after one second, an error message is sent and the dpu_ack semaphore remains unset. This ensures that any waiting RTS will timeout and proceed with its coded error recovery.

For Mode commands, if no ACK or NAK response is received within one second, no attempt is made to send it a second time. Instead, an error message is sent and the dpu_ack semaphore again remains unset. The error recovery for this condition (which is to send a DPU stop command) is then handled by the controlling RTS. This is to allow for the error condition where, in fact, the DPU had successfully processed the Mode command but failed to respond in time. It ensures that the ICU and DPU remain synchronized. If an acknowledgement is received within one second, the dpu_ack semaphore is set to allow the waiting RTS to proceed and it then handles the remainder of the ICU-DPU protocol handshake.

The SSI_FEED task then proceeds to the next command in the queue.

Note that as the DPU_data_manager task is the one that receives the ACK or NAK messages, SSI_FEED determines whether they have been received by performing a rendezvous - with a one second timeout - with that task.

Start_ssi_feed starts the ssi_feed task. This feeds commands to the DPU via the SSI interface. It is called once from the icu.ada main program.

Start starts the DPU data record monitoring task DPU_DATA_MANAGER.

Stop stops the DPU data record monitoring task DPU_DATA_MANAGER.

DPU_Data_Manager

This is implemented as a simple state machine – see diagram. The code starts in the INITIAL state. The diagram implies that if stop is called before the task is started (by a internal call to start from icu.ada), it will enter the SUSPENDED state and will then be returned to the INITIAL state by the internal start call. This facility is a copy of one in the basic code that provides a mechanism to prevent this task running. This provides a workaround should some bug be discovered at a later date in that PROM based unchangeable code. Note, however, that if these two events happen within the same state change request poll interval, it is possible for the internal start command not to be seen. It will then require two external (i..e  tele-command) calls to Start to proceed to the STARTED state.

In the STOPPED, INITIAL and SUSPENDED states, a poll is performed every second to see if a command requesting a state change has been received.

In the STARTED state, the SSI interface is polled every 0.1 s (using the SSI package) to see whether an error has occurred or if a DPU packet has arrived.

If an error has occurred, it increments the bad SSI counter and then issues a command to reset the SSI interface, waits one second, then re-commences polling the interface.

If a DPU packet has arrived, it is first echoed to the ICUs telemetry stream.  It is then checked for internal consistency – the APID and message ID should both be within an expected range and the message ID should be consistent with the APID (see ICU-DPU protocol document). If they are not, the packet is rejected and an error message issued. If these checks are passed, the action that follows depends on the type of the DPU packet.

HEARTBEAT: the heartbeat counter is incremented and will be used in the HK. The time since the last heartbeat is calculated for later use by heartbeat_inc. The HK data is extracted from the record for later use by status.

ACK or NAK: This causes the task to rendezvous with the SSI_FEED task in order to supply the response that task is expecting to a command sent. Should it not be expecting it, an error message is issued.

MODE_READY: The task sets the DPU_MODE_READY semaphore for use by any waiting RTS.

CHANNEL_BOUNDARIES:  The boundary values are extracted, checked for validity and sent to the DETDIGITAL package for loading. If they are invalid, an error message is generated and the CENTROID_TABLE_LOADED semaphore set to prevent any waiting RTS from ‘hanging’.

Heartbeat_inc returns the time in seconds since the last heartbeat. It is used by the limit checking code to detect the absence of heartbeats.

Valid_bit_set returns a non-zero value if any of the HK supplied in the heartbeat has their invalid bit set.

Monitor_active returns TRUE if the DPU_DATA_MANAGER task is in a STARTED state and is used in housekeeping.

Request places commands to be sent into the queue used by SSI_FEED. If the source of the command was an RTS, it also rendezvous with that task to ensure the command is actually sent before returning. This avoids an RTS race condition in which it incorrectly thinks a command has not been obeyed. If the queue is full, it issues an error message and sets the DCS semaphore DPU_ACK to false to force an error in any controlling RTS.

## Package eeprom

### Summary

This provides a suite of routines to read and write to both the EEPROM areas A and B.

### Description

It reads and writes to/from EEPROM a block at a time.

code_write writes a block to EEPROM-A.

code_read reads a block from EEPROM-A.

starcat_write writes a block to EEPROM-B.

starcat_read reads a block from EEPROM-B.

It has to change wait states before writing to EEPROM and then change back after.

Interrupts are turned off during the write to maintain the required speed.

All operations are mutex protected.

## Package elementary_f_exceptions

### Summary

Defines an exception code used by elementary_functions. It has no body.

## Package elementary_functions

### Summary

Single precision implementation of elementary functions e.g. sqrt, log, cos, sin etc.

## Package float_rep

### Summary

This package contains definitions of types and specification and body combinations that are needed by primitive_functions. They are dependent on the floating-point hardware.

## Package heater

### Summary

This package controls the telescope module heaters.

### Heaters and Thermistors

There are 8 thermistors named and located as follows:

| Name | Channel | Location |
| --- | --- | --- |
| BPE | 0 | Blue Processing Electronics |
| Ref B | 1 | Interface Flange |
| Ref C | 2 | Interface Flange |
| Main | 3 | Near Main Interface Heater |
| Forward 1 | 4 | Near Forward Heater |
| Forward 2 | 5 | Near Forward Heater |
| CCD | 6 | CCD |
| Ref A | 7 | Interface Flange |

The four instrument heaters, and their function, are summarized as follows:

| Main Interface Heater (HTR1) | This is located close to the interface flange on the telescope tube, and is intended to control the temperature at the interface bolts to 19.5  0.5°C using a closed loop algorithm. It has a control thermistor (Main) located close to it and there are 3 monitoring thermistors (Ref A, Ref B and Ref C) on the interface flange. |
| --- | --- |
| Forward Heater (HTR2) | This is located at the forward of the telescope tube and is intended to control that area and hence the whole telescope tube to about the same temperature as the main interface heater - the default setting are 19.5  1.5 °C using a closed loop algorithm. This should ensure that all of the telescope optical elements are sensibly isothermal. It is controlled by one of two thermistors (Forward 1 or Forward 2) mounted close to it. |
| Metering Rod Heaters (HTR3) | These are a set of three parallel heaters, one mounted on each of the metering rods. These are used to extend the distance between the primary and secondary mirror by a small amount if necessary. This is done using an open loop algorithm that defines an on-off ratio. The power developed is a function of the square of the current spacecraft voltage. Therefore the on-off ratio will decrease as the spacecraft voltage increases. This algorithm is disabled by default |
| Secondary Mirror Mount Heater (HTR4) | This heater is used to shorten the separation of the primary and secondary mirror. NOTE that therefore this heater and the metering rod heaters will not be powered at the same time during normal operation. This is done using an open loop algorithm that defines an on-off ratio. The power developed is a function of square of the current spacecraft voltage. Therefore the on-off ratio will decrease as the spacecraft voltage increases. This algorithm is disabled by default. |

### Description

The heater algorithm is implemented as a simple state machine in the task HEATER_CONTROL – see diagram.

STOPPED

In this state, it simply monitors every 0.1 s for incoming state change commands such as might be issued by the start and stop procedures to transition to either the STARTING or STOPPING state.

STARTING

In this state, it simply initializes the algorithm. This involves setting the initial state of the heaters to all off, noting whether any control of the focusing heaters is required and setting up the initial scaling factors based on the current spacecraft voltage. It zeroes the clock that will be used as the algorithm proceeds (see STARTED). It then transitions to the STARTED state.

STARTED

If either we are due to turn on or off a focus heater, or polling the thermistors (an action performed every 10 seconds) that monitor the other heaters within the next 0.1 s, then the action closest in time is performed. Otherwise it reviews the situation again after 0.1 s. If a focusing heater is commanded, the spacecraft voltage is re-sampled and the interval to the next operation re-calculated. If the thermistors are polled, then the heaters are commanded appropriately i.e. if too hot, then the relevant heater is disabled, too cold and it is re-enabled, otherwise it is left in the current state. The interval to the next polling is recalculated. All commanding of heaters is performed with a call to change_config.

STOPPING

This simply ensures that all heaters are turned off and then transitions to STOPPED.

Note that, as start or stop can be called at any time, it is possible to transition to the STOPPING and STARTING from any state.

Set_params specifies heater control parameters. This routine allows direct manipulation of the parameters that control the heater algorithm. It is there to support the /ihtrparams command which is documented in the User Manual.

Start starts the heater control algorithm. This command may be called externally by command or internally from the mechanism package. It must therefore resolve possible race conditions. Provided these conditions are met, the state of the algorithm is set to STARTING.

Stop stops the heater control algorithm. This command may be called externally by command or internally from the mechanism package. The state of the algorithm is set to STOPPING and the source of the command noted to resolve possible race conditions when start is called.

Config is simply, for programming convenience, a pointer to the heater_config routine in package tmpsu. It is used for HK purposes.

Change_config commands heaters on or off.  It is given an argument that details the required on/off status of the four heaters. It then cycles through all four, leaving a gap of 0.1 s between each to avoid simultaneous commanding of more than one heater to minimize switching transients. It compares the requested status of the heaters against the current status and commands them only if there is a change.

Init initialize the package. This involves reading in the heater control parameters from EEPROM and validating them.

Control_active returns the status of heater control. This returns TRUE if the state of the algorithm is STARTED. It is used for HK purposes and to validate that it is OK to load heater parameters.

## Package hk

### Summary

This package obtains housekeeping items (HK), constructs the HK packet and forwards it to the telemetry queue package tmq.

### Description

ON starts the HK acquisition task PROCESS by requesting a transition to the Initial or Started states (see below).

OFF stops the HK acquisition task PROCESS by requesting a transition to the Stopped or Suspended state (see below).

Change_rate changes how often an HK packet is acquired.

Enable_tdrss_hk causes duplication of the next specified number of HK packets to TDRSS.

PROCESS

This is implemented as a simple state machine – see diagram. The code starts in the INITIAL state. The diagram implies that if off is called before the task is started (by a internal call to on from icu.ada), it will enter the SUSPENDED state and will then be returned to the INITIAL state by the internal start call. This facility is a copy of one in the basic code that provides a mechanism to prevent this task running. This provides a workaround should some bug be discovered at a later date in that PROM based unchangeable code. Note, however, that if these two events happen within the same state change request poll interval, it is possible for the internal on command not to be seen. It will then require two external (i.e. tele-command) calls to on to proceed to the STARTED state.

In the STOPPED, INITIAL and SUSPENDED states, a poll is performed every second to see if a command requesting a state change has been received.

In the STARTED state, the task commences its loop by calculating the time interval to the next HK packet acquisition. If greater than 1.2 seconds, it waits 1 s and then recalculates in the interval (and also examines whether a state change request has come in).  Otherwise it starts collecting the HK. It determines (using the ALIVE function of the ICB package) if the TM is powered and suppresses the collection of TM HK if it is not. Non-TM HK is always acquired. It then constructs and sends the HK packet, and optionally sends a copy to TDRSS. It then calculates the absolute time of the next HK packet acquisition and then returns to the start of the loop.

## Package icb

### Summary

This package provides mutex-controlled access to the ICB routines in package icb_driver.

### Description

Put writes to the ICB interface.

Get reads from the ICB interface.

Reset resets the ICB interface.

Report procedure supports the /icbread command. It calls get then sends out a packet containing the datum requested.

Hk_status returns the status of the ICB interface, but only for the last error.

Error_count returns a mod 256 count of ICB errors since the ICU started.

Busy returns TRUE of the interface in already is use i.e. if it has been seized by the mutex. This is used at the start of a mechanism move to ensure single usage by the mechanism task prior to beginning the interrupt driven movement.

Alive returns TRUE if the ICB interface is OK. This is determined by a test read from a TMPSU port.

## Package icb_driver

### Summary

This contains a suite of routines to perform I/O on the ICB interface.

### Description

Put writes to the ICB interface.

Get reads from the ICB interface.

Reset resets the ICB interface.

Error_count returns a mod 256 count of ICB errors since the ICU started.

Alive returns TRUE if the ICB interface is OK. This is determined by a test read from a TMPSU port.

Hk_status returns the status of the ICB interface, but only for the last error.

## Main Program icu.ada

### Summary

This main program starts up interfaces and initializes packages in the order required.

## Package icu_mem_manager

### Summary

This package contains code to perform ICU memory dump and loads.

### Description

read_block can read blocks of memory from detector RAM (using package detdigital, processor RAM (using package peek_poke), EEPROM (using package eeprom). It is called from the dump_memory task which loops through the memory requested a block at a time reading telemetry-packet-sized blocks. This is a low-priority task.

This same task and read_block are used for CRC reads and memory dumps. When it is a memory dump, there is a 0.25s delay between the reading of each block. This is because the telemetry rate is slow so a wait has to be induced.

load_memory does lots of checks and uses package peek_poke to write to processor RAM or package eeprom to write to eeprom. Blocks are written one packet at a time, which is sometimes not optimal for EEPROM but much simpler and produces much smaller code.

Function dump_memory sets up the variables for the memory_dump task.

## Package ieee

### Summary

This contains a set of routines to convert IEEE standard 32 or 64 bit floating point numbers to Tartan Ada standard. Such code is required because all floating-point numbers supplied in packets by the spacecraft are in IEEE format.

### Description

The IEEE (Institute of Electrical and Electronics Engineers) has produced a standard for floating point arithmetic. This standard specifies how single precision (32 bit) and double precision (64 bit) floating point numbers are to be represented, as well as how arithmetic should be carried out on them.

Function FLOAT32 implements the conversion for 32 bit IEEE numbers. The IEEE single precision floating point standard representation requires a 32 bit word, which may be represented as numbered from 0 to 31, left to right. The first bit is the sign bit, S, the next eight bits are the exponent bits, 'E', and the final 23 bits are the fraction 'F':

S EEEEEEEE FFFFFFFFFFFFFFFFFFFFFFF

0 1      8 9                    31

The value V represented by the word is determined as follows:

- If E=255 and F is nonzero, then V=NaN ("Not a number")

- If E=255 and F is zero and S is 1, then V=-Infinity

- If E=255 and F is zero and S is 0, then V=Infinity

- If 0<E<255 then V=(-1)**S * 2 ** (E-127) * (1.F) where "1.F" is intended to represent the binary number created by prefixing F with an implicit leading 1 and a binary point.

- If E=0 and F is nonzero, then V=(-1)**S * 2 ** (-126) * (0.F) These are "unnormalized" values.

- If E=0 and F is zero and S is 1, then V=-0

- If E=0 and F is zero and S is 0, then V=0

In particular,

0 00000000 00000000000000000000000 = 0

1 00000000 00000000000000000000000 = -0

0 11111111 00000000000000000000000 = Infinity

1 11111111 00000000000000000000000 = -Infinity

0 11111111 00000100000000000000000 = NaN

1 11111111 00100010001001010101010 = NaN

0 10000000 00000000000000000000000 = +1 * 2**(128-127) * 1.0 = 2

0 10000001 10100000000000000000000 = +1 * 2**(129-127) * 1.101 = 6.5

1 10000001 10100000000000000000000 = -1 * 2**(129-127) * 1.101 = -6.5

0 00000001 00000000000000000000000 = +1 * 2**(1-127) * 1.0 = 2**(-126)

0 00000000 10000000000000000000000 = +1 * 2**(-126) * 0.1 = 2**(-127)

0 00000000 00000000000000000000001 = +1 * 2**(-126) *

0.00000000000000000000001 =

2**(-149)  (Smallest positive value)

Function FLOAT64 implements the conversion for 64 bit IEEE numbers. The IEEE double precision floating point standard representation requires a 64 bit word, which may be represented as numbered from 0 to 63, left to right. The first bit is the sign bit, S, the next eleven bits are the exponent bits, 'E', and the final 52 bits are the fraction 'F':

S EEEEEEEEEEE FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF

0 1        11 12                                                63

The value V represented by the word is determined as follows:

- If E=2047 and F is nonzero, then V=NaN ("Not a number")

- If E=2047 and F is zero and S is 1, then V=-Infinity

- If E=2047 and F is zero and S is 0, then V=Infinity

- If 0<E<2047 then V=(-1)**S * 2 ** (E-1023) * (1.F) where "1.F" is intended to represent the binary number created by prefixing F with an implicit leading 1 and a binary point.

- If E=0 and F is nonzero, then V=(-1)**S * 2 ** (-1022) * (0.F). These are "unnormalized" values.

- If E=0 and F is zero and S is 1, then V=-0

- If E=0 and F is zero and S is 0, then V=0

Reference:

ANSI/IEEE Standard 754-1985,
Standard for Binary Floating Point Arithmetic

## Package importance

### Summary

This defines the priority of all tasks. It has no body.

## Assembler Routine INTVEC.ASM

### Summary

This defines the linkage and service pointers for all interrupts

## Package limit

### Summary

This acquires data that is then checked against allowed limits tabulated in EEPROM. If that range is exceeded, recovery action specified in the same EEPROM located table is taken.

### Description

Checking_enabled returns TRUE if the MONITORING task is in the STARTED state (see below). This result is used in Housekeeping.

Start starts the limit checking task MONITORING by forcing a transition to the STARTING state (but see note below about the initial stages i.e. as code starts).

Stop stops the limit checking task MONITORING.

Change supports the /ichangelimit command and enables ‘on-the-fly’ changing of the limit range of an item for diagnostic purposes. Whilst performing the change, it mutex protects access to the table and resets any execeed count values (see below) for that item.

Monitoring task

This is implemented as a simple state machine – see diagram. The code starts in the INITIAL state. The diagram implies that if stop is called before the task is started (by a internal call to start from icu.ada), it will enter the SUSPENDED state and will then be returned to the INITIAL state by the internal start call. This facility is a copy of one in the basic code that provides a mechanism to prevent this task running. This provides a workaround should some bug be discovered at a later date in that PROM based unchangeable code. Note, however, that if these two events happen within the same state change request poll interval, it is possible for the internal start command not to be seen. It will then require two external (i..e  tele-command) calls to Start to proceed to the STARTING state.

In the STOPPED, INITIAL and SUSPENDED states, a poll is performed every 0.5 s to see if a command requesting a state change has been received.

In the STARTING state, the EEPROM table is reloaded and checked for validity. If it is OK, the code proceeds to the STARTED state. If not OK, it proceeds to the STOPPED state. Note that even if the code is in a STARTED state, a call to the start function will force a transition (and therefore a table reload and check|) to this state.

In the STARTED state, the limit checking is performed. The task loops continuously, every 1/No of channels polled seconds extracting the next item from the EEPROM table that may be eligible for polling.. After every complete pass though the table, it also looks to se if a state change request has come in. This therefore will take just over a second, depending on how many items have been requested this pass by the table to be limit checked.

For each item in the table, the following checks are performed.

- Is this item enabled for checking?

- Whether it is the correct phase of this items poll cycle for checking i.e. if we are meant to check this item 1 out of 10 passes though the table, is it on the 1st, 2nd etc pass and are we at that pass.

- Does the sub-system code specified in the table valid?

If these checks indicate it should be limit checked, its value is obtained and compared against the limits in the EEPROM table. If outside the expected range, an exceed limit count for that item is incremented. If that value then exceeds the value given in the table, the recovery action is extracted from the table.

For basic code, this is either a ‘do nothing’, ‘power down UVOT’ or ‘power down telescope module’ code. For the latter two, the appropriate RT2RT command is sent to the spacecraft.

For operational code, this is either a ‘do nothing’ or an RTS code. In the latter case, a request to run the RTS is made using the RUN_RTS procedure of the DCS package.

For operational code, an error message is sent out – but only on the first occurrence of the limit violation -describing the error. Also 10 HK records will be echoed to TDRSS. Should the item go back into limits then out again, this cycle will repeat.

## Package math_constants

### Summary

Defines mathematical constants such as , e etc. It has no body.

## Package mechanism

### Summary

The package contains code to set up and terminate the movement of the dichroic and filter wheel mechanisms. The actual movement is carried out by the timer_a_ih package.  The latter provides code that is run at interrupt level and uses Timer A to control the filter wheel or dichroic, subject to constraints on that movement supplied by the mechanism package. It therefore requires a separate set of qualifiers during compilation. Placing the code is a separate package in a convenient way to achieve this.

### Filter Wheel

Eleven optical elements are placed at equal angles around the filter wheel. A pinion drives the wheel on a 4-phase stepper motor shaft, with a gear ratio of 11 to 1. Thus, one revolution of the motor, which requires 200 steps, moves the wheel from one optical element to another and 2200 steps will completely rotate the filter wheel.

| Number | Description | Steps from datum | Coarse | Fine |
| --- | --- | --- | --- | --- |
| 0 | Blocked (Datum) | 0 | True | True |
| 1 | Grism 1 (UV) | 200 | False | True |
| 2 | UVW2 | 400 | False | True |
| 3 | V | 600 | False | True |
| 4 | UVM2 | 800 | False | True |
| 5 | Grism (Visible) | 1000 | False | True |
| 6 | UVW1 | 1200 | False | True |
| 7 | U | 1400 | False | True |
| 8 | Magnifier | 1600 | False | True |
| 9 | B | 1800 | False | True |
| 10 | White | 2000 | False | True |

The wheel position will normally be determined by step counting from a known datum position. Coarse and fine position sensors are provided to relocate the datum position should it be lost, verify the wheel position during and after every rotation and to confirm that the centre of any optical element has been found, although the element is not identified. The coarse and fine sensors only both give a true output at the datum (see table above).

Tests indicated that the filter wheel should be rotated at a default pull-in speed of 200 Hz, a cruise speed of 420 Hz and an acceleration of 2000 Hz per second. These rates are applied when moving from filter to filter or from datum to filter. However, in order to ensure success when seeking datum, the filter wheel is rotated at a constant 200 Hz until the coarse sensor is detected and then at 10 Hz until the fine sensor is seen.

Points to Note:

- The filter wheel is a limited lifetime item – it has been designed and tested for 50,000 complete rotations for each redundant half of the instrument.

- The filter wheel movement is controlled via the ICB. This is also the main channel for acquiring housekeeping and controlling heaters. It was found during testing that activity on the ICB during a filter wheel movement could introduce erratic motion of the filter wheel and cause occasional failure in reading the fine sensor. Therefore, whilst the filter wheel is being moved, all other ICB activity (such a housekeeping acquisition and heater control) is stopped.

- commands are sent internally to disable the flood LEDs (see below) prior to any filter wheel movement should they have been left on accidentally after e.g. a calibration exposure, to prevent an illumination excess on the detector and

- the safety circuit is also disabled to prevent false triggering.

### Dichroic Mechanism

The dichroic mechanism contains a mirror placed at 45 in the path of the incoming beam. The purpose of the mechanism is to steer the reflected light beam from one of two redundant detector systems to the other. It will be rotated from one position to the other by pulse counting. The final step will drive the rotor to its stop where it will be held by a magnetic detent. The dichroic mechanism is rotated 180 between the stops and by a 4 step per revolution motor geared at 14.5:1. Therefore, the motor needs to be driven up to 29 steps from one position to the other. One further step in each direction means that the rotor is driven hard onto its stop. Thus, the total number of steps required is 31. The step sequence has to be reversed to return. As there is no harm in overdriving the system against this stop, the motor is always driven the maximum number of steps required in the specified direction. The drive frequency is 2 Hz. As there are no sensors in the system, the control mechanism is always open loop.

The following algorithm is used. If we label the 4 phases 1,2,3 and 4, a clockwise rotor drive (viewed from the shaft end) towards the redundant detector is achieved by stepping in a positive direction (e.g. the phases are energized in the order 1,2,3,4,1…) until the step count is equal to or greater than 31 and the phase is 1. Similarly a counter-clockwise rotor drive towards the primary detector is achieved by stepping in a negative direction (e.g. the phases are energized in the order 4,3,2,1,4…) until the step count is equal to or greater than 31 and the phase is 2.

### Description

Set_filter_wheel_movement is used to specify what the next filter movement will (e.g. to datum, to a specific filter etc) and also the conditions implied by that movement to determine when it has finished. The code in the package timer_a_ih performs the monitoring of those conditions.

Set_dichroic_direction is used to specify what direction the next dichroic movement will be (i.e. to or from redundant). The code in the package timer_a_ih performs the monitoring of those conditions.

Move_filter_wheel performs pre-movement checks and action and then requests code in timer_a_ih to perform the movement.

If it is a request to move the filter wheel, the following actions, which vary according to the type of movement requested, are performed.

For a move to a specific filter (a number in the range 0 => 10), it first checks that the datum position has been previously determined and forces and exit if this has not happened. On the basis of the filter number, it determines the final position of the filter wheel and, if the wheel is already known to be at that position, forces a successful exit. If we need to move, a failsafe zeroing of the flood LEDs then takes place to prevent any detector damage during the move. It determines how many motor steps are needed to move to the required position. If the requested filter position is known to be located on a multiple of 200 filter wheel steps, where we would expect the fine sensor to be detected, a flag is set to ensure this check is carried out at the end of the movement. Although all the filters are centred on multiples of 200, for optical reasons the wheel may not always be requested to move to the central position. A flag is set specifying that the initial part of a filter wheel will involve acceleration. This flag will be used by the code in timer_a_ih.

For a move to an absolute filter wheel position (a number in the range 0 => 2199), similar checks to the above regarding the datum and the current location are performed and the number of steps to move calculated and the acceleration flag set. As this is a contingency operation, no further actions take place.

If the contingency action of a move by a relative number of steps is requested, only the acceleration flag is set.

If the contingency action of a move by a fixed number of fine sensor pulses is specified, that implicitly defines the end condition and flags are set accordingly. As a failsafe, that sensor is turned on. A flag is set confirming that this action is performed without an acceleration to high speed but, instead, at the starting slow speed.

For a move to datum request, the acceleration flag is set and flags set to indicate that both the fine and coarse sensors should be true at termination. As a precaution, those sensors are turned on. After a short delay, they are polled so that a successful exit can be forced if we are already at datum.

For a move to coarse request, the flag is set to indicate no acceleration of the wheel is required. A flag set to indicate that coarse sensor should be true at termination. As a precaution, that sensor is turned on. After a short delay, it is polled so that a successful exit can be forced if we are already at coarse.

A call to the task MECH is then made. If it is active already, the code exits with a busy error. Task MECH is described below.

Move_dichroic performs a set of similar but simpler actions for the dichroic. They are simpler because there are no sensors involved as all movement is by ‘dead reckoning’ and it only whether the movement in valid (we must be in the safe state) and which direction is requested. It then calls MECH.

Task MECH first turns off the heaters in order to minimize power during the movement. If it is appropriate to warn the BAT that the filter is about to move (it is enabled, it is not an emergency move of the filter wheel and not early in an AT exposure sequence, a /BAFWWARNING message is sent and then the code waits for the EEPROM tabulated amount. Also the detector safety circuit is disabled to avoid a false trigger during the move and the UVOTMAN package is instructed to calculate the number of theoretical counts the detector would have received during the wait.

If the filter wheel is being moved, the point at which it must start braking is calculated for later use by the timer_a_ih package. It also calculates the initial pattern of phase lines to be powered. If a dichroic move is requested, only the latter is calculated.

It then calculates the length of the initial inter-pulse gap, seizes the mutex FW_MOVE which effectively stops all other traffic on the ICB and starts the interrupt handling code in timer_a_ih to perform the movement.

The task then polls every 0.1 seconds to see if the code in timer_a_ih has finished the movement. When it detects this is the case, it calls terminate_movement (see below).

Terminate_movement performs a general failsafe shutdown of the movement and performs checks that it was successful.

It first does a failsafe cancelling of any pending Timer A interrupts, ensures all phase lines are un-powered and releases the mutex FW_MOVE so activity of the ICB can resume. It also re-enables the detector safety circuit. After a short delay to allow the mechanism to settle, the status of the coarse and fine sensors determined and checked for TRUE as required by the movement commanded. They are then turned off. If required, the BAT is informed of the completion of the move with a second /BATFWWARNING message. The heaters are restarted and the a success of failure message issued as appropriate.

Fw_Position and DM_position return the position of the filter and dichroic for use in the HK display.

Change_pulse_rate is used to change the default speed and acceleration settings for the filter wheel. In the heritage instrument, it also could change the dichroic rate, but this option was omitted for Swift. This accounts for the now redundant routine change_fw_step_rate.

Last_fw_movement_ok is a heritage from XMM-OM and is no longer used. The optimizing Ada compiler ensures no code is generated.

At_coarse_sensor and at_fine_sensor return whether the coarse and fine sensors respectively were TRUE when last examined i.e. at the end of the last movement and not their current state. This information is used in the HK.

Init is called to determine the coarse and fine sensor status on turn-on.

## Package memloc

### Summary

This defines memory locations for those variables or data areas requiring a fixed location in RAM. It has no body. See “Ada Exception Handling and Debugging”, section 2.8.

## Package mem_manager

### Summary

This validates any received memory management packet and calls the appropriate routines in icu_mem_manager.

## Package modeman

### Summary

TO_MODE performs the state transitions supported by the Basic code i.e. restart back to Basic or the switch to Safe state in operational. It is therefore only present in the Basic code. For HK purposes, it also provides which State the ICU is in with the function MODE.

## Package monitor

### Summary

This specification only package defines the tasks that are to be monitored by the watchdog package.

## Package mutex

### Summary

This package provides the routines to allow exclusive use of a given resource, in order to prevent simultaneous modification of a critical data area of use of a resource such as an interface.

### Description

Procedure SEIZE will not return to the calling routine, and thus blocks it, until the requested resource (as defined by the mutex name supplied) is available. It works by

- Entering a critical region.

- Polling the flag that indicates whether the resource is taken.

- If it is taken, it leaves the critical region, waits 0.1s then tries the sequence again until the resource is available.

- If it is not taken, it sets the flag saying the resource is now taken, leaves the critical region and returns to the calling routine.

Procedure RELEASE will release the requested resource (specified by the supplied mutex name) by simply unsetting the appropriate flag.

## Package nhk

### Summary

This constructs Non-periodic Housekeeping (NHK) packets and calls package tmq to queue them for output. It consists of one procedure put.

## Package packet

### Summary

This specification only package defines the format for all telemetry, telecommand and RT-to-RT packets handled by the ICU. It has no body.

## Package peek_poke

### Summary

Examines or modifies address in ICU RAM in all addressing modes.

### Description

It is coded in assembler.  It changes the address state and read and writes values to processor RAM.

## Packages pps_ih and pps2_ih

### Summary

pps_ih provides the 1 PPS interrupt handler for Side A

pps2_ih provides the 1 PPS interrupt handler for Side B

### Description

When an interrupt arrives to either of these, they store the value of the 1553 timer in an area of memory for the Ada code to read. They also each increment a different interrupt counter.

If two of the 1pps-A interrupts arrive too close together (less than approx 0.7s), the 1pps-A interrupt handler will switch to the 1pps-B by commanding a switch on the ICU board thereby disabling 1pps-A and enabling 1ppsB.  This is to avoid too many interrupts jamming up the ICU.

## Package primitive_functions

### Summary

The package provides functions that act on floating point types whose implementation depends on access to the floating point or mantissa of the floating-point number.

## Package reset

### Summary

Basic Code – this contains one routine to perform the switch to operational.

Operational Code – this contains one routine that will reload the operational code from EEPROM and then execute it, thus performing a ‘warm’ restart of the ICU operational code.

They both do this by calling the loader 2 that was loaded by the bootstrap.

### Description

The codes works by disabling all interrupts, stopping timer B, selecting address state 0 then jumping to the start of the code loader in PROM.

## Package safetyport

### Summary

This specification-only package provides the format details of the ICB ports used to control the detector safety circuit.

## Package sc1553

### Summary

This contains routines to read/write packets to the 1553 interface.

### Description

The package provides a complete driver for 1553 interface. It does not assume too much about timing or synchronicity.

The interrupt handler in package sc1553_ih counts the telecommands as they arrive and the sc1553 functions/procedures read these regularly when tcq tells them to.  It also checks for errors.

First sets up the descriptor_table then sets up the 1553 chip and performs the built-in test.

Continuously checks all flags and the descriptor table for errors.

The important procedures are read_packet and reset

The important functions are write_packet and write_packet_rt_to_rt - their names are self –descriptive.

The package also provides read_timetag to read the timer for testing purposes. It returns a counter incrementing every 64us.

## Package sc1553_ih

### Summary

This contains the 1553 interrupt handler, coded in assembler.

## Package ssi

### Summary

This package read and writes packets to the SSI interface.

### Description

This package supplies a reset that resets the interface.

read_status reads the SSI status.

get_word reads a word from the interface (from the DPU)

get reads a block from the SSI performing lots of checks on the data and input fifo.

put increased the priority of the task to be the highest then writes a block to the SSI in a loop afterwards resets the priority of the task to normal . This is needed because any time gaps will be interpreted as block gaps and corrupt the data.

## Package ssi_ih

### Summary

This package handles SSI interface interrupts. The body is implemented in assembler.

## Package starcat

### Summary

This scans an on-board catalogue of stellar objects and returns stars present within the supplied field of view. It works in conjunction with the astro package to provide information about possible bright objects in the field of view.

### Description

Onboard Star Catalogue Dataset

The onboard catalogue is split into three parts – the pointer table, the main catalogue and the addendum. It is stored on EEPROM B, Mid 5.

The three parts of the catalogue follow each other sequentially, with no gaps, as in fig. 1 below. A much more detailed memory map, with addresses for each sky area within the main catalogue, is autogenerated on catalogue preparation, and stored in file memorymap.dat. This file can be found in the star catalogue archive provided.

Address     Data stored

0x6f39c    End of addendum data

Addendum

Main Catalogue

Pointer Table

0x6f26d    Start of addendum data

0x6f26c    End of main catalogue

0x1413     Start of main catalogue

0x1412     End of pointer table

00000	     Start of pointer table

Memory Map

The Pointer Table

This is stored from address 0 to address 0x1412 inclusive.

Each pointer is 2 words long.

The first 44 pointers point to the start of each declination band within the pointer table itself. The bands run in order from the north band nearest the equator to the north polar band, followed by the south band nearest the equator to the south polar band.

The next 2524 pointers point to the start of each sky area in the main catalogue. The sky areas run along the declination bands, in increasing order of RA.

The last pointer points to the start of the addendum.

The last entry in the pointer table is a one word CRC value.

The Main Catalogue

This is stored from address 0x1413 to address 0x6f26c inclusive.

It contains 223807 entries. These are stored according to sky area, with sky areas arranged as in the pointer table.

Each entry is stored as two words;

The first byte is its RA offset from the origin of the sky area.

The second is the declination offset.

The third is its magnitude code.

The fourth is its colour index code.

Each sky area ends in a one word CRC value.

The Addendum

This is stored from address 0x6f26d to address 0x6f39c inclusive.

It contains space for 100 entries, each of which is stored as three words;

The first byte is the RA division the source is in.

The second byte is an encoded version of its declination band.

The third byte is the RA offset from the origin of its sky area.

The fourth is the declination offset.

The fifth is its magnitude code.

The sixth is its colour index code.

It also contains an end of list marker consisting of three words, all of them 0xFFFF. This will be positioned at the end of the relevant entries in the addendum. If there are no entries it will be at the beginning of the addendum. All allotted space after the end of list marker will be zero filled.

The last word of the addendum is the CRC value.

Using the Package

To initiate a search call STARCAT.START_SEARCH(RA, DEC, RAD).

RA is the right ascension of the centre of the field of view in degrees, as a float.

DEC is its declination, also in degrees, as a float.

RAD is the radius of the field of view, in arcmin, as a float.

Please note that RA runs from 0.0 to 360.0 and Dec from –90.0 to 90.0.

To retrieve all the sources in the field of view, call STARCAT.GET_STAR(FINISHED, CRC_ERROR, SOURCE_OFFSET, SOURCE_MAG, SOURCE_INDEX, SOURCE_RA, SOURCE_DEC, SKY_AREA, ORIG_OFFSETS) continuously until FINISHED is returned as true. This final return will contain dummy values for the three source parameters, which should be ignored.

FINISHED is a boolean, which will be set to true if all sources have been returned, and false otherwise.

CRC_ERROR is a boolean, which will be set to true if any of the CRC checks made during execution discovered memory corruption in the onboard catalogue. In that case the final return would be issued immediately, and all information from previous calls should be ignored. See later for further details.

SOURCE_OFFSET is the angular distance between the found source and the centre of the field of view, in arc minutes, as a float.

SOURCE_MAG is the magnitude code of the source (see later), as an integer.

SOURCE_INDEX is the colour index code of the source (see later), as an integer.

SOURCE_RA is the RA stored in the onboard catalogue for the source.

SOURCE_DEC is the declination stored in the onboard catalogue for the source.

SKY_AREA is an encoded version of the sky area the source is in – see the CRC section below for the method of decoding it.

ORIG_OFFSETS contains the offsets of the source from the origin of the sky area, the first byte is the RA offset and the second is the declination offset.

Code Overview and CRC error checking

The code runs as follows;

Receive a call to start_search.

CRC check the pointer table.

Locate and CRC check the addendum.

Work out which declination bands are in the field of view.

Loop through the bands.

For each band work out which sky areas are in the field of view.

Loop through the sky areas.

For each sky area, run a CRC check

For each sky area call package astro to calculate the angular separation of each source within it from the centre of the field of view. Return any sources within the field of view via get_star.

After each sky area check the addendum to see if any of its entries are within that sky area. Repeat the angular calculation for any sources found and return them via get_star if necessary.

Return a ‘finished’ flag via get_star.

If any CRC check is failed the code will automatically return get_star with the ‘finished’ flag set. It will also send an NHK message, ‘Corrupted EEPROM Data’. The first parameter of the NHK message will be the catalogue identifier, 0x0004, and the second will identify the catalogue area that is corrupted, as follows;

0xEEEE – the pointer table

0xAAAA – the addendum

< 0x4E20 - one of the individual sky areas in the main catalogue. To work out which,

- convert it to a decimal value

- left pad it with zeroes to make a 5 digit number

- if the first digit is 1, it is a southern hemisphere area, otherwise it is northern

- the second and third digits are the declination band

- the fourth and fifth digits are the right ascension division

so for example, 0x04C0 becomes decimal 01216, which indicates sky area north 12 16.

To find the location any of these areas in memory read the file memorymap.dat, which is in the star catalogue archive.

Sky Area Grids and Naming Convention

The sky is taken to be two hemispheres that are mirror images of each other.

Each hemisphere is split into 22 declination bands, numbered 0 to 21 starting from the equator and going to the pole. Each band is 255 arc minutes wide except for the polar bands, which are 45 arc minutes wide. Any source within half an arc minutes of the equator is taken to be in the first band of the northern hemisphere only.

Each declination band is divided into a number of equal right ascension divisions, the number depending on how the width of the field of view changes with respect to right ascension at each declination.

Bands 0 to 4 have 90 divisions each.

Bands 5 and 6 have 80 divisions each.

Band 7 has 75 divisions.

Bands 8 to 10 have 72 divisions each.

Band 11 has 60 divisions.

Band 12 has 54 divisions.

Band 13 has 50 divisions.

Band 14 has 45 divisions.

Band 15 has 40 divisions.

Band 16 has 36 divisions.

Band 17 has 27 divisions.

Band 18 has 24 divisions.

Band 19 has 15 divisions.

Band 20 has 8 divisions.

Band 21 has 2 divisions.

The right ascension divisions are numbered from 0 upwards starting at RA 0.0 degrees.

So for example, sky area south 12 30 would be in the southern hemisphere in declination band 12, which extends from –51 degrees to –55 degrees 15 arc minutes, and right ascension division 30, which in this band extends from 200 degrees to 206 degrees 40 arc minutes.

Each sky area is then subdivided into a 255 by 255 grid and every source assigned to its nearest grid point. The RA and declination offsets stored in the catalogue are the coordinates of this grid point within the sky area, counting from 0 to 254 in each axis. These are the offsets stored in ORIG_OFFSETS Positional accuracy in the field of view in either axis is to half an arc minutes or better at all times.

If any source in the catalogue is given offset coordinates of (255,255) that is an indication to the code that this source has been ‘deleted’ from the catalogue. That source will be ignored by the code.

Magnitude Codes

The magnitude code stored on board is derived from the visual magnitude (Johnson V magnitude).

If V is < -1.4 then the code is 0.

From there on, the magnitudes are gathered into bands, each 0.1 of a magnitude wide, numbered from 1 to 134 ascending.

So for example,

-1.3 > V >= -1.4 has code 1.

-1.2 > V >= -1.3 has code 2.

11.9 > V >= 11.8 has code 133.

V >= 11.9 has code 134.

No source fainter than V = 12.0 will be stored in the catalogue.

Colour Index Codes

In this case the colour index is B-V in Johnson magnitudes. This is split into uneven bands as follows;

B-V < -0.33 has code 0.

-0.33 <= B-V < -0.27 has code 1.

-0.27 <= B-V < -0.17 has code 2.

-0.17 <= B-V < -0.11 has code 3.

-0.11 <= B-V < 0.01 has code 4.

0.01 <= B-V < 0.15 has code 5.

0.15 <= B-V < 0.30 has code 6.

0.30 <= B-V < 0.44 has code 7.

0.44 <= B-V < 0.52 has code 8.

0.52 <= B-V < 0.63 has code 9.

0.63 <= B-V < 0.68 has code 10.

0.68 <= B-V < 0.74 has code 11.

0.74 <= B-V < 0.81 has code 12.

0.81 <= B-V < 1.10 has code 13.

1.10 <= B-V < 1.49 has code 14.

1.49 <= B-V < 01.64 has code 15.

B-V >= 1.64 has code 16.

## Package support_functions

### Summary

This is a package of single-precision functions that support the package elementary_functions.

## Package taskman

### Summary

This command, working in concert with the command_distributor package, completes the parameter validation and distribution of commands to the telescope module, watchdog and the virtual machine that executes RTSs, which is contained within the package dcs.adb.

### Description

All packets are acknowledged with a verification message. Any error messages resulting from the rejection of a packet will have been issued by the relevant package with the exception of those caused by an unrecognized APID or Function Code, which are issued by this package.

The taskman package is used separately to check this block of commands for code heritage reasons. The two packages could have been merged if the effort had been considered worthwhile.

## Package task_report

### Summary

The put procedure constructs a Task Parameter report and places it in the queue for telemetry using package tmq. Such reports are generated when polling a single ICB port for diagnostic reasons.

## Package tcq

### Summary

This extracts packets from the 1553 interface and forwards them to the command_distributor package. It also sends out RT2RT packets.

## Description

Function put uses a mutex to obtain exclusive use of the sc1553 RT2RT port. It then generates, from the supplied APID and Function Code, the required RT2RT packet and sends it. It validates the operation and then increments the error count or good packet count, for HK purposes, accordingly. Note that is does not support the sending of any application data.

Task poll_tc polls the 1553 interface every 0.01 until a packet arrives. It uses the internal function get to perform this action. This function also validates the packet (using the internal function valid_packet) and will not return until it obtains a valid packet. If an invalid packet is detected, an appropriate error message is issued. One a valid packet is returned, it is then sent to the command_distributor package.

Function sequence_count_field generates the next sequence count, as a function of the supplied APID, to be used in

- A RT2RT command to be sent out by put.

- A TC to the DPU via the SSI.

## Package tc_verify

### Summary

This provides a suite of routines to generate and send error or event messages associated with verification of incoming packets.

### Description

Successful_acceptance generates and sends a successful acceptance message packet to the telemetry queue.

Unsuccessful_acceptance generates and sends an unsuccessful acceptance message packet to the telemetry queue with the supplied error code and parameters.

Unsuccessful_execution generates and sends an unsuccessful command execution message packet to the telemetry queue with the supplied error code and parameters.

Report_error is a simplified version of Unsuccessful_acceptance that does not accept parameters.

## Package time

### Summary

By examining the time-tone packets, it provides Mission Elapsed Time (MET) and corrected spacecraft time.

### Description

Time has get_met() which adds the time of the last TIMETONE packet to the 1553 timer value and adds the UT correction factor UTF.

get_corrected_spacecraft_time returns a UTC which is the Modified Julian Day (MJD) minus 51910.0 so to get MJD call get_corrected_spacecraft_time and add 51910.0

sync_set() is called when a TIMETONE packet arrives and stores the values from the packet for the next time calculation

## Package timer_a_ih

### Summary

This package works in conjunction with the mechanism packages. It provides code that, although in Ada, is run at interrupt level and uses Timer A to control the filter wheel or dichroic, subject to constraints on that movement supplied by the mechanism package. It therefore requires a separate set of qualifiers during compilation. Placing the code is a separate package in a convenient way to achieve this.

### Description

The Tartan supplied package ALARMCLOCK is used to control the Timer-A to produce interrupts at the rate required to pulse the filter wheel or dichroic motors. A similar correction to that applied to all timer-B operations, such as for a normal delay as documented in section 2.9, “ICU Delay Adjustment”, is also applied. However, in this case it is directly applied in the Ada code rather than in a library.

The constructs seen in the specification, and as advised by Tartan, lead to the routine when_alarm_happens being called, using the interrupt stack, when a Timer-A interrupt occurs. Restrictions imposed by the Tartan compiler require that no tasking operations be performed in such an interrupt level routine; in particular no calls to delay must take place.  As the routine calls the icb package to send commands to the mechanisms, and this package uses delays as part of the mutex protection, it must retry at a slightly later time if such a mutex is in effect and prevent that package calling the mutex routines when it does run. The flags ICB_BUSY and AT_INTERRUPT_LEVEL are used to achieve these two effects respectively.

Start commences the movement of the mechanisms by setting up the first interrupt. The movement is sustained by when_alarm_happens setting up the next interrupt. All interrupt times are set up as absolute rather than relative times. This ensures a set of regular interrupt pulses, subject to conflicts with the icb package. This latter effect is minimized by stopping as much activity on the ICB as possible by halting housekeeping acquisition, limit monitoring and heater control. The package mechanism looks after that process.

Stop kills the movement by suppressing the next interrupt pulse.

When_alarm_happens is called at each interrupt. Its first act is to determine if the ICB is currently in use and to reschedule itself if that is the case. It then set up the next interrupt. Then, with the exception of the first pulse, when no movement will have taken place yet, it will first determine if the movement is now complete. The precise terminating conditions are a function of which type of movement that has been requested, and will have been set up by the mechanism package. This may include the use of one or more of coarse_position_sensed or fine_position_sensed, which inform it whether the coarse or fine position sensors respectively read TRUE. If a terminating condition has been reached, the stop procedure is called and the routine exits. Otherwise the next motor phase pulse is determined and sent, using set_phase. At this point the mechanism begins moving. Various bookkeeping counters are incremented e.g. number of pulses, and then the interval to the next interrupt determined. This will depend on which mechanism and, if it is the filter wheel, whether it is accelerating, cruising or slowing which it must first determine.

## Package tmpsu

### Summary

This provides a suite of routines to control and monitor those items powered and controlled by the Telescope Module Power Supply (TMPSU).

### Description

Set_motor_temp_control enables or disable the motor temperature control circuit.

Motor_temp_control_enabled returns the status of the temperature control circuit.

Set_coarse_position_sensor_current sets the current for the coarse sensor LED that illuminates the coarse sensor of the filter wheel. Note that the value is not used until a later call to coarse_sensor. Any values greater than the maximum of 15 are set to fifteen.

Coarse_sensor_current returns, for HK purposes, the value specified in Set_coarse_position_sensor_current.

Coarse_sensor turns on or off the illuminating LED of the coarse sensor of the filter wheel. The value used is supplied by and earlier call to Set_coarse_position_sensor_current.

Set_heater_config commands the on/off status of all the heaters. The configuration is checked against a list of permitted heater configurations and rejected if not included. This prevents both focusing heaters being enabled at once.

Heater_config returns, for HK purposes, the on/off status on all the heaters.

Current returns the current for the specified secondary supply circuit for HK purposes. It works by initiating an analogue to digital conversion, waiting for it to complete and then and returning the value. This is used for limit checking and HK purposes.  As this involves two ICB ports (the first to initiate the A-to-D on a particular channel, the second to read the value on that channel, a mutex is set up prior to using the first port and only removed after the use of the second port. This is to avoid a ‘race’ condition between two different tasks (the HK and limit checking task for example) in which the channel being polled by one task is changed by the other task. This is in addition to the mutex applied internally in the icb package, which is called to perform the interrogation of the ICB ports.

## Package tmq

### Summary

This queues packets for the 1553 interface using procedure put.

### Description

Procedure put first validates the supplied packet and will exit with an error message if this check fails. Should its APID indicate it is an error message, a copy is then sent to run_recovery_rts in package uvotman for processing.  The time stamp is generated and inserted into the package, unless it is an error message with a code indicating a time related error, in which case the previous value is used. It then polls every 0.1s the Swift Telemetry Protocol Data Unit buffer (ST_PDU) until it is possible to insert the packet. A mutex is applied during this process. The sequence counter and checksum are then generated, the packet inserted, associated buffer pointers updated, various HK counters incremented and the mutex released.

Task feed_1553 polls, every 0.95s under mutex control, the ST_PDU that has been filled earlier by put. When it finds packets in the buffer, it sets up the packet so it can be sent over the 1553 interface. If this latter process results in an error, it will increment an bad TM counter after 2 re-tries.

## Package types

### Summary

This defines additional standard types used throughout the code. It has no body.

## Package uvotman

### Summary

Note: This package is only present in the operational code. Therefore, it only deals with those state transitions applicable to that code.

### Description

This suite of routines coordinates aspects of UVOT operational code that involve a state change. As such, they provide overall control, not only of science observations, but engineering and calibration exposures and safing procedures. However, those state changes, with the exception of some specialist safety-related transitions, are performed by RTSs. See the operational User Manual for a summary of the RTSs.

The package works by providing routines that monitor, and extract information from, the

- S/C attitude record (process_siscattitude) that, as well positional information, contains settled, SAA and within 10 arc min flags and information that indicates the next exposure will be a safe-pointing. It also contains a flag warning of an emergency power down.

- SISLEWWARNING record (process_sislewwarning) which warns of impending slews

- SISLEWABORT (process_sislewabort) that warns of aborted slews

- FONEXTOBSINFO record detailing any impending non safe-pointing exposure (process_nextobsinfo)

- BATGRBFLUXINFO record that may indicate the need to choose an exposure sequence more suitable for a bright source (process_batgrbfluxinfo)

- XRTPOS record (process_xrtpos) that gives the XRT determined source position, which may lead to an adjustment of the observing window.

Communication between this set of routines and the RTS system is provided by

- Raising and lowering of a set of semaphores (see the package dcs description)

- Extraction, or derivation, of the values of variables and placed into reserved memory locations which are examined by the RTS system

- Routines that provide ‘services’ to the RTS e.g. reading in an exposure configuration from EEPROM.

- The RTS (or routines it calls or initiates) issuing error messages that are eventually processed by the procedure run_recovery_rts that does what its name suggests.

The state change required is determined by function to_state

function to_state

This routine coordinates state changes.

In order to achieve its goals, the ICU must successfully transition the UVOT between several instrument configuration states. The possible transitions are shown in the diagram.

Each state transition is performed by a RTS. A look-up table located in EEPROM contains a list of those RTSs against the requested transition. For those transitions that may be autonomous – see below – it also contains the required state of 13 internal ICU flags for that particular transition to take place.

| Flag | Column | Description |
| --- | --- | --- |
| OUT_OF_SAFE_ENABLED | 0 | Defined by 1st parameter of /istatetrans command |
| AT_OBS_ENABLED | 1 | Defined by 2nd parameter of /istatetrans command |
| PT_OBS_ENABLED | 2 | Defined by 3rd parameter of /istatetrans command |
| SAFEPOINT_OBS_ENABLED | 3 | Defined by 4th parameter of /istatetrans command |
| AUTO_ENABLED | 4 | If set to 1, is considered for autonomous transition |
| IN_SAA | 5 | If 1, s/c is in SAA |
| IN_10_ARC_MIN | 6 | If 1, s/c is within 10 arc min of final position |
| SETTLED | 7 | If 1, s/c is settled |
| POWER_DOWN | 8 | If 1, s/c is about to perform emergency power down |
| NEW_AT_SLEW | 9 | If 1, we have not observed this GRB before, |
| OBS_TYPE | 10 | Type of observation = 0/1/2 = AT/PT/SP |
| SLEW_WARNING | 11 | Set to 1 if a slew warning has been received |
| FINDING_CHART_OBTAINED | 12 | ATs: Set to 1 if finding chart exposure was completed or abandoned |

The current value of the flags is determined by other routines in this package (see below).

This table is used in two ways.

| On command: On reception of an /istate telecommand requesting a particular transition that may have been issued from the ground or, alternatively, from a running RTS, a call to TO_STATE in a valid range (1=>13, see table) the table is scanned for a match against both the current state and the requested state. If a match is found, the relevant RTS code number is selected and returned for execution by the calling code. If no match is found, zero is returned. State | Code |
| --- | --- |
| SAFE | 1 |
| SLEW | 2 |
| SETTLING | 3 |
| FINDING | 4 |
| AUTOMATED | 5 |
| PLANNED | 6 |
| SAFEPOINT | 7 |
| SAA | 8 |
| SLEWSAA | 9 |
| IDLE | 10 |
| OFFURGENT | 11 |
| OFFUVOT | 12 |
| OFFUVOTTM | 13 |

- Autonomously: A number of events will cause the ICU to select the required state transition itself by comparing the table of internal ICU flags against the current values. This is done with a call to to_state, usually from an RTS via the “chain ?” instruction, with an argument of AUTO (= 0xaa). If a match is found, the RTS is selected and executed. If no match is found, the request is ignored.

Procedure process_sisattitude

This procedure is called for every occurrence of this 5Hz rate message.

It first examines the record to determine whether a safe-pointing exposure is being signalled with the observation number in the range 1=>6. If so, it sets flags accordingly.

It then examines the record for any change in the within_10_arc_min flag. It so, it sets the appropriate RTS semaphore in case a script is waiting on it. If the flag has been raised, it validates whether we really are within 10 arc minutes of any predicted position (such as would have been given in a preceeding FONEXTOBSINFO record) and issues an error message if this is not true.

The settled flag is then examined for any change. If it is now false, the RTS semaphore signalling the start of a slew is raised. It also verifies that it was expected i.e. the change has come after a SISLEWWARNING record and issues a error message if it was unexpected. If the flag is now true, the semaphore indicating the spacecraft has settled is raised. It then checks whether a FONEXTOBSINFO message has been received this snapshot. Provided we are not in a safe state, a check is made that we are within absolute drift limits of the predicted position and an error message issued if this condition is violated. Otherwise, it simply assumes that the current position is the predicted position. This would be the case for a safe-pointing exposure.

In order the save some CPU, the following checks are then performed only every 2 seconds i.e. every 10 records.

The sun, earth and moon positions are checked to ensure they have not violated the angular constraints. Error messages are issued every minute if this is the case, except when slewing, in which case they are issued every check i.e. every 2 seconds.

If we are settled, a check is made that we are within relative drift limits since the last check. An error message is issued if this condition is violated. The absolute drift check performed when we first settled  (see above) is then repeated.

As the spacecraft bus voltage is provided in this record, it is then extracted for housekeeping and limit checking purposes. It is also made available to the heater algorithm (see package heater) via the function bus_voltage.

A check is made that the power down flag has not been raised. If it has, the RTS to perform the emergency safing is started.

Finally, a check is made on the status of the SAA flag. If it has changed, and we are not in safe, a request to to_state is performed to select the appropriate RTS and it is then started.

Procedure process_sislewwarning

As its name implies, the spacecraft warns of an impending slew by issuing this message.

If the UVOT is already in a safe condition, this routine immediately issues the appropriate RT-RT message (SACSLEWSAFEREPLY) to tell the spacecraft is OK to proceed. Otherwise, it starts the appropriate RTS that handles the preparation for slew (i.e. setting the Vcathode high voltage to zero and informing the spacecraft). That same RTS then initiates the preparation for the next observation after the slew, which in turn is controlled by the setting of semaphores and values described for this package.

As the ICU effectively only has a ‘memory’ of observations over one slew (with the exception of a small amount of observational detail for AT exposures) the code uses this as an appropriate place to clear various flags that control the later flow.

Procedure process_sislewabort

Provided we not in the safe state, this routine, called on receiving a /sislewabort, runs the appropriate RTS to recover from this condition.

Procedure process_nextobsinfo.

This routine is called on reception of a FONEXTOBSINFO. This record, as it name implies, carries information about the next observation.

It first extracts the supplied target pointing position. Provided they are in a valid range, it then proceeds to store in locally information that will either later be used to choose the exposure configuration or passed on to the DPU. This includes items such as the UVOT mode, the observation number, when the GRB was detected and the predicted length of the snapshot. In addition flags that will be used to choose the appropriate RTS to run for the required exposure will be stored e.g. whether this the first time a particular GRB has been observed. If this latter is true, the small context the ICU maintains about the current GRB is cleared in favour or this new one. This includes such items as the last filter used and any XRT offset calculated. Additionally, if the observation number in this case matches the observation number supplied in a previous GRBFLUXINFO packet, then the appropriate RTS semaphore is raised to indicate that GRB flux information relevant to the current exposure is now available.

Procedure process_xrtpos

This is called on receipt of an XRT RT-RT message detailing the derived GRB position.

The procedure first extracts the XRT supplied RA and Declination.

Provided those values are in range, it then obtains the radial offset r from the predicted pointing position. As the angles are small, it uses an angular distance formula accurate for small angles given in Peter Duffet-Smith, “Practical Astronomy with Your Calculator - 3rd Edition” by Peter Duffet-Smith. Note that in the rest of the procedure, angular difference are calculated using a simple vector dot product because the accuracy is not required.

The pointing vector P corresponding to the supplied XRT position αXRT,δXRTis then calculated using

P = cosδXRTcosαXRTcosδXRTsinαXRT

The bore-sight pointing vector B is similarly calculated from the target position αB,δB

B = cosδBcosαBcosδBsinαB

A check is then performed that the above radial offset is within twice the angular radius of the field of view (to allow for the possible range of bore-sight offsets that will be added in at the last stage of the calculation). If it is not, an error message is issued and the XRT offset is set to zero.

Provided the radial offset is at least one pixel size, the code then proceeds to calculate the offset that must be applied to the image by the DPU to re-center it on the GRB. Otherwise, it is assumed to be zero.

The EEPROM tabulated UVOT roll with respect to the s/c axes is then added to the supplied bore-sight roll. This gives the roll R of the UVOT axes.

The UVOT image X and Y axes vectors – which are the negated 2nd column and positive 3rd columns respectively of the matrix R that would rotate the right-handed set of vectors from the 1st point of aries (x vector) to the target position and roll – are given by

X = —cosαBsinδBsinR−sinαBcosRsinαBsinδBsinR+cosαBcosR

Y = −cosαBsinδBcosR−sinαBsinR−sinαBsinδBcosR+cosαBsinR

( Note

R = M(α,z) * M(δ,y) * M(R,x)

and

M(α,z) = cosαsinα0sinαcosα0001

M(δ,y) = cosδ0−sinδ010sinδ0cosδ

M(R,x) = 1000cosRsinR0−sinRcosR

)

The B vector is then subtracted from the P vector to give the difference vector D. As a precaution, B is first scaled by the reciprocal of the cosine of the radial offset r in order to extend it into the projection plane. However, in view of the small angles involved, this is probably unnecessary.

D = P/cos(r) - B

The code then obtains the components (x,y) of this difference vector against the image vectors using the dot product.

x = D.X

y = D.Y

These components correspond to the amount the image needs to be offset (with respect to the boresight) to recentre on the GRB.

The angle subtended by these offsets is

atan(offset/(magnitude of boresight pointing vector).

Since, by convention, the magnitude of the pointing vector is one, the angle subtended is atan(offset) ~= offset (where units are radians) as the angle is small. Therefore the component values are, to sufficient accuracy, the subtended angle of the offsets in radians. Thus the component values can be readily converted into the equivalent number of pixels.

These offsets then have the EEPROM tabulated offsets of the bore-sight within the UVOT image added. The result is then corrected to move back into the image if the total is outside the field of view.

The appropriate DPU command is then constructed and forwarded to the DPU informing it of the necessary corrections and a message is issued that summarizes the processing.

Procedure process_batgrbfluxinfo

This routine is called on reception of a BATGRBFLUXINFO record.

The GRB peak flux value is extracted form the record. Assuming the value is sensible, the observation number is extracted. If we have already received a FONEXTOBSINFO record containing the same observation number, then the RTS semaphore is raised to signal to any waiting RTS that the BAT GRB flux info relevant to this observation is now available.

Procedure calculate_sun_moon_planets

This procedure is called at startup within the Ada code and once every slew from an RTS. It ensures we have up-to-date positions of the sun, moon and planets for the constraint violation checks. It uses routines from the astro package.

function check_moving_bright_sources

This function determines if there is a constraint violation for any of the planets at the proposed observation positions. It is called as part of the preparation for every observation from an RTS. It uses routines from the astro package.

Function check_catalog_sources

This function is called by the get_at_config, get_pt_config and get_sp_config routines once per observation. It is used to determine the maximum exposure time at the target position for the filter to be used.

For the first call within a snapshot, it uses the routines in package starcat to determine those stars present in the field of view. It then calculates, using EEPROM tabulated count rates as a function of filter and colour, the count rate for each star present. It then calculates, for each star, the difference between the theoretical counts already accumulated this snapshot and an EEPROM tabulated count damage limit. This difference is then compared to the count rate and a maximum exposure time for each star deduced. The smallest number, which may even be zero, is returned as the answer. See procedures zero_accumulate_counts, start_accumulate_counts and end_accumulate_counts for details on how the theoretical counts are deduced.

Note that the faintest planets Uranus and Neptune are considered as stars for this exercise.

procedure zero_accumulate_counts

procedure start_accumulate_counts

procedure end_accumulate_counts

These three procedures work together to calculate the theoretical accumulated counts against each star in the field of view. zero_accumulated_counts is called at the start of the slew from Ada to reset the counts. start_accumulate_counts and end_accumulate_counts are called by RTSs at the appropriate moments in the exposure to start and stop the calculation. The locations of the calls the RTSs are intended to avoid counting when the filter wheel is moving.

function get_at_config

This function is called from an RTS in order to obtain the exposure configuration required for the next AT exposure from an EEPROM resident table or tables. If it deduces that such an exposure is not possible e.g. a bright source is in the field of view, it returns a value of FALSE.

The procedure first derives the time since burst, using the time_since_burst function. It sets a flag for later use by the filter wheel control software that states whether we are early in the burst. Having set up sensible defaults for items known not to be in the table (as they never change for ATs) and checking that the supplied UVOT mode is valid, it adds on 1 to that value should the controlling RTS have determined that this is a bright GRB. This ensures that an exposure sequence more appropriate is chosen.

It then commences a scan of exposure configurations, looking for a match of both UVOT mode and time since burst. The configurations are stored in a linked list set of blocks. When a match is found, an appropriate configuration is selected also based on whether an XRT corrected position was received and which filter is required. Any tabulated exposure with exposure times of zero are ignored and the next cyclic filter in the table is tried until a match is found. If no match is found, FALSE is sent back to the controlling RTS. If a match is found, the routine check_catatalog_sources is used to obtain the maximum exposure time (which may override the tabulated value).

The image center specified is corrected for any new offset calculated by process_xrtpos and the detector windows derived from the supplied DPU detector windows. The modified exposure configuration is then copied in to a local data area for use by the controlling RTS and a value of TRUE is returned, subject to minimum useful exposure time of 4 seconds.

function get_pt_config

This function is called from an RTS in order to obtain the exposure configuration required for the next PT exposure from an EEPROM resident table or tables. If it deduces that such an exposure is not possible e.g. a bright source is in the field of view, it returns a value of FALSE.

The maximum exposure time for all the PT exposures implied by the value of UVOT mode is given in the FONEXTOBSINFO record that preceded the exposure. However, that value included an allowance for interruption predicted to occur when entering the SAA. When we restart the PT sequence after an SAA interruption, we must carry on with the PT exposure in progress at the time of the exposure. Therefore, the first test this code performs is to check whether such an event had occurred and, if so, to simply return the previous PT configuration.

It then commences scan of an index of exposure configurations. Those indexes point to one or more configurations for each UVOT mode. For the first exposure in a snapshot an additional preliminary scan is made to find out now many exposures are to be used this pass and the total exposure time required. Then a scan is performed to select the correct configuration for this call by following the pointer given in the index. The indexes are stored in a linked list set of blocks. The scan starts at the last block examined in any prior scan, or at the beginning, in order to improve search times. For UVOT modes greater than or equal to 0x1000, the exposure time in the exposure table is scaled so as to fit into the time given in the FONEXTOBSINFO. If no match is found, FALSE is sent back to the controlling RTS. If a match is found, the routine check_catatalog_sources is used to obtain the maximum exposure time (which may override the tabulated value).

The image center specified is corrected for any new offset calculated by process_xrtpos and the detector windows derived from the supplied DPU detector windows. The modified exposure configuration is then copied in to a local data area for use by the controlling RTS and a value of TRUE is returned, subject to minimum useful exposure time of 4 seconds.

Functions get_config, get_sp_config, get_settling_config and get_fc_config are simply specialized versions of get_pt_config . They are used to obtain the safe pointing, settling and finding chart configurations respectively, as they are considered to PTs by the ICU (as they never change and are therefore ‘planned’).

Procedures starting_exposure, ending_exposure, inc_fc_attempts

These are routines called by RTSs and are used to instruct the ICU to perform various ‘book-keeping’ activities associated with the starting and end of an exposure respectively.

starting_exposure simply make a note of the starting time and increments the count of the number exposures performed this snapshot.

ending_exposure determines the total exposure time obtained. If we have just ended an AT exposure, it moves on to the next filter to use in a cyclic manner. However, if this was the first attempt at an exposure sequence for a new GRB that was interrupted by a short snapshot, this action is reversed so that we try again next snapshot. If we have just ended a PT exposure, it first checks if a SAA entry interrupted it. If it was, a flag is set and the remaining exposure time is calculated. The count of PT exposure performed is also suppressed. If there was no SAA interruption, then the number of PT exposure performed so far incremented

inc_fc_attempts In view of the scientific importance of the finding chart exposure, this routine is called by the controlling RTS at the start of the finding chart exposure to log how many attempts are made. Once the maximum attempts are made, it gives up and sets the flag to the a finding chart exposure has been taken. This is not true, but it prevents an infinite loop situation.

Function init

This initializes the package. In particular, it sets up useful defaults in the area of memory that contains the values read in from the ‘standard’ area of EEPROM should there be a failure in that operation. It also loads in the error action table, sending a TDRSS message if there is an error and load the angular constraint table or sets up defaults if that operation fails.

Procedure run_recovery_rts

This is called by the tmq package when it inspects the telemetry and discovers an NHK error message. The parameters from that message are passed to this routine for action.

It scans the copied to RAM from EEPROM error action table for a match of error and parameters. If one is found, it starts the RTS running if so supplied and if allowed to when the ICU is already in safe. It also, if so requested by entries in the table, echoes the message to TDRSS and set a flag that prevents the ICU leaving the safe state without manual intervention.

## Assembler Routine USERDEFS.ASM

### Summary

This defines the timer correction factor and Ada run-time constants and masks.

## Package version

### Summary

This simply provides the software version number. It has no body.

## Package watchdog

### Summary

This package provides the watchdog control code. It keeps resetting the hardware watchdog and also monitors all other tasks for a ‘hung’ condition.

### Description

The algorithm used is based upon an ‘Embedded Systems’ article.

The high priority watchdog task goes round in a big loop with a 1s delay. Every 4 (= RESET_INT) of these loops, it kicks the watchdog.

Tasks that are waiting to be started e.g. that are sitting at an Ada ‘accept’, flag themselves as ‘asleep’. Once started, they set themselves as ‘alive’ and continue to reset themselves as ‘alive’ every loop (or sub-loop if necessary).

Every 6 (= POLL_TASKS) of the watchdog task loop, (i.e. 1 in 24 loops) the watchdog task checks tasks not flagged as ‘asleep’ for ‘aliveness’. If one or more are set to ‘unknown’ the h/w watchdog is withheld. This leads to a watchdog reboot. Otherwise, all ‘Alive’ task flags are then reset to ‘Unknown’.

Thus, if a task has become inactive for some reason, it will loose the ‘race’ with the watchdog task and will not reset this flag to ’Alive’ or ‘Asleep’. The loop described in the previous paragraph will eventually detect this and lead to a watchdog reboot.

The RESET_INT (default 4) and the POLL_TASKS (default=6 ) are the first two arguments to the /IWATCHDOG INTERVAL telecommand. Note – for Basic, the default value of POLL_TASKS is 3.

The third argument of this telecommand is the task mask, which defines which tasks are to be checked. By default, all tasks are monitored. The mask values are implicitly defined by their order of appearance in package monitor.ads.

| Task | Mask |
| --- | --- |
| TCQ | 0x8000000 |
| HV_RAMP | 0x4000000 |
| DPU_DATA_MANAGER | 0x2000000 |
| HEATER_CONTROL | 0x1000000 |
| LIMIT_CHECK | 0x0800000 |
| MECHANISM_MOVE | 0x0400000 |
| FEED_1553 | 0x0200000 |
| HOUSEKEEPING | 0x0100000 |
| MEMORY_DUMP | 0x0080000 |
| DCS | 0x0040000 |
| LOAD_CENTROID_TABLE | 0x0020000 |
| LOAD_WINDOW_TABLE | 0x0010000 |
| FEED_SSI | 0x0008000 |
| STAR_CAT | 0x0004000 |

            - Ada Summary

              - Introduction

This section is present to introduce Ada notation conventions for which readers familiar with other languages may need clarification.

              - Ada Procedure and Function Notation.

The list of parameters in a subprogram or function call is known as actual parameters; inside the subprogram they are called formal parameters. They are passed in one of 3 calling modes:

| - in | Only the actual value is used; the subprogram cannot modify the value. |
| --- | --- |
| - out | The subprogram creates a value but does not use the value of the actual parameter |
| - in out | The subprogram uses the value from the actual parameter and may assign a new value to it. |

- If omitted, in mode is the default.

The notation used in subprogram and function calls is as follows, e.g.:

Procedure COUNT_LEAVES_ON_BINARY_TREE;

illustrates an interface to a procedure with no arguments.

procedure ROTATE 
	(POINT 	: in out	TRANSFORM.COORDINATE; 
	 ANGLE	: in 		UNITS.RADIANS);

states that procedure ROTATEs formal first argument has the name POINT, is of type COORDINATE (which in turn is defined in the package TRANSFORM) and that its calling mode is in out. Similarly the second argument has the name ANGLE, is of type RADIANS (defined in package UNITS) and that its calling mode is in.

Similarly for functions:

function COS 
     (ANGLE		: in		UNITS.RADIANS) return FLOAT;

shows that the function COS returns the predefined type FLOAT.

Note that function arguments can only be of mode in.

              - Ada Task and Entry Notation

A task specification introduces the name of the task, together with any entry points to the task. It

thus defines the communication paths (entries) available to other tasks. It may also define the priority at which a task runs. An entry declaration has a form similar to a subprogram specification. e.g:

task PROTECTED_STACK is 
  pragma priority(100);			-- define priority 
  entry POP (ELEMENT : out INTEGER);	-- define entry point 
  entry PUSH(ELEMENT : in INTEGER);	-- define entry point 
end PROTECTED_STACK;

It is always necessary to prefix entry calls with the task name. e.g.:

- PROTECTED_STACK.POP(MY_VALUE);

Occasionally, tasks are specified as task types. This is done if there is more than one instance of the task or to allow certain Ada pragmas (special instructions to the compiler) to be obeyed that are only supported under task types. The following is equivalent to the above specification.

task type PROTECTED_STACK_TYPE is	-- define the type. 
  pragma priority(100);			-- define priority 
  entry POP (ELEMENT : out INTEGER);	-- define entry point 
  entry PUSH(ELEMENT : in INTEGER);	-- define entry point 
end PROTECTED_STACK_TYPE;

PROTECTED_STACK : PROTECTED_STACK_TYPE;	-- create an instance of the task.

              - Use of Ada Pragma’s

A pragma is a statement that conveys information to the compiler. The following Ada pragmas are used throughout the code. The following is a summary of the usage.

              - ELABORATE

Takes one or more simple names denoting library units as arguments. This pragma is only allowed immediately after the context clause of a compilation unit (before the subsequent library unit or secondary unit). Each argument must be the simple name of a library unit mentioned by the context clause.

This pragma specifies that the corresponding library unit body must be elaborated before the given compilation unit. If the given compilation unit is a subunit, the library unit body must be elaborated before the body of the ancestor library unit of the subunit.

              - FOREIGN_BODY

This provides a  way to access entities written in languages other than Ada. It must appear in the visible part of the package before any declarations – see section 4.1.2.2 of the Tartan Ada Compilation System Manual.

It dictates that all subprograms and objects in the package are provided by means of a foreign object module. In the case of the ICU, the language used is always assembler and therefore the pragma takes the argument string “ASM”.

              - INLINE

Takes one or more names as arguments; each name is either the name of a subprogram or the name of a generic subprogram. This pragma is only allowed at the place of a declarative item in a declarative part or package specification, or after a library unit in a compilation, but before any subsequent compilation unit.

This pragma specifies that the subprogram bodies should be expanded inline at each call whenever possible.

              - LINKAGE_NAME

This pragma associates an Ada entity (e.g. subprogram or variable name) with a text string meaningful externally to, say, a linkage editor. It is usually used to equate that entity to its equivalent in assembler code.

              - OPTIMIZE

Takes one of the identifiers TIME or SPACE as the single argument. This pragma is only allowed within a declarative part and it applies to the block or body enclosing the declarative part.

It specifies whether time or space is the primary optimisation criterion.

              - PACK

Takes the simple name of a record or array type as the single argument. The allowed positions for this pragma, and the restrictions on the named type, are governed by the same rules as for a representation clause.

The pragma specifies that storage minimization should be the main criterion when selecting the representation of the given type.

              - PRIORITY

Takes a static expression of the predefined integer subtype PRIORITY as the single argument. This pragma is only allowed within the specification of a task unit or immediately within the outermost declarative part of a main program.

It specifies the priority of the task (or tasks of the task type) or the priority of the main program. N.B. The package IMPORTANCE defines of all task priorities used as arguments to this pragma.

              - SHARED

Takes the simple name of a variable as the single argument. This pragma is allowed only for a variable declared by an object declaration and whose type is a scalar or access type; the variable declaration and the pragma must both occur (in this order) immediately within the same declarative part or package specification.

This pragma specifies that every read or update of the variable is a synchronization point for that variable i.e. no optimisation is performed which might lead to the value contained in the variable not always being up-to-date.

              - SUPPRESS

Takes as arguments the identifier of a check and optionally also the name of either an object, a type or subtype, a subprogram, a task unit, or a generic unit. This pragma is only allowed either immediately within a declarative part or immediately within a package specification. In the latter case, the only allowed form is with a name that denotes an entity (or several overloaded subprograms) declared immediately within the package specification. The permission to omit the given check extends from the place of the pragma to the end of the declarative region associated with the innermost enclosing block statement or program unit. For a pragma given in a package specification, the permission extends to the end of the scope of the named entity.

If the pragma includes a name, the permission to omit the given check is further restricted: it is given only for operations on the named object or on all objects of the base type of a named type or subtype; for calls of a named subprogram; for activations of tasks of the named task type; or for instantiations of the given generic unit.

The identifier is that of the check that can be omitted. The name (if present) must be either a simple name or an expanded name and it must denote either an object, a type or subtype, a task unit, or a generic unit; alternatively the name can be a subprogram name, in which case it can stand for several visible overloaded subprograms.

The following checks correspond to situations in which the exception CONSTRAINT_ERROR may be raised; for these checks, the name (if present) must denote either an object or a type.

- ACCESS_CHECK: When accessing a selected component, an indexed component, a slice, or an attribute, of an object designated by an access value, check that the access value is not null.

- DISCRIMINANT_CHECK: Check that a discriminant of a composite value has the value imposed by a discriminant constraint. Also, when accessing a record component, check that it exists for the current discriminant values.

- INDEX_CHECK: Check that the bounds of an array value are equal to the corresponding bounds of an index constraint. Also, when accessing a component of an array object, check for each dimension that the given index value belongs to the range defined by the bounds of the array object. Also, when accessing a slice of an array object, check that the given discrete range is compatible with the range defined by the bounds of the array object.

- LENGTH_CHECK: Check that there is a matching component for each component of an array, in the case of array assignments, type conversions, and logical operators for arrays of boolean components.

- RANGE_CHECK: Check that a value satisfies a range constraint. Also, for the elaboration of a subtype indication, check that the constraint (if present) is compatible with the type mark. Also, for an aggregate, check that an index or discriminant value belongs to the corresponding subtype. Finally, check for any constraint checks performed by a generic instantiation.

The following checks correspond to situations in which the exception NUMERIC_ERROR is raised. The only allowed names in the corresponding pragmas are names of numeric types.

- DIVISION_CHECK: Check that the second operand is not zero for the operations /, rem and mod.

- OVERFLOW_CHECK: Check that the result of a numeric operation does not overflow.

The following check corresponds to situations in which the exception PROGRAM_ERROR is raised. The only allowed names in the corresponding pragmas are names denoting task units, generic units, or subprograms.

- ELABORATION_CHECK: When either a subprogram is called, a task activation is accomplished, or a generic instantiation is elaborated, check that the body of the corresponding unit has already been elaborated.

The following check corresponds to situations in which the exception STORAGE_ERROR is raised. The only allowed names in the corresponding pragmas are names denoting access types, task units, or subprograms.

- STORAGE_CHECK: Check that execution of an allocator does not require more space than is available for a collection. Check that the space available for a task or subprogram has not been exceeded.